3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108325

Online, August 16 – 27, 2021

Agenda item:		9.2.2	Support of Non continuous coverage

Source:		NEC

Title:		Support of discontinuous coverage

Document for:		Discussion and Decision

# 1. Introduction

At RAN #92e meeting, SI on NB-IoT/eMTC support for NTN was finished [1] and a corresponding WI was agreed [2] with one RAN2 objective on support of discontinuous coverage as below:

- Others: - Support of discontinuous coverage without excessive UE power consumption and without excessive failures / recovery actions. Minor enhancements to the existing power saving mechanisms e.g. DRX, PSM, eDRX, relaxed monitoring, and (G)WUS can be considered, and if found needed, specified, to support discontinuous coverage;

In this contribution, we share our initial opinion regarding eDRX and PSM feature under discontinuous coverage scenario:

# 2. Discussion

PSM and eDRX are both configured via NAS ATTACH and or TAU procedures:

UE monitors paging during active period according to idle (e)DRX and or PSM configuration. With discontinuous coverage, paging monitoring window will be further limited to coverage window.

We cannot 100% avoid that a paging is missed by UE due to out of coverage, but this should be avoided as much as possible. To be able to do so, network should be able to predicate when a given UE will be in coverage or out of coverage based on UE location, satellite ephemeris information and so.

It is even better if proper networks configuration of eDRX and PSM could make sure that UE’s active time does not mismatch with coverage window too much. This again needs core network has enough information to predict UE’s in coverage and out of coverage time window and then configure accordingly.

Observation 1: Regardless any enhancement on eDRX or PSM, it is better for core network having enough information to predict UE’s in and out coverage window to avoid paging a UE while it is out of coverage.

2.2 PSM

If PSM is request by UE and configured by network:

- UE will enter power saving mode, when T3324 expiry, which is (re)started as soon as the UE enters idle mode.

- UE will end power saving mode, when UE initiates any mobile originated data or signaling, e.g. periodical TAU after T3412 expiry

When UE is in power saving mode, it will shut down the AS and stay in deep sleep state without doing any measurement or monitoring.

When UE is out of coverage, it is ideal that UE also in deep sleep mode as in PSM. Moreover it is mentioned in TR that it should be possible for UE to predict discontinuous coverage based on the satellite assistance information. Question would be whether we extend the PSM procedure to cover out of coverage window or we specify it separately in AS layer, this needs coordination with CT1.

Observation 2: When UE is out of coverage by prediction, UE should be in deep sleep mode for power saving, FFS whether this is defined as PSM or not.

2.2 eDRX

When UE configured with eDRX, UE will monitor PO during PTW (paging transmission window) only.

Currently the PTW length and eDRX cycle are configurable, but the PTW position of different UEs are spread out evenly over the time based on UE-ID (see figure below and page after, refer to 36.304) in order to distribute the paging load.

Figure: UEx and UEy with same eDRX configuration have different PTW position

This method works well when all UEs are in coverage all or nearly all time. This may not work well for discontinuous coverage scenario. If UE is out of coverage only at small amount/portion of time, this issue may be ignorable. If UE is out of coverage at big amount of time, it would be challenging to match all UE’s PTW position with their coverage window most of time, especially if the coverage window length and or position are variable.

Observation 3:  it needs to be clarified how discontinues the coverage would be for further possible eDRX enhancement

# 3. Conclusion

In this contribution. We have following initial observations:

Observation 1: Regardless any enhancement on eDRX or PSM, it is better for core network having enough information to predict UE’s in and out coverage window to avoid paging a UE while it is out of coverage.

Observation 2: When UE is out of coverage by prediction, UE should be in deep sleep mode for power saving, FFS whether this is defined as PSM or not.

Observation 3:  it needs to be clarified how discontinuous the coverage would be for further possible eDRX enhancement discussion

Based on these observations, we propose:

Proposal 1: When UE is out of coverage by prediction, UE should be in deep sleep mode for power saving

Proposal 2: Clarify how discontinuous the coverage would be

Proposal 3: send LS to SA2 and CT1 to kindly inform that

- the discontinuous coverage scenario in IoT NTN

- core network would need to predict UE’s in and out coverage window to avoid paging a UE while it is out of coverage.

- UE will be in deep sleep mode when it is out of coverage, FFS whether this can be defined as PSM

- Necessarily of re-evaluating the eDRX feature in discontinuous coverage scenario

# References

[1] TR 36.763

[2] WID RP‑211601