3GPP TSG-RAN WG2 Meeting #115-e	R2-2108555

Online, 16 - 27 August 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on slice based RACH configuration

Agenda Item:	8.8.3	RACH

Document for:	Discussion and decision

- Introduction

During the RAN2 #114-e meeting, following agreements [1] for slice based RACH were achieved,

- 4: RAN2 confirm for a slice group, separated RO and/or separate preamble can be configured within the existing RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA

- 5: Same as NR Rel-15 conclusion, RAN2 conclude that there is no RA-RNTI collision between slice specific RACH and legacy RACH in shared RO

- 6: Same as NR Rel-15 conclusion, RAN2 conclude that the RA-RNTI collision between slice specific RACH and legacy RACH may happen in separate RO.

- Working assumption: this can be left to network implementation to resolve it (e.g. network configure RO in different time)

- FFS how many slice groups we can have and how they are indicated.

In this contribution, the discussions are mainly about the remaining issues of slice based RACH.

- Discussion

  - Slice group for slice specific RACH

In our discussion paper [2], we have detailed analysis and proposals for the slice group mechanism. We think that the slice group mechanism should be common for relevant features (slice based cell reselection, slice based RACH), so there should be a common discussion in the standard. In this case, we think that the slice group can be discussed in either of the features.

Proposal 1: It is proposed that RAN2 agree to apply unified slice group mechanism for both slice based RACH and slice based cell reselection.

  - Slice based RACH parameters prioritization

According to the agreement that, “ScalingFactorBI and powerRampingStepHighPriority can be configured at least in SIB”, i.e., the MPS/MCS related RACH parameters are reused for the slice based RACH prioritization. However, these two parameters may not be sufficient to meet the various requirements of different slices. To achieve RACH prioritization among different slices, slice-specific RACH parameters should be introduced. For example, a shorter contention resolution timer (i.e., ra-ContentionResolutionTimer) can satisfy the delay-sensitive requirement of URLLC slices. Other possible RACH parameters include, for example, preambleTransMax, ra-ResponseWindow, etc.

Proposal 2: It is proposed that RAN2 agree to support slice-specific RACH parameters for different slice groups, e.g., ra-ContentionResolutionTimer, preambleTransMax, and ra-ResponseWindow, etc.

  - Slice based RACH fallback

8 cases were proposed for the slice based RACH fallback in the offline discussion [3]. However, the classification may be inaccurate. According to TS 38.331 [4], the 4-step common RACH is configured in the RACH-ConfigCommon IE in SIB1, i.e., regardless of whether the slice based RACH is introduced, the UE can at least use the 4-step common RACH. Therefore, the 4-step common RACH is configured for the UE by default.

Observation 1: For slice based RACH, the 4-step common RACH is configured for the UE by default.

For the current spec, the RACH fallback is from 2-step RA to 4-step RA triggered by the msgA-TransMax reaching the maximum value. The difference between 2-step RA and 4-step RA is the size of MsgA and Msg1. For a RSRP higher than the msgA-RSRP-Threshold, the UE can use the 2-step RA to transfer more data in MsgA and access the network faster. If the network condition becomes worse, the UE can fallback to 4-step RA and access the network in a relatively slow way.

However, compared with the legacy RACH fallback, the fallback within the same RA type, e.g., from 2-step slice specific RA to 2-step common RA, and from 4-step slice specific RA to 4-step common RA, seems unnecessary. For the same RA type, the main difference between the slice specific RACH and common RACH is the isolated PRACH resources. The fallback will affect the access of normal UEs due to the occupation of common PRACH resources. On the other side, if the fallback of the same RA type is introduced, extra parameters, e.g., slice specific msgA-TransMax and msg1-TransMax should be defined, which will make the RACH procedure complicated and cause a large spec impact.

Proposal 3: The fallback within the same RA type, i.e., from 2-step slice specific RA to 2-step common RA, and from 4-step slice specific RA to 4-step common RA, should not be supported.

According to the above analysis, both the original table (Table 1) and the revised fallback case table (Table 2) are given as follows:

Table 1: RACH fallback analysis for RAN slicing (Original)

| Cases | RACH resource configuration in one BWP | RACH type selection for slice triggered access | Fallback after MSGA or MSG1 attempt number beyond threshold |
| --- | --- | --- | --- |
| Case 1 | 2-step slice specific RACH 4-step common RACH | FFS Always perform 2-step slice specific RACH | Fallback to 4-step common RACH |
| Case 2 | 2-step slice specific RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step slice specific RACH. FFS Fallback from 4-step slice specific RACH to 4-step common RACH |
| FFS Case 3 is valid | 4-step slice specific RACH 2-step common RACH | FFS Always perform 4-step slice specific RACH | FFS: No fallback vs. Fallback to common RACH |
| Case 4 | 4-step slice specific RACH 4-step common RACH | Always perform 4-step slice specific RACH | FFS: No fallback vs. Fallback to common RACH |
| Case 5 | 2-step slice specific RACH 2-step common RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step slice specific RACH. FFS Fallback from 4-step slice specific RACH to 4-step common RACH. |
| FFS Case 6 is valid | 2-step slice specific RACH 2-step common RACH | Always perform 2-step slice specific RACH | FFS: No fallback vs. Fallback to common RACH |
| Case 7 | 2-step slice specific RACH 2-step common RACH 4-step common RACH | FFS Always perform 2-step slice specific RACH | Fallback to 4-step common RACH. No fallback to 2-step common RACH. |
| FFS Case 8 is valid | 4-step slice specific RACH 2-step common RACH 4-step common RACH | FFS Always perform 4-step slice specific RACH | FFS Fallback from 4-step slice specific RACH to 4-step common RACH. |

Table 2: RACH fallback analysis for RAN slicing (Updated)

| Cases | RACH resource configuration in one BWP | RACH type selection for slice triggered access | Fallback after MSGA attempt number beyond threshold |
| --- | --- | --- | --- |
| Case 1 | 2-step slice specific RACH 4-step common RACH | Always perform 2-step slice specific RACH | Fallback to 4-step common RACH |
| Case 2 | 2-step common RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step common RACH |
| Case 3 | 2-step slice specific RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step slice specific RACH. |
| Case 4 | 4-step slice specific RACH 4-step common RACH | Always perform 4-step slice specific RACH | No fallback |
| Case 5 | 2-step slice specific RACH 2-step common RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step slice specific RACH. |
| Case 6 | 2-step slice specific RACH 2-step common RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step common RACH. |
| Case 7 | 4-step slice specific RACH 2-step common RACH 4-step common RACH | Always perform 4-step slice specific RACH | No fallback |

Proposal 4: It is proposed to adopt Table 2 for RACH fallback for slice based RACH.

- Conclusion

In this contribution, we mainly discuss the remaining issues of the slice based RACH. It is proposed:

Observation 1: For slice based RACH, the 4-step common RACH is configured for the UE by default.

Proposal 1: It is proposed that RAN2 agree to apply unified slice group mechanism for both slice based RACH and slice based cell reselection.

Proposal 2: It is proposed that RAN2 agree to support slice-specific RACH parameters for different slice groups, e.g., ra-ContentionResolutionTimer, preambleTransMax, and ra-ResponseWindow, etc.

Proposal 3: The fallback within the same RA type, i.e., from 2-step slice specific RA to 2-step common RA, and from 4-step slice specific RA to 4-step common RA, should not be supported.

Proposal 4: It is proposed to adopt Table 2 for RACH fallback for slice based RACH.

- References

- R2-2106471, Report on LTE legacy, Mobility, DCCA, Multi-SIM and RAN slicing.

- R2-2108554, Discussion on slice based cell reselection under network control, Huawei, HiSilicon

- R2-2104322, Summary for [AT113bis-e][252][NR] Slice-specific RACH.

- 3GPP TS 38.331 V16.4.1, Radio Resource Control (RRC) protocol specification.