3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108687

Online, August 09 – August 27, 2021, 2021

Source:	CATT

Title:	Further Consideration on Configuration of TRS/CRI-RS

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In last RAN2#113 e-meeting, the following initial conclusions on TRS/CSI-RS for idle/inactive UEs were made:

[041] On signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - SIB signalling is the baseline; - Other dedicated high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) can be additionally considered with justification. It is assumed they do not work alone. [041] RAN2 will down select from the following options on SIB signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - Option 2: Existing SIB, other than SIB1; - Option 3: New SIB type, e.g. SIB-x;

In this contribution, we would like to give our further consideration on configuration of TRS/CSI-RS for idle/inactive UEs.

Discussion

- SIB signalling

Although the exact parameters for TRS/CSI-RS for idle/inactive UEs haven’t been concluded yet, RAN1 has issued a preliminary list of TRS/CSI-RS parameters as follows:

In RAN1#105-e: Agreement: Support applicable values for the following configuration parameters as below. - powerControlOffsetSS: {-3, 0, 3, 6}dB - scramblingID: 0 to 1023 - firstOFDMSymbolInTimeDomain: 0 to 9 - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4 - startingRB: 0 to 274 - nrofRBs: 24 to 276 Agreement: The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. - FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration - FFS: QCL type, which is predetermined Agreement: Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset {10, 20, 40, 80} ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - FFS Configuration index - details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source

Although still incomplete, considering the configuration of TRS/CSI-RS in RRC_CONNECTED it can already be assessed that the overhead for the related configurations is not small. Therefore, we prefer to have a new SIB to include these parameters, which can also avoid the impact to the UEs which are not interested in TRS/CSI-RS configurations.

Proposal 1: Define a new SIB for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

- Dedicated signalling

RAN1 leaves it to RAN2 to decide whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.).

The TRS/CSI-RS configuration for idle/inactive UE(s) is not expected to be UE-specific. There is no strong motivation to introduce dedicated TRS/CSI-RS configuration for idle/inactive UEs. On the other hand, if the configuration of TRS/CSI-RS occasions for idle/inactive UEs is provided via dedicated RRC signaling, that means the network needs to keep the configuration for a certain UE for a certain time. Furthermore, we need to introduce a mechanism for the validity of the configuration of TRS/CSI-RS occasions for idle/inactive UEs. This will introduce additional complexity and unnecessary resource cost. Hence, we propose:

Proposal 2: Dedicated signalling is not supported for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

- Configuration update

From RAN1 LS [1]:

Agreements: The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs. - Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share). - Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted. - Note: Always-on TRS/CSI-RS transmission by gNodeB is not required - At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported - FFS for other RS types - FFS: Whether UE blind detection is required or not. Agreements: Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly). - Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

RAN1 agreed that always-on TRS/CSI-RS transmission by gNB is not required, i.e. TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted. And the TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs. On the other hand, we think the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s) doesn’t change frequently. Otherwise, additional UE power consumption will be introduced for a UE to acquire the updated configuration. As SIB signalling is used for the configuration of TRS/CSI-RS occasion(s) for idle/inactive, legacy SI update mechanism can be used for the configuration update.

As for the TRS/CSI-RS availability, the following agreements were made in RAN1#105-e:

Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Agreement: Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured. - FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

We can wait for RAN1’s further agreements on the availability indication of TRS/CSI-RS for idle/inactive UE(s).

Proposal 3: Legacy SI update mechanism can be used for (at least) the update of configuration of TRS/CSI-RS occasion(s) for idle/inactive.

Conclusion

In this contribution, we discuss our further consideration on TRS/CSI-RS or idle/inactive UEs and propose:

Proposal 1: Define a new SIB for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

Proposal 2: Dedicated signalling is not supported for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

Proposal 3: Legacy SI update mechanism can be used for (at least) the update of configuration of TRS/CSI-RS occasion(s) for idle/inactive.

Reference

- R2-210030	 LS on signalling method for TRS/CSI-RS occasion(s) for idle/inactive UE(s) (R1-2009848; contact: Samsung), RAN2#113-e

- Chair’s Notes RAN1#105-e