3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108714

Online, August 16 – 27, 2021	Revision of R2-2101753

Agenda Item:		8.6.5

Source:				ASUSTeK

Title:		Discussion on CS-RNTI for CG-SDT

Document for: 	Discussion and Decision

- Introduction

In the last meeting, the CS-RNTI based dynamic retransmission mechanism has been discussed and remains FFS.

Agreement

- FFS CS-RNTI based dynamic retransmission is reused for CG-SDT

This issue is continued in the e-mail discussion [Post114-e][508]. And in this contribution, we further discuss the CS-RNTI for CG-SDT.

- Discussion

For CG-SDT, the UE performs the small data transmission(s) using Type 1 configured grant. If a small data transmission is not successful, NW will schedule UL grant to the UE to retransmit the small data. To receive the retransmission scheduling, the UE needs to monitor a UE specific RNTI on PDCCH. In NR connected mode, the UE is configured with CS-RNTI for retransmission when the CG Type 1 is configured, and the UE monitors CS-RNTI for potential dynamic grant for retransmission. In RAN2 #114 meeting [1], the discussion about the CS-RNTI based retransmission mechanism for CG-SDT has been triggered. And in the e-mail discussion [Post114-e][508], companies mention that the existing mechanism could be reuse. That is, the UE should monitor the CS-RNTI during the CG-SDT procedure as legacy NR with less specification impact.

Since the UE could be configured with Type 1 CG configuration in connected state, the CS-RNTI used in connected state can be reused or the CG-SDT. When the UE is in RRC_CONNECTED state, the UE could be configured with Type 1 CG configuration, and the UE would store the CG configuration in UE Inactive AS Context when transiting to RRC_INACTIVE state. The CG configuration including CS-RNTI can be restored when the UE initiates the CG-SDT procedure in RRC_INACTIVE state.

On the other hand, according to the agreement in RAN2#112e [2], the configuration of CG-SDT resource should be contained in the RRCRelease message. The configuration of CS-RNTI for CG-SDT could also be included in the configuration in RRCRelease message. When the UE transits to RRC_INACTIVE state from RRC_CONNECTED state, the UE could receive the CG configuration including CS-RNTI in the RRCRelease message.

That is, if the UE receives a CS-RNTI in the RRCRelease message, the UE could use the received CS-RNTI for CG-SDT. If the UE doesn’t receive a new CS-RNTI in RRCRelease message, the UE could restore the CS-RNTI from the stored configuration in CG-SDT procedure. During the CG-SDT procedure, a UE could monitor a CS-RNTI obtained from the stored CG configuration (used in RRC_CONNECTED state) or obtained from the RRCRelease message.

Proposal 1: The CS-RNTI for CG-SDT is restored from the stored configuration if the UE doesn’t receives a CS-RNTI in RRCRelease message.

Proposal 2: The CS-RNTI for CG-SDT is the CS-RNTI received in RRCRelease message if the UE receives a CS-RNTI in the RRCRelease message.

- Conclusion

In this contribution, we discuss the CS-RNTI used for CG-SDT, and have the following proposals:

Proposal 1: The CS-RNTI for CG-SDT is restored from the stored configuration if the UE doesn’t receives a CS-RNTI in RRCRelease message.

Proposal 2: The CS-RNTI for CG-SDT is the CS-RNTI received in RRCRelease message if the UE receives a CS-RNTI in the RRCRelease message.

- Reference

- R2-21069xx, “Report of 3GPP TSG RAN WG2 meeting #114-e, Online”

- R2-2100001, “Report of 3GPP TSG RAN2#112-e meeting, Online”