3GPP TSG-RAN WG2 Meeting #116 electronic 	R2-2109546

Online, Nov 1-12, 2021

Agenda Item:	8.7.2.2

Source: 	MediaTek Inc.

Title:  	Remaining open issues for Service Continuity

Document for:	Discussion and decision

# Introduction

There are still some remaining open issues for service continuity following the discussion at RAN2#115e for sidelink relay.

This paper intends to discuss the solutions for these remaining issues.

# Support of RRC_IDLE / RRC_INACTIVE Relay UE

During the email discussion at RAN2#115e meeting (with report in R2-2108939), there is a discussion on the support of RRC_IDLE / RRC_INACTIVE Relay UE during direct-to-indirect switching. There were diverse views expressed and no consensus was reached at RAN2#115e. It should be noted that the current procedure for direct-to-indirect switching as captured in the SL relay running CR of 38.300 (in R2-2108924) is applicable only to Relay UE in RRC_CONNECTED.

If the Relay UE is in RRC_IDLE, the network needs to reach the Relay UE via CN based paging before switching the link from direct path to indirect path, since in RRC_IDLE, the only thing the Relay UE monitors for is CN paging, and the only ID by which it can be addressed is 5G-S-TMSI. According to the current common understanding, the Remote UE may report the L2 ID of the Relay UE to its connected cell following relay discovery and sidelink measurement. Then the network may derive the Relay UE’s 5G-S-TMSI based on such L2 ID according to the report from Remote UE. Such derivation may be done at 5GC and then coordination with SA2 may be needed to enable that solution.

If the Relay UE is in RRC_ INACTIVE, the network can reach the Relay UE via RAN based paging before switching the link from direct path to indirect path, since in RRC_ INACTIVE, the Relay UE monitors RAN paging. During the RAN based paging, the Relay UE is addressed via I-RNTI. In case of intra-gNB direct-to-indirect switching, the gNB may maintain the mapping relation between the I-RNTI and L2 ID for Relay UE. Then when Remote UE reports the L2 ID of the Relay UE to its connected cell following relay discovery and sidelink measurement, the gNB can derive the I-RNTI of the Relay UE and initiate the RAN based paging to wake up the Relay UE to provide relaying service. In case of inter-gNB direct-to-indirect switching, context fetch procedure is needed to enable the RAN based paging.

As can be seen, to support direct-to-indirect switching to Relay UE in RRC_IDLE / RRC_INACTIVE, additional spec impact is expected. We do not think it is the right idea to discuss this in addition to the direct-to-indirect switching to connected Relay UE, due to the work load of Rel-17 sidelink relay.

Proposal-1: The support of direct-to-indirect switching to Relay UE in RRC_IDLE / RRC_INACTIVE is postponed to next release.

# New timer for direct-to-indirect switching

During the discussion within R2-2108939, there is a discussion on whether the timer T304 or similar timer is needed for the path switch procedure and also whether a different T304 is needed for the case of path switch from direct to indirect path and vice versa.

According to the replies received within R2-2108939, all or majority companies replied no to the question to introduce a new timer for path switch from indirect to direct path. Our understanding is that the legacy T304 could be reused for path switch from indirect to direct path.

Proposal-2: the legacy T304 is reused for path switch from indirect to direct path.

Meanwhile, for the path switch procedure from direct to indirect path, it would need a new timer. The timer T304 is started at the reception of the reconfiguration with sync and is stopped when the RACH is completed. However, in path switch the Remote UE does not perform RACH and thus timer T304 cannot be reused. This means the stop condition for the new timer is a bit different from that of T304. We also think that legacy reconfiguration with sync cannot be reused as such for the path switch from direct to indirect.

Proposal-3: A new timer (T304 alike) is introduced for path switch from direct to indirect path.

# Conclusion

The following proposals are made:

Proposal-1: The support of direct-to-indirect switching to Relay UE in RRC_IDLE / RRC_INACTIVE is postponed to next release.

Proposal-2: the legacy T304 is reused for path switch from indirect to direct path.

Proposal-3: A new timer (T304 alike) is introduced for path switch from direct to indirect path.

# References

[1]	R2-2108939, Summary of [AT115-e][609][Relay] Service continuity procedures (MediaTek)