3GPP TSG RAN WG2 Meeting #116-e           		              	R2-2109614

Electronic meeting, 1st – 12th Nov 2021

Agenda Item:	8.4.3

Source:	Intel Corporation

Title:	Inter-donor CU topology migration, topology redundancy and CP-UP separation

Document for:	Discussion and Decision

# Introduction

In email discussion [Post115-e][088][eIAB]inter-CU routing open issues [1], several open issues of inter-donor CU routing was discussed, including terminology, concatenate traffic and non-concatenated traffic differentiation, BAP processing, configurations, etc. In this contribution, we first discuss the traffic flows which will be handled at the boundary IAB-node based on potential topology, then propose the analysis of how to define a common solution for various scenarios.

# Discussion

## Inter-Donor CU Topology Migration and Topology Redundancy

In R3-212981, two terminologies of inter-donor migration are defined by RAN3[2], i.e. partial migration and full migration:

- Partial Migration: the boundary IAB-MT is migrated to the 2nd IAB-donor-CU, while the boundary IAB-DU and descendant IAB node(s) (if any) are terminated to the 1st IAB-donor-CU. - Full Migration: the boundary IAB node and the descendant IAB node(s) (if any) are migrated (both RRC and F1 connection) to the 2nd IAB-donor-CU from 1st IAB-donor-CU.

In RAN2 #115e meeting, RAN2 discussed the feasibility of two alternatives proposed by RAN3 for full migration. RAN2 further concluded that both solutions need further standards impact analysis in RAN2. Hence, in this contribution, we mainly focus on the partial migration scenario.

### Inter-donor CU Scenario

In post-meeting email discussion [Post-115e][088][eIAB]inter-CU routing open issues [1], two types of scenario are discussed (shown as below):

Scenario 1: Inter-donor CU partial migration with CU1’s traffic only

Due to the unavailability of BH link between IAB-DU1 and IAB-MT2, IAB-MT2 migrates and establishes the connection with IAB-DU3 under IAB-donor CU2’s topology. Partial migration of IAB-node 2 is considered in this scenario, IAB-donor CU1 remains the F1-termination donor CU. All traffic of IAB-donor CU1 are routed via CU2’s topology. There’s no user plane traffic from/to IAB-donor CU2.

Figure 2-1. Inter-donor CU partial migration                  Figure 2-2. Inter-donor CU topology redundancy

Scenario 2: Inter-CU topology redundancy with CU1’s traffic only

In this scenario, IAB-MT2 (single IAB-MT) is dual connected with IAB-DU1 and IAB-DU3. IAB-DU2’s F1 interface is terminated at IAB-donor CU1. Due to load balancing, the traffic of IAB-donor CU1 may be routed via its own topology or via CU2’s topology. There’s no user plane traffic from/to IAB-donor CU2.

Besides above two scenarios discussed in [1] which only considers IAB-donor CU1’s traffic, it is also that IAB-donor CU2 has its own traffic towards its own descendant IAB-node(s) (e.g. scenario 3) or even some descendant IAB-nodes are partial migrated to IAB-donor CU1 (e.g. scenario 4).

Scenario 3: Inter-donor CU topology redundancy with CU2’s traffic routes via its own topology

Compared with Scenario 2, IAB-node 3 from CU2’s topology also has a descendant IAB-node. Both CU2’s non-concatenated traffic (e.g. traffic in blue line) and CU1’s concatenated traffic via CU2 (e.g. traffic in purple and brown line) are routed through IAB-donor DU2 and IAB-node 3. However, the traffic of IAB-node 5 is directly routed from IAB-node 3. In this scenario, the boundary IAB-node is not impacted by CU2’s traffic.

Figure 2-3. Scenario 3                                                       Figure 2-4. Scenario 4

Scenario 4: CU2’s descendant IAB-node inter-donor CU partial migration via boundary IAB-node of CU1’s topology

As agreed in RAN3 #112e meeting:

WA: boundary and descendant nodes may have a different F1-termination node.

On top of scenario 3, in case BH link “IAB-node 5 <-> IAB-node 3” are not available (e.g. due to RLF), IAB-node 5 can also perform partial migration and connects to the boundary IAB-node (i.e. IAB-node 2). After partial migration, IAB-node 5 becomes the descendant IAB-node of IAB-node 2, and IAB-DU 5 remains its F1 termination at IAB-donor CU2. As shown in Figure 2-34, IAB-node 2 and IAB-node 5 has different F1-termination node. However, as IAB-MT 2 is dual connected with both IAB-DU1 and IAB-DU3, the traffic of IAB-node 5 can be routed via the path “IAB-node 2 <-> IAB-node 3 <-> IAB-donor DU2 <-> IAB-donor CU2”.

Alternatively, it is equivalent when the traffic in blue line is routed via “IAB-node 2 <-> IAB-node 1 <-> IAB-donor DU1 <-> IAB-donor CU2”.

- Four scenarios should be considered for inter-donor CU partial migration and topology redundancy:

  - Scenario 1: Inter-donor CU migration with CU1’s traffic only

  - Scenario 2: Inter-donor CU topology redundancy with CU1’s traffic only

  - Scenario 3: Inter-donor CU topology redundancy with CU2’s traffic routes via its own topology

  - Scenario 4: CU2’s descendant IAB-node inter-donor CU partial migration via boundary IAB-node of CU1’s topology

### Traffic at the Boundary IAB-node

The terminology of concatenated traffic and non-concatenated traffic was discussed in [1]:

Concatenated traffic: The traffic routing across two topologies which belong to different CUs. Non-concatenated traffic: The traffic routing across one topology which belong to one CU.

From above four scenarios, there are four types of traffic flow across the topology:

1) CU1’s non-concatenated traffic (e.g. traffic in red and orange line)

2) CU1’s concatenated traffic via CU2 (e.g. traffic in purple and brown line)

3) CU2’s non-concatenated traffic (e.g. traffic in blue line)

4) CU2’s concatenated traffic (e.g. traffic in pink line)

To differentiate traffic to the upper layer of the boundary IAB-node and the traffic routing to its parent/descendant IAB-node, we further define two terminologies based on the destination of the traffic:

Access Traffic: Accessed UE’s traffic of the IAB-node. Routing Traffic: The traffic routing to IAB-node in the next hop. Next hop IAB-node is either parent IAB-node or child IAB-node.

To understand BAP processing at the boundary IAB-node, we first analysis what types of traffic are processed at the boundary IAB-node.

Type 1: CU1’s non-concatenated access traffic

Type 2: CU1’s non-concatenated routing traffic

Type 3: CU1’s concatenated access traffic via CU2’s topology

Type 4: CU1’s concatenated routing traffic via CU2’s topology

Type 5: CU2’s concatenated routing traffic via CU1’s topology

- The boundary IAB-node needs to process and differentiate among five types of traffic flow:

  - CU1’s non-concatenated access traffic

  - CU1’s non-concatenated routing traffic

  - CU1’s concatenated access traffic via CU2’s topology

  - CU1’s concatenated routing traffic via CU2’s topology

  - CU2’s concatenated routing traffic via CU1’s topology

### Configuration Update

Following shows an example of configuration update of IAB-node 2 inter-donor CU partial migration:

Figure 2-5. Inter-donor CU Partial Migration

During partial migration of IAB-node 2, the procedure of IAB-MT2’s migration to IAB-donor CU2 is shown as Step 1 to Step 9. Additionally, IAB-donor CU2 sends RRCReconfiguration message to IAB-MT2, which contains the new BAP address of the boundary IAB-node and default UL mapping of IAB-donor CU2’s topology. As IAB-DU2 remains F1 connection with IAB-donor CU1, IAB-donor CU1 should configure the updated F1AP configuration by considering BH link between IAB-node 2 and IAB-node 3, i.e. Step 10. Meanwhile, IAB-donor CU2 also needs to send updated F1AP configuration to IAB-DU3 by considering BH link “IAB-node 2 <-> IAB-node 3”. The RRCReconfiguration message to descendant IAB-node of the boundary IAB-node (i.e. IAB-node 4) will be encapsulated into F1-C message and routed via IAB-donor DU2, as shown in Step 11 to Step 14.

For inter-donor CU topology migration, bap-config to IAB-MT2 is configured by IAB-donor CU2. This also includes BAP address of the boundary IAB-node configured by IAB-donor CU2. For inter-donor CU topology redundancy, IAB-MT2 is now dual-connected with two IAB-donor CUs. The boundary IAB-node has two BAP addresses, one is originally configured by IAB-donor CU1, another is configured/newly added by IAB-donor CU2.

In summary, below table shows the termination node and the corresponding configuration at the boundary IAB-node (i.e. IAB-node 2):

|  | Termination | Configuration | IEs related to routing | Impact Traffic |
| --- | --- | --- | --- | --- |
| IAB-MT2 | IAB-donor CU2 | RRC | BAP address DefaultUL-BAP-Routing-ID DefaultUL-BAP-RLC-Channel | Access Traffic |
| IAB-DU2 | IAB-donor CU1 | F1AP | BH Routing Configuration (i.e. BH Information), including Routing ID and egress BAP address, egress BH RLC CH ID | Access Traffic |
|  |  |  | BAP Mapping Configuration | Routing Traffic |
|  |  |  | BAP layer BH RLC CH Mapping Information List | Routing Traffic |

- Routing configurations of IAB-MT and IAB-DU at the boundary IAB-node are configured by different IAB-donor CUs separately.

We then analyze the traffic routing and processing at each IAB-node of the routing path.

Upstream Traffic

For upstream traffic, to (re)route packets to IAB-donor DU2, the BAP address of IAB-donor DU2 should be added in the BAP header, as well as the path ID in CU2’s topology.

Considering there’s no assumption that IAB-donor CU1 and IAB-donor CU2 coordinate the BAP configuration with each other, IAB-donor CU1 cannot include IAB-donor DU2’s real BAP address in BAP configurations in F1AP messages. Hence, upstream traffic generated in the boundary IAB-node and its descendant IAB-nodes cannot add IAB-donor DU2’s real BAP address as DESTINATION in the BAP header. To reroute packets to CU2’s topology in upstream, a pseudo-BAP address is considered to represent IAB-donor DU2 as the destination. To avoid BAP address collision between pseudo-BAP address configured for IAB-donor DU2 with other IAB-nodes in CU2’s topology, the pseudo-BAP address and pseudo-path identity for IAB-donor DU2 are configured by IAB-donor CU2.

- For upstream traffic, during inter-donor CU partial migration:

  - The target IAB-donor CU configures a pseudo-BAP address and pseudo-path identity to the boundary IAB-node, representing the target IAB-donor DU and path to be selected.

  - The target IAB-donor CU forwards the configured pseudo-BAP address and pseudo-path identity to the source IAB-node

  - The source IAB-donor CU reconfigures BAP Routing configuration of IAB-DU at the boundary IAB-node with entries corresponding to “pseudo-BAP address” and “pseudo-path identity”.

- CU1’s non-concatenated traffic

For CU1’s non-concatenated access traffic and non-concatenated routing traffic, as there’s no need to rewrite the BAP header and the packets are still routed via the same topology, data transfer procedure is the same as Rel-16. There’s no need to update the routing configuration for CU1’s non-concatenated traffic via CU1’s topology.

- Rel-16 data transfer procedure is used for CU1’s non-concatenated traffic. No need to update the routing configuration for CU1’s non-concatenated traffic.

- CU1’s concatenated traffic via CU2’s topology

For CU1’s concatenated access traffic via CU2 during inter-donor CU partial migration, pseudo-BAP address and pseudo-path identity is added in the BAP header.

For CU1’s concatenated routing traffic via CU2, the boundary IAB-node first needs to compare DESTIANTION field in the BAP header with its BAP address allocated by either CU2 (for inter-donor CU topology migration) or CU1 (for inter-donor CU topology redundancy). If destination is not matched, the boundary IAB-node will decide to route the packet to the next hop. However, the destination address added in routing packets is IAB-donor DU1’s BAP address. Hence, to reroute packets via IAB-donor DU2 during inter-donor CU partial migration, the destination BAP address and path ID in the BAP header should be updated to the pseudo-BAP address and pseudo-path ID. As agreed in RAN2 #115e meeting:

- As baseline, support the 1:1 and N:1 mapping from “previous routing ID” to “new routing ID” for BAP header rewriting at the boundary node, in inter-CU routing.

The pseudo-BAP address and pseudo-path ID can be considered as the new routing ID when BAP header rewriting is processed at the boundary IAB-node.

- During inter-donor CU partial migration, following procedures at the boundary IAB-node are considered for upstream traffic:

  - the boundary IAB-node adds pseudo-BAP address and pseudo-path ID in the BAP header of the source donor CU’s concatenated access traffic.

  - When receiving a packet from its descendant IAB-node, the boundary IAB-node compares the DESTINATION field in the BAP header with its own BAP addresses

  - rewrites routing ID in BAP header of source donor CU’s concatenated routing traffic into “pseudo-BAP address and pseudo-path identity”, if DESTINATION field in BAP header does not match boundary IAB-node’s BAP address (which is configured by target donor CU for topology migration or configured by source donor CU for topology redundancy).

When routing the upstream traffic during inter-donor CU partial migration, IAB-donor CU1 can configure new egress BH RLC channels for the new BH link, i.e. “IAB-MT2 <-> IAB-DU3”, which is not collided with previous configuration in CU1’s topology. In the corresponding entry of BH Routing Configuration and BH RLC channel mapping Information list, a pseudo “Next Hop BAP Address” is also needed to represent IAB-DU3 in CU1’s topology.

- During inter-donor CU partial migration, the source IAB-donor CU reconfigures BAP Routing configuration of IAB-DU at the boundary IAB-node with pseudo “Next Hop BAP address”, representing the next hop IAB-DU in the target topology.

When receiving upstream traffic from the boundary IAB-node, IAB-DU3 need to further forward and route CU1’s concatenated traffic to its parent IAB-node (i.e. IAB-donor DU2). According to Rel-16 data transfer procedure:

Routing: else if there is an entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, whose BAP path identity is the same as the PATH field, and whose egress link corresponding to the Next Hop BAP Address is available: - select the egress link corresponding to the Next Hop BAP Address of the entry; NOTE 1: An egress link is not considered to be available if the link is in BH RLF. NOTE 2: For each combination of a BAP address and a BAP path identity, there should be at most one entry in the BH Routing Configuration. There could be multiple entries of the same BAP address in the BH Routing Configuration. - else if there is at least one entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available: - select an entry from the BH Routing Configuration whose BAP address is the same as the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available; - select the egress link corresponding to the Next Hop BAP Address of the entry selected above; Receiving operation: if DESTINATION field of this BAP Data PDU matches the BAP address of this node: - remove the BAP header of this BAP Data PDU and deliver the BAP SDU to upper layers; - else: - deliver the BAP Data Packet to the transmitting part of the collocated BAP entity. Handling of unknown, unforeseen, and erroneous protocol data When a BAP Data PDU that contains a BAP address which is not included in the configured BH Routing Configuration and is not the BAP address of this node is received; or when a BAP Control PDU that contains reserved or invalid values is received the BAP entity shall: - discard the received BAP PDU.

As the IAB-donor CU2 configures pseudo-BAP address used in CU1’s topology, CU1’s concatenated upstream traffic will not be discarded at IAB-nodes under CU2’s topology. BAP Routing Configuration at IAB-DU3 should also be updated with an entry corresponding to the destination BAP address of IAB-donor DU2’s pseudo-BAP address. When IAB-donor DU2 receives CU1’s concatenated traffic via CU2, as DESTINATION field matches its pseudo-BAP address of this node, the BAP Data Packet will be delivered to the upper layer after removing the BAP header.

- During inter-donor CU partial migration, the target IAB-donor CU reconfigures BAP Routing Configuration of ancestor IAB-nodes of the boundary IAB-node in the target topology with at least an entry of target IAB-donor DU’s pseudo-BAP address.

- CU2’s concatenated traffic via CU1’s topology

As shown in Figure 2.4, due to RLF or receiving type-2 RLF indication from IAB-DU3, IAB-node 5 can also partially migrated to the boundary IAB-node (i.e. IAB-DU2). The upstream traffic of IAB-node 5 is routed via the pink link.

The DESTINATION BAP address in the BAP header of CU2’s concatenated traffic is added in IAB-node 5. As IAB-DU5 remains its F1AP termination at IAB-donor CU2, IAB-DU2’s real BAP address is used as the DESTINATION BAP address, so does the path identity.

When the traffic is routed in the upstream, there are two possibilities may occur at the boundary IAB-node:

1) the DESTINATION BAP address of CU2’s concatenated traffic is collided with BAP address of IAB-nodes in CU1’s topology (e.g. boundary IAB-node), the packets will be delivered to the upper layer;

2) there’s no BAP address in the BAP Routing Configuration matching with the DESTINATION filed, the packets will be discarded.

However, they are not the proper processing to CU2’s concatenated traffic.

- When processing CU2’s concatenated traffic via CU1’s topology, the boundary IAB-node may deliver the received packets to the upper layer or discard it due to BAP address collision or mismatch.

Considering the complexity of the scenario, we propose to deprioritize this scenario and confirm that F1 termination of the boundary IAB-node and its descendant IAB-node should be the same.

- During inter-donor topology migration and topology redundancy, RAN2 confirms F1 termination of the boundary IAB-node and its descendant IAB-node should be the same in Rel-17.

Downstream Traffic

As discussed above, the BAP address of the boundary IAB-node is added (for topology redundancy)/reconfigured (for topology migration) by IAB-donor CU2 during RRCReconfiguration. For downstream access traffic to the boundary IAB-node, the real BAP address of the boundary IAB-node can be added in the BAP header.

- For downstream access traffic of the boundary IAB-node, target IAB-donor DU can add the real BAP address of the boundary IAB-node in the BAP header, as it is configured by CU2.

Similar as the boundary IAB-node, IAB-donor DU2 also no information about real BAP addresses of descendant IAB-node of the boundary IAB-node which used in CU1’s topology. For those descendant IAB-nodes in CU1’s topology, pseudo-BAP addresses are considered, which are known by IAB-donor CU2. These pseudo-BAP addresses should also be updated as new entries of BAP configuration at IAB-node of CU2’s topology via F1AP, so that the right egress link, Next Hop BAP address, egress BH RLC CH ID can be selected properly. To avoid collision with other real destination BAP addresses in CU2’s topology, the pseudo-BAP addresses of descendant IAB-node in CU1’s topology are configured by IAB-donor CU2.

- For downstream traffic, during inter-donor CU partial migration, the target IAB-donor CU:

  - Configures pseudo-BAP addresses and pseudo-path identities to ancestor IAB-nodes of the boundary IAB-node in target topology, representing descendant IAB-nodes of the boundary IAB-node and path in the source topology

  - reconfigures BAP Routing configuration of IAB-DU in the target topology with entries corresponding to “pseudo-BAP address” and “pseudo-path identity”.

- CU1’s non-concatenated traffic

Similar as upstream traffic, Rel-16 procedures are reused to transfer CU1’s non-concatenated traffic to the descendant IAB-nodes.

- CU1’s concatenated traffic via CU2’s topology

After IAB-donor DU2 receives CU1’s concatenated traffic from IAB-donor CU1, similar as upstream access traffic, pseudo-BAP address and pseudo-path identity are added in the BAP header. The egress BH link and egress BH RLC channel ID can be selected by each IAB-node in CU2’s topology by using the corresponding entry with pseudo-BAP address which matches the DESTINATION BAP Address in the BAP header of the downstream packets.

For the boundary IAB-node, it is straightforward for the boundary IAB-node differentiate CU1’s non-concatenated and concatenated traffic based on the ingress BH links. Additionally, it also receives two other types of traffic from IAB-DU3 in the downstream:

1) CU1’s concatenated traffic via CU2’s topology

2) CU2’s concatenated traffic via CU1’s topology

As proposed in Proposal 5, CU2’s concatenated traffic via CU1’s topology is deprioritized. To successfully delivery downstream traffic to each IAB-node in CU1’s topology, a BAP header rewriting table containing the mapping between real BAP address and pseudo-BAP address of descendant IAB-nodes are proposed to be configured at the boundary IAB-node. The boundary IAB-node will first compare and decide whether the received packet is of its own. If not, for routing traffic, if DESTINATION BAP address in the BAP header matches the pseudo-BAP address in the BAP header rewriting table, this traffic is identified as CU1’s concatenated traffic via CU2’s topology, and BAP header rewriting is performed.

- To successfully deliver downstream traffic to CU1’s concatenated traffic via CU2 to its destination (e.g. descendant IAB-node of boundary IAB-node), following configurations and procedures are proposed:

  - The target IAB-donor CU forwards pseudo-BAP addresses of descendant IAB-nodes of the boundary IAB-node to the source IAB-donor CU via Xn interface

  - The source IAB-donor CU configures a BAP header rewriting table, containing the mapping between real and pseudo-BAP address of those descendant IAB-nodes

  - The boundary IAB-node performs BAP header rewriting according to the configured BAP header rewriting table if DESTINATION field in the BAP header does not match with boundary IAB-node’s BAP address and also matches one of the pseudo-BAP addresses in the BAP header rewriting table.

  - The boundary IAB-node selects the egress BH ink and egress BH RLC Channel ID corresponding to the real DESTINATION BAP address

## CP/UP separation

As agreed in RAN2 #113bis-e meeting, SRB2 and split SRB2 are used for F1-C transport in CP/UP separation scenario 1 and scenario 2, respectively.

- SRB2 can be used for F1-C transport in CP/UP-separation scenario 1 (FFS other cases) - Split SRB2 can be used for F1-C transport in CP/UP-separation scenario 2 (FFS other cases)

It was discussed in [4] about whether SRB3 can be used for the transport of F1-C in scenario 2 (i.e. MN has donor functionality). SRB3 is used to carry the specific RRC messages when UE is in (NG)EN-DC or NR-DC, which is configured with the highest priority Channel Access Priority Class as SRB0 and SRB1. SRB2 has a lower priority than SRB1/SRB3. Compared with RRC messages which are essential for connection establishment between IAB-nodes and their parent IAB-nodes, F1AP messages can be treated as messages with lower priority.

- SRB3 is not used for F1-C transport in CP/UP separation scenario 2.

When using split-SRB for scenario 2, one may also consider some enhancements to differentiate RRC messages and select SCG link for F1-C transport only. From our understanding, RRC messages from IAB-MT should follow the same principle as the normal UE. Hence, with split SRB2, the donor CU can decide whether to send the F1-C traffic via MN and/or SN as normal UE’s behavior.

- Encapsulated F1-C transport via split SRB2 in scenario 2 can be sent via MN and/or SN as normal UE’s behavior.

# Conclusion

In this contribution, we proposed four scenarios during inter-donor CU migration and topology redundancy, then mainly focused on the discussion of BAP processing during inter-donor CU migration. In the end, we analyzed SRB3 for CP/UP separation.

We propose following observations and proposals:

Observation 1: Four scenarios should be considered for inter-donor CU partial migration and topology redundancy:

  - Scenario 1: Inter-donor CU migration with CU1’s traffic only

  - Scenario 2: Inter-donor CU topology redundancy with CU1’s traffic only

  - Scenario 3: Inter-donor CU topology redundancy with CU2’s traffic routes via its own topology

  - Scenario 4: CU2’s descendant IAB-node inter-donor CU partial migration via boundary IAB-node of CU1’s topology

Observation 2: The boundary IAB-node needs to process and differentiate among five types of traffic flow:

- CU1’s non-concatenated access traffic

- CU1’s non-concatenated routing traffic

- CU1’s concatenated access traffic via CU2’s topology

- CU1’s concatenated routing traffic via CU2’s topology

- CU2’s concatenated routing traffic via CU1’s topology

Observation 3: Routing configurations of IAB-MT and IAB-DU at the boundary IAB-node are configured by different IAB-donor CUs separately.

Proposal 1: For upstream traffic, during inter-donor CU partial migration:

  - The target IAB-donor CU configures a pseudo-BAP address and pseudo-path identity to the boundary IAB-node, representing the target IAB-donor DU and path to be selected.

  - The target IAB-donor CU forwards the configured pseudo-BAP address and pseudo-path identity to the source IAB-node

  - The source IAB-donor CU reconfigures BAP Routing configuration of IAB-DU at the boundary IAB-node with entries corresponding to “pseudo-BAP address” and “pseudo-path identity”.

Observation 4: Rel-16 data transfer procedure is used for CU1’s non-concatenated traffic. No need to update the routing configuration for CU1’s non-concatenated traffic.

Proposal 2: During inter-donor CU partial migration, following procedures at the boundary IAB-node are considered for upstream traffic:

- the boundary IAB-node adds pseudo-BAP address and pseudo-path ID in the BAP header of the source donor CU’s concatenated access traffic.

- When receiving a packet from its descendant IAB-node, the boundary IAB-node compares the DESTINATION field in the BAP header with its own BAP addresses

- rewrites routing ID in BAP header of source donor CU’s concatenated routing traffic into “pseudo-BAP address and pseudo-path identity”, if DESTINATION field in BAP header does not match boundary IAB-node’s BAP address (which is configured by target donor CU for topology migration or configured by source donor CU for topology redundancy).

Proposal 3: During inter-donor CU partial migration, the source IAB-donor CU reconfigures BAP Routing configuration of IAB-DU at the boundary IAB-node with pseudo “Next Hop BAP address”, representing the next hop IAB-DU in the target topology.

Proposal 4: During inter-donor CU partial migration, the target IAB-donor CU reconfigures BAP Routing Configuration of ancestor IAB-nodes of the boundary IAB-node in the target topology with at least an entry of target IAB-donor DU’s pseudo-BAP address.

Observation 5: When processing CU2’s concatenated traffic via CU1’s topology, the boundary IAB-node may deliver the received packets to the upper layer or discard it due to BAP address collision or mismatch.

Proposal 5: During inter-donor topology migration and topology redundancy, RAN2 confirms F1 termination of the boundary IAB-node and its descendant IAB-node should be the same in Rel-17.

Observation 6: For downstream access traffic of the boundary IAB-node, target IAB-donor DU can add the real BAP address of the boundary IAB-node in the BAP header, as it is configured by CU2.

Proposal 6: For downstream traffic, during inter-donor CU partial migration, the target IAB-donor CU:

- Configures pseudo-BAP addresses and pseudo-path identities to ancestor IAB-nodes of the boundary IAB-node in target topology, representing descendant IAB-nodes of the boundary IAB-node and path in the source topology

- reconfigures BAP Routing configuration of IAB-DU in the target topology with entries corresponding to “pseudo-BAP address” and “pseudo-path identity”.

Proposal 7:To successfully deliver downstream traffic to CU1’s concatenated traffic via CU2 to its destination (e.g. descendant IAB-node of boundary IAB-node), following configurations and procedures are proposed:

- The target IAB-donor CU forwards pseudo-BAP addresses of descendant IAB-nodes of the boundary IAB-node to the source IAB-donor CU via Xn interface

- The source IAB-donor CU configures a BAP header rewriting table, containing the mapping between real and pseudo-BAP address of those descendant IAB-nodes

- The boundary IAB-node performs BAP header rewriting according to the configured BAP header rewriting table if DESTINATION field in the BAP header does not match with boundary IAB-node’s BAP address and also matches one of the pseudo-BAP addresses in the BAP header rewriting table.

- The boundary IAB-node selects the egress BH ink and egress BH RLC Channel ID corresponding to the real DESTINATION BAP address

Proposal 8: SRB3 is not used for F1-C transport in CP/UP separation scenario 2.

Proposal 9: Encapsulated F1-C transport via split SRB2 in scenario 2 can be sent via MN and/or SN as normal UE’s behavior.

# References

- [Post-114-e][088][eIAB] inter-CU routing open issues

- R3-212981, LS on Inter-donor migration

- TS38.340, NR Backhaul Adaptation Protocol (BAP) specification (Rel-16)

- R2-2103083, Report [Post113-e][058][IAB17] Inter-donor topology adaptation