3GPP TSG-RAN WG2 Meeting #116-e	R2-2109785

Online, 1st-12th November 2021

Source: 	Huawei, HiSilicon

Title: 	RLF indication for R17-IAB

Agenda Item:	8.4.3

Document for:	Discussion

- Introduction

In the RAN2#114-e meeting, following agreements were achieved for Type-2 RLF indication [1]:

- The trigger to generate a type 2 RLF indication is at RLF detection. FFS whether for both: single and dual connection cases.

- The trigger for type 3 RLF indication transmission is successful recovery after BH RLF. FFS whether for both: single and dual connection cases.

- Type-2 and Type-3 BH RLF Indications are transmitted via BAP Control PDU.

- Upon reception of the type 2 indication, the IAB node does not initiate RRC re-establishment.

- If an IAB node with dual parents (via DC) receives type-2 BH RLF indication from one parent, IAB-node may trigger a local re-routing to the other parent. The detail of local re-routing and whether/how the action on type-2 indication is configurable is FFS.

In this contribution, the discussion is mainly about the remaining issues on the Type-2 RLF indication, including the granularity and the behaviours upon receiving the Type-2 indication.

- Discussion

  - Terminologies

There was some concern on the terminology of R16 “BH RLF indication” to describe type 4 indication. But, we think changing the R16 terminology is not essential and also requires changes to many spec. Thus, we suggest:

Proposal 1: The terminology of Type-4 indication “BH RLF indication” should NOT be changed in R17.

Proposal 2: RAN2 uses the terminologies of “BH recovering indication” for Type-2 indication and “BH recovered indication” for Type-3 indication.

  - Granularity and trigger of Type-2 RLF indication

The granularity of the Type-2 indication can be per BH link or per routing ID.

- Per BH link

For the single connection case, the IAB-node generates Type-2 indication upon the initiation of RRC re-establishment;

For the dual connection case, the IAB-node generates Type-2 indication upon the initiation of RRC re-establishment, rather than the RLF on one CG;

This is used to inform the child node that the whole BH link is currently recovering and not be able to route any data.

- Per routing ID

If some routing ID is not able to find the backup path after local re-routing (including inter-donor-DU re-routing) if only one CG suffers RLF for a dual connected IAB-node, then the Type-2 indication is sent on that routing ID. This is used to inform child node that data with some specific routing ID may not be able to be routed.

Proposal 3: The granularity of Type-2 indication can be either per BH link or per routing ID.

Proposal 4a: IAB-node may trigger the Type-2 indication upon RLF on any CG.

Proposal 4b: When constructing the Type-2 indication BAP control PDU:

- IAB-node includes the “BH link level” in the triggered Type-2 indication, in case of RRC re-establishment.

- IAB-node includes the “routing ID level” in the triggered Type-2 indication, in case only some routing ID(s) is not be able to be routed to the next hop (e.g. not being able to local re-route due to one CG RLF in NR-DC case).

  - Behaviours upon receiving Type-2 RLF indication

Given the preceding discussion on the granularity of the RLF indication, the IAB-MT behaviour on receiving the Type-2 indication is clear for the single connection case. Upon receiving the BH link-level Type-2 RLF indication, the IAB-MT should suspend any data to the indicated parent link. If the Type-2 indication is for some routing ID(s), the IAB-MT should suspend any data with the indicated routing ID(s). Therefore, it is proposed,

Proposal 5a: IAB-MT with single parent should suspend routing any data to its parent node, upon receiving Type-2 indication on BH link level.

Proposal 5b: IAB-MT with single parent should suspend routing data with the indicated routing ID to its parent node, upon receiving Type-2 indication on routing ID level.

However, for the dual connection case, upon receiving the Type-2 indication from one parent node, it is not clear whether the IAB-MT should suspend the data for this parent node or try local re-routing to another parent node. This largely depends on the link conditions of the upstream IAB-nodes and the previous routing configuration. If both the MCG and SCG are not available in the DC case, the IAB-MT should suspend routing data to its parent node. Thus, we suggest:

Proposal 6a: IAB-MT with NR-DC dual parent does not consider the BH link as available for the purpose of local re-routing, upon receiving Type-2 indication on BH link level.

Proposal 6b: IAB-MT with NR-DC dual parent does not consider the BH link for the indicated routing ID as available for the purpose of local re-routing, upon receiving Type-2 indication on routing ID level on the BH link.

As for the propagation of the Type-2 RLF indication, it depends on the IAB-node behaviour upon receiving the Type-2 indication. If the receiving IAB-node can meet the trigger condition of Type-2 indication, then the receiving IAB-node will send Type-2 indication to its child naturally. And this should not be considered as “propagation”. As long as the Type-2 RLF indication trigger condition is clearly defined at each IAB-node, there is no need to discuss the propagation behaviour.

Therefore, it is proposed,

Proposal 7: RAN2 does not support the propagation of Type-2 indication.

Proposal 8: The discussion on Type-3 indication can be postponed until RAN2 makes clear agreement on the Type-2 indication.

- Conclusion

In this contribution, we mainly discuss the remaining issues on the type-2 RLF indication. It is proposed:

Proposal 1: The terminology of Type-4 indication “BH RLF indication” should NOT be changed in R17.

Proposal 2: RAN2 uses the terminologies of “BH recovering indication” for Type-2 indication and “BH recovered indication” for Type-3 indication.

Proposal 3: The granularity of Type-2 indication can be either per BH link or per routing ID.

Proposal 4a: IAB-node may trigger the Type-2 indication upon RLF on any CG.

Proposal 4b: When constructing the Type-2 indication BAP control PDU:

- IAB-node includes the “BH link level” in the triggered Type-2 indication, in case of RRC re-establishment.

- IAB-node includes the “routing ID level” in the triggered Type-2 indication, in case only some routing ID(s) is not be able to be routed to the next hop (e.g. not being able to local re-route due to one CG RLF in NR-DC case).

Proposal 5a: IAB-MT with single parent should suspend routing any data to its parent node, upon receiving Type-2 indication on BH link level.

Proposal 5b: IAB-MT with single parent should suspend routing data with the indicated routing ID to its parent node, upon receiving Type-2 indication on routing ID level.

Proposal 6a: IAB-MT with NR-DC dual parent does not consider the BH link as available for the purpose of local re-routing, upon receiving Type-2 indication on BH link level.

Proposal 6b: IAB-MT with NR-DC dual parent does not consider the BH link for the indicated routing ID as available for the purpose of local re-routing, upon receiving Type-2 indication on routing ID level on the BH link.

Proposal 7: RAN2 does not support the propagation of Type-2 indication.

Proposal 8: The discussion on Type-3 indication can be postponed until RAN2 makes clear agreement on the Type-2 indication.

- References

- Chairman notes of 3GPP TSG-RAN WG2 meeting #114-e.