3GPP TSG RAN WG2 #116e			R2-2109787

e-Meeting, November 1st – 12th, 2021

Agenda Item:	8.8.2

Source:	BT Plc

Title:	Discussion on slice-based cell reselection prioritization

Document for:	Discussion/Decision

# Introduction

During the last RAN2#115-e meeting [1], the following agreements were made the following agreements leaving for FFS if the end of the slice list has not been reached go back to step 2.

- Agreements

2	Following is taken as the baseline for Solution Option 4:

- The “slice info” (for a single slice or slice group) agreed to be provided to the UE in the last RAN2 meeting using both broadcast and dedicated signaling are provided for the serving as well as neighboring frequencies. The following steps are used for slice based cell (re)selection in AS:

- Step 0: NAS layer at UE provides slice information to AS layer at UE, including slice priorities.

- Step 1: AS sorts slices in priority order starting with highest priority slice.

- Step 2: Select slices in priority order starting with the highest priority slice.

- Step 3: For the selected slice assign priority to frequencies received from network.

- Step 4: Starting with the highest priority frequency, perform measurements (same as legacy).

- Step 5: If the highest ranked cell is suitable (as defined in 38.304) and supports the selected slice in step 2 then camp on the cell and exit this sequence of operation; FFS: How the UE determines whether the highest ranked cell supports the selected slice.

- Step 6: If there are remaining frequencies then go back to step 4.

- Step 7: FFS: If the end of the slice list has not been reached go back to step 2.

- Step 8: Perform legacy cell reselection.

1: Solution Option 4 is selected for further work i.e., resolve the FFSs, send any required LSs and consequently start to draft specification CRs.

This topic has been discussed in a long email discussion [2] where companies are divided. In our previous paper [3], we presented the need for step 7 to provide operators the required flexibility on their deployments.

# Discussion

## Cell reselection prioritization

As we presented in our previous contribution [3], Figure 1 shows the UE behaviour to prioritize the slices based on the “slice -> frequency(ies) -> absolute priority of each of the frequency” information received from the network.

Figure 1 UE sequence diagram to prioritize slice - frequency combo

As part of current agreements [4], it was captured as a note that slice may also mean "slice group" so it is possible to share the same frequency priorities with several slices. In the most extreme use case, an operator will include all the slices in the slice group which implies prioritization is based on frequency. For example, it is possible to engineer the network in a way that the slice group contains all slices supported so the frequency priority will be shared, e.g., [slice_URLLC, Slice_eMBB] -> [freq_A, freq_B, freq_C, freq_D, freq_E] -> [5, 5, 3, 7, 7] if the network only support slice_URLLC and Slice_eMBB

Observation 1: an operator can easily prioritize per slice or per frequency.

Homogenous deployments are assumed in Rel-17 but it does not mean that all the cells in the TA will support the same frequencies. How the slices are deployed in the TA or in TA borders is completely up to operator deployment and frequency continuity cannot be guaranteed under any circumstance.

Observation 3: homogenous deployments do not imply that all cells in the TA will deploy the same frequencies.

The fact that step 7 is not considered as part of the process has operational consequences since the flexibility is reduced to one set of “slice -> frequency(ies) -> absolute priority of each of the frequency”. If step 7 is not considered, operators will reduce their network configurations to a single slice prioritization making observation 1 not valid. More than a couple of very high prioritized slices may create congestion due to all the UEs of prioritized slices will camp in the same frequency of the gNB. A typical deployment with prioritized slices will avoid critical use cases sharing the frequency to avoid congestion problems during the attachment.

For example, an operator will set different frequency priorities to an emergency service network slice, to gaming slice, to AR slice and to default eMBB slice. Without an iteration for highest priority slice - frequency, operators will have two options only. Option 1, all slices must share the same frequencies priorities or option 2, configure only the highest prioritized slice and leave all the other to legacy cell reselection mechanisms. For option 1, it is clear that the same behaviour can be achieved with legacy mechanisms while option 2 is reduced to a single slice. In our understanding, both options are against the spirit of cell reselection enhancement.

As proposed in Figure 1 point 2, the UE can do the measurements only once. After the measurements, it will camp in the most prioritized frequency that fulfils the reselection conditions. From a UE point of view, we don’t see any difference to prioritize slice -frequency or to use legacy procedures if the frequencies to be measured match in both cases.

The fact that one slice(s) – frequencies are prioritized does not mean that a UE will initiate the call on that slice. That can never be guaranteed but it is neither the intention. The main objective is to ensure that if a UE needs to move to connected mode for that specific slice, it already camps in the desired frequency where attachment procedures and the radio resources can be optimized to avoid extra latency in the process and ensure complete slice isolation if required. For BT, reduce the network attachment time is more important than the latency of cell reselection if any extra cell reselection time is added with the new method.

Proposal 1: allow the UE to always camp in the highest priority slice by allowing multiple iterations.

Once the UE extracts the information send by the network, step 0 and step 1 of RAN2#115-e agreements, it may consider its supported slices only and avoid unnecessary operations. This will reduce the power consumption since the UE does not evaluate non-intended slices. If the UE does not support any of the slices indicated, it directly fallback to legacy procedures reducing and no extra changes are required.

Proposal 2: the UE only considers its intended slices.

Proposal 3: if the UE does not support any slice, it will fallback to legacy procedures immediately.

# Conclusion

Observation 1: an operator can easily prioritize per slice or per frequency.

Observation 2: homogenous deployments do not imply that all cells in the TA will deploy the same frequencies.

Proposal 1: allow the UE to always camp in the highest priority slice by allowing multiple iterations.

Proposal 2: the UE only considers its intended slices.

Proposal 3: if the UE does not support any slice, it will fallback to legacy procedures immediately.

# References

- R2-2109xxx, Report of 3GPP TSG RAN WG2 meeting #115-e, Online

- R2-210xxxx [Post115-e][244][Slicing] Resolving FFSs_Summary_Final.docx

- R2-2107929, Discussion on slice-based cell reselection prioritization, Aug 2021

- 3GPP RAN2#115-e, Chairman’s Notes.