3GPP TSG-RAN WG2 Meeting #116-e		R2-2109981

e-Meeting, 1st - 12th November, 2021

Source:	vivo

Title:          Discussion on on-demand PRS

Agenda Item:	8.11.4

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meetings, the following agreements were reached regarding the on-demand PRS.

RAN2 113bis:

- UE-initiated on-demand PRS request is enabled by enhancing LPP RequestAssistanceData.  FFS how much control the network has over the UE request.

- The UE-initiated mechanism is enabled by the UE request triggering a request from the LMF, and the actual PRS changes are requested by the LMF irrespective of whether the procedure is UE- or LMF-initiated.

- Put the stage 2 description for UE-initiated and LMF-initiated PRS request under the same framework.

RAN2 114:

- The network can signal predefined PRS configurations to the UE and the UE can select one to request.  FFS if the UE can request a configuration with different parameters and exactly which parameters are flexible.

- Define a new LPP assistance data IE which can contain a set of possible on-demand DL-PRS configurations, where each on-demand DL-PRS configuration has an associated identifier.

- This new LPP assistance data IE can be included in an LPP Provide Assistance Data message and/or a new posSIB.

- The procedure(s) for on-demand DL-PRS should support at least the following functionality (up to RAN3 what is in NRPPa vs. OAM, etc.):

o	Providing the requested on-demand DL-PRS configuration information from an LMF to the gNB (e.g., explicit parameter or identifier of a predefined DL-PRS configuration), and confirmation of the request by the gNB

o	Provision of (possible/allowed) on-demand DL-PRS configurations that the gNB can support from a gNB to an LMF

o	TRP capability transfer (e.g., whether the RAN node supports the reconfiguration of DL-PRS, etc.)

RAN2 115:

- Before providing available DL-PRS configuration to the UE, the LMF may obtain configuration information on what DL-PRS can be supported from one or more TRPs via NRPPa.

- Capture the steps provided above as a baseline, along with a note indicating it remains FFS if the UE can send the MO-LR to request on-demand PRS.

- FFS if we indicate to SA2 that MO-LR can be used to trigger on-demand PRS procedure.

- It is up to Network (LMF) implementation on the steps to follow (accept/reject/ignore) on receiving request from UE for changing the DL-PRS configurations.

In this contribution, we further discuss the details about the on-demand PRS, including overall procedure, on-demand PRS configuration, and failure of on-demand PRS.

- Discussion

  - Stage 2 baseline procedure for on-demand PRS

According to the post-meeting email discussion [Post115-e][609][POS], the baseline procedure of on-demand PRS is illustrated in Figure 1.

Figure 1 Procedures to support On-Demand PRS transmission

0. The LMF may receive information on the possible On-Demand PRS configurations that the gNB can support during the TRP Configuration Information Exchange procedure. 1. In case of UE-initiated On-demand PRS, the LMF may configure the UE with pre-defined PRS configurations via LPP Provide Assistance Data message or via posSI. 2. In case of UE-initiated on-demand PRS, the UE sends an On-Demand PRS request to the LMF via LPP Request Assistance Data message. The On-Demand PRS request may be a request for PRS transmission or change to the PRS transmission characteristics for positioning measurements. 3. The LMF determines the need for PRS transmission or change to PRS transmission characteristics. In case of LMF-initiated On-demand PRS, the LMF may obtain assistance information, e.g. UE measurements prior to step 3. 4. If the LMF determines to perform on-demand PRS request, the LMF requests the serving and non-serving gNBs/TRPs for new PRS transmission or PRS transmission with changes to the PRS configuration via NRPPa PRS CONFIGURATION REQUEST message. 5. The gNBs/TRPs provide the PRS transmission update in the NRPPa PRS CONFIGURATION RESPONSE message accordingly if the request from the LMF is accepted. 6. LMF provides the updated PRS configuration used for PRS transmission via LPP Provide Assistance Data message or posSI to the UE.

To our understanding, there are two types of PRS resources:

- Type 1: PRS resources that are always transmitted by the gNB, e.g., the PRS resources configured in Rel-16 TRP INFORMATION RESPONSE message.

- Type 2: on-demand PRS resources, i.e., the PRS resources being introduced in Rel-17. The PRS resources are assumed by the UE to be not actually transmitted but can be activated to transmit after requested. Therefore, they are further divided into two:

  - Type 2a: on-demand PRS resources which are not activated (not transmitted).

  - Type 2b: on-demand PRS resources which are activated (being transmitted after requested by the UE or LMF).

PosSIB may broadcast both the Type 1 and Type 2 PRS resources as NR-DL-PRS-AssistanceData. When the Type 2a PRS resources are activated, the Type 2a PRS resources become Type 2b PRS resources. If the PosSIB distinguishes Type 2a PRS resources and Type 2b PRS resources (i.e., the activation/deactivation status of Type 2 resources are reflected in PosSIB), the PosSIB will be updated according to the step 6b in the above stage 2 baseline procedure, which may cause very frequent system information update. It further causes the unnecessary consumption of UE power due to the reading of updated system information. It does so in case of the deactivation of the on-demand PRS. Therefore, we suggest that Type 2a PRS resources and Type 2b PRS resources are not distinguished in the PosSIB, and LMF may only provide updated PRS configuration via LPP message in step 6, i.e., remove ‘or posSI’ in step 6.

Observation 1: The PRS resource type can be divided into the following:

- Type 1: PRS resources that are always transmitted by the gNB.

- Type 2: on-demand PRS resources. They are further divided into two:

  - Type 2a: on-demand PRS resources which are not activated (not transmitted).

  - Type 2b: on-demand PRS resources which are activated (being transmitted).

Proposal 1: No need to distinguish the activated on-demand PRS resources and inactivated on-demand PRS resources in the PosSIB.

Proposal 2: LMF should not inform which on-demand PRS configuration is activated via posSIB.

  - On-demand PRS request with MO-LR

In the post-meeting email discussion [Post115-e][POS], most companies supported the on-demand PRS request with MO-LR in case of the MO-LR with the MOLR-Type set to 'assistanceData' for autonomous self-location. Besides, in the clause 7.3.3 “MO-LR Service Support” of the TS38.305 [1], it has been described that “The MO-LR location service request message may carry an LPP PDU to instigate one or more LPP procedures to transfer capabilities, request assistance data, and/or transfer location information”. The current specification has supported on-demand PRS request with MO-LR.

One question is whether we need to restrict the case as autonomous self-location (exclude other cases) in the specification. In the last RAN2 meeting, RAN2 has reached the agreement of “It is up to Network (LMF) implementation on the steps to follow (accept/reject/ignore) on receiving request from UE for changing the DL-PRS configurations”. Anyway, the network has the flexibility to accept/reject/ignore the request, e.g., based on best efforts or any other implementation strategy. Therefore, we suggest:

Proposal 3: For on-demand PRS requests in MO-LR, other cases than the autonomous self-location is not excluded.

  - Configuration of on-demand PRS

After the UE gets the configuration of possible available on-demand DL-PRS, the UE should store it for the measurement at some future time. Therefore, a timer should be introduced. Before the timer expires, the UE shall use it for the procedure of on-demand PRS. Meanwhile, the configuration of possible available on-demand DL-PRS containing the PRS configuration of some TRPs is valid within a specific area. When the UE gets out of the area due to mobility, the configuration will turn invalid.

Proposal 4: The configuration of possible available on-demand DL-PRS is valid within a specific area and period.

Including some explicit parameters in the on-demand PRS configuration request may increase the flexibility of on-demand PRS. However, to ensure that the LMF can handle the requests from multiple UEs, only a subset of parameters in LPP IE NR DL-PRS-AssistanceData can be explicitly on-demand. The subset of parameters may rely on RAN1 decision.

Proposal 5: Some explicit parameters can be included in the on-demand PRS configuration request.

Further, to support mobility, the configuration of possible available on-demand DL-PRS may include a list of available PRS configurations associated with different areas. For instance, in Figure 2, the available on-demand DL-PRS configuration (i.e. list 1) of TRP1~3 can associate with Area 1 and the available on-demand DL-PRS configuration (i.e. list 2) of TRP 4~6 can associate with Area 2. When the target UE is served by TRP1, the UE can request the preferred PRS configuration in list 1. After the UE moves from Area1 to Area 2 and is served by TRP4, the UE can request the preferred PRS configuration in list 2. With this, the configuration of possible available on-demand DL-PRS of different areas can be sent to target UE at once.

Figure 2 overall procedure of on-demand PRS

Proposal 6: The configuration of possible available on-demand DL-PRS can include a list available PRS configurations associated with different areas (e.g., distinguished by cell ID).

  - Failure of on-demand PRS

As illustrated in Figure 1, LMF shall activate the updated PRS configuration to UE after the TRP response to the LMF that the requested on-demand PRS is acknowledged.

If the TRP rejects the request of PRS configuration update in step 6 due to own reason, e.g., available DL-PRS resources change, the LMF should send an LPP ProvideAssistanceData message to UE to inform the failure of on-demand PRS with the cause ‘assistanceDataSupportedButCurrentlyNotAvailableByServer’ in the corresponding location method.

Meanwhile, to avoid the target UE request for the on-demand PRS again soon after the former request fails, one timer can be configured along with the error cause in the ProvideAssistanceData message. The UE can request another on-demand PRS when the timer expires.

– NR-DL-TDOA-ProvideAssistanceData The IE NR-DL-TDOA-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR DL-TDOA. It may also be used to provide NR DL-TDOA positioning specific error reason. -- ASN1START NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE { nr-DL-PRS-AssistanceData-r16 NR-DL-PRS-AssistanceData-r16 OPTIONAL, -- Need ON nr-SelectedDL-PRS-IndexList-r16 NR-SelectedDL-PRS-IndexList-r16 OPTIONAL, -- Need ON nr-PositionCalculationAssistance-r16 NR-PositionCalculationAssistance-r16 OPTIONAL,-- Cond UEB nr-DL-TDOA-Error-r16 NR-DL-TDOA-Error-r16 OPTIONAL, -- Need ON ... nr-On-Demand-PRS-Request-Timer NR-On-Demand-PRS-Request-Timer OPTIONAL, } -- ASN1STOP

Proposal 7: If the requested on-demand PRS is not available, the LMF shall return an error indication and a timer to prevent the UE from requesting on-demand PRS again in a specific duration.

- Conclusion

Based on the discussion, we propose the following:

Observation 1: The PRS resource type can be divided into the following:

- Type 1: PRS resources that are always transmitted by the gNB.

- Type 2: on-demand PRS resources. They are further divided into two:

  - Type 2a: on-demand PRS resources which are not activated (not transmitted).

  - Type 2b: on-demand PRS resources which are activated (being transmitted).

Proposal 1: No need to distinguish the activated on-demand PRS resources and inactivated on-demand PRS resources in the PosSIB.

Proposal 2: LMF should not inform which on-demand PRS configuration is activated via posSIB.

Proposal 3: For on-demand PRS requests in MO-LR, other cases than the autonomous self-location is not excluded.

Proposal 4: The configuration of possible available on-demand DL-PRS is valid within a specific area and period.

Proposal 5: Some explicit parameters can be included in the on-demand PRS configuration request.

Proposal 6: The configuration of possible available on-demand DL-PRS can include a list available PRS configurations associated with different areas (e.g., distinguished by cell ID).

Proposal 7: If the requested on-demand PRS is not available, the LMF shall return an error indication and a timer to prevent the UE from requesting on-demand PRS again in a specific duration.

- Reference

- TS38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN