3GPP TSG-RAN WG2 Meeting #116-e	R2-2110058

# Electronic, Novermber 1-12, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1167 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the usage of sl-ReselectAfter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, OPPO, Qualcomm Incorporated, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | #### In LTE V2X, only blind HARQ retransmissions are supported. As a result, every transmission opportunity reserved by a TX UE are supposed to be used. However, in NR V2X, transmission resources reserved will be skipped by TX UE if no retransmission is needed based on positive HARQ feedback. Hence, the “sl-reselectAfter” shall not count those transmission opportunities which are not used due to HARQ feedback. As the value is only configured in 1 to 9 in RRC spec, this is very easy to be mis-triggered. sl-ReselectAfter-r16 ENUMERATED {n1, n2, n3, n4, n5, n6, n7, n8, n9} OPTIONAL, -- Need R #### For example, resource re-selection will be triggered whenever a retransmission resource reserved but not used due to HARQ ACK, if n1 is configured for sl-ReselectAfter. This is definitely not the correct behavior. Also, in NR V2X, it would difficulty to take re-transmission into account for this, considering all the features of PSFCH, pre-emption/re-evaluation, etc. #### Based on RAN1 LS R1-2108438, RAN1’s understadning of this issue is as follow: #### “Only if none of the resources of a grant in a resource reservation period is used for (re)transmission, no matter whether they are indicated in SCI, it shall be counted towards one of consecutive unused transmission opportunities to trigger resource reselection”. #### Based on the above understanding, the fix is to clarify that each unit of this value is incremented by 1 when “none of the resources of a SL grant in each resource reservation period is used”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Explained that the UE comapres the sl-ReslectAfter threshold with “the number of consecutive unsued transmission opportunity which is incremented by 1 only when none of the resources of the selected sidelink grant within a resource reservation interval is used” Impact analysis Impacted architecture options: NR SL Impacted functionality: Resource Reselection Inter-operability: If a UE implements the CR and the NW does not, then there is no interoperability issue. If one Sidleink UE implements the CR but another Sidelink UE does not, there is no inter-operabilty issue because the impact of one SL UE’s resource (re)selection has been taken into account by another SL UE’s sensing procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mode 2 resource reselection will be erroneously triggered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 5.22.1.2	TX resource (re-)selection check

If the TX resource (re-)selection check procedure is triggered on the selected pool of resources for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process:

1>	if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by RRC in sl-ProbResourceKeep; or

1>	if the pool of resources is configured or reconfigured by RRC; or

1>	if there is no selected sidelink grant on the selected pool of resources; or

1>	if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the selected sidelink grant during the last second; or

1>	if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant, which is incremented by 1 when none of the resources of the selected sidelink grant within a resource reservation interval is used, is equal to sl-ReselectAfter; or

1>	if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU; or

NOTE 1:	If the selected sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection.

1>	if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical channel according to the associated priority, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU:

NOTE 2:	If the remaining PDB is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection.

NOTE 3:	It is left for UE implementation whether to trigger the TX resource (re-)selection due to the latency requirement of the MAC CE triggered according to clause 5.22.1.7.

2>	clear the selected sidelink grant associated to the Sidelink process, if available;

2>	trigger the TX resource (re-)selection.

NOTE 4:	Void.

NOTE 5:	Void.

End of Changes