3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110309

Online, November 1 - November 12, 2021                     Revision of R2-2107910

Agenda item:		8.10.3.2

Source:		Lenovo, Motorola Mobility

Title:		Considerations on ephemeris provision for NTN

Document for:		Discussion

- Introduction

The RAN1#105 meeting made agreements on ephemeris format(s) to support broadcasting position/velocity state vectors and/or orbital parameters. Although FFS remains regarding field size and accuracy, it is better to start RAN2 discussion on ephemeris provision (e.g. serving and neighboring ephemeris via broadcasting) considering the progress of Rel-17 WI and the impact of ephemeris provision on connected/idle mobility. This contribution focuses on other issues including ephemeris provision for serving and neighboring cells, signalling overhead and implicit indication of network type/platform via ephemeris.

- Discussion

The RAN2#113e meeting made the following agreements regarding ephemeris and network type indication for NTN:

- The NTN ephemeris is divided into serving cell’s ephemeris and neighbour’s ephemeris. FFS how would they differ regarding e.g. the required accuracy or signalling impact.

- Consider pre-configuration in uSIM, NAS, SIB and RRC signalling for providing the NTN ephemeris. Further discussion depends on the agreed ephemeris contents.

- RAN2 thinks that a UE needs to know whether the network is a TN or NTN no later than SIB1 reception.

And the RAN1#105 meeting made the following agreements regarding NTN ephemeris format:

Specifications should support delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements.

In this contribution we further discuss ephemeris provision of serving and neighboring NTN cells (e.g. as a satellite/HAP constellation). The ephemeris data is used to indicate the trajectories and/or position coordinates of satellites/HAPs, so that the UE can perform TA pre-compensation, location/time-based handover and cell reselection. As RAN1 have made agreements on broadcasted ephemeris format, it is expected from RAN2 perspective to discuss how to include the ephemeris in SIB(s), and possibly indicating network type or scenario via ephemeris.

Ephemeris size and signalling overhead

As indicated in TR38.821 for Set 2 (orbital parameter ephemeris), the minimum ephemeris data of a single satellite needs at least seven double-precision floating point numbers plus some overhead, which is more than 7*8Byte=56Byte (including 5*8Byte =40Byte orbital plane parameters and 2*8Byte =16Byte satellite level parameters), and representation of Set 1 (position and velocity state vectors) requires more considering the accuracy. To assist in connected/idle mobility, ephemeris provision of the neighboring NTN cells is necessary, and the scale of the whole ephemeris data can be quite substantial for system information broadcasting and UE reception. Considering that a satellite/HAP constellation belonging to the same operator may typically share a common orbital plane or altitude, and may operate with organized and fixed intervals of distance, time or angle, there is no need to indicate the full ephemeris for each satellite/HAP in it. From this perspective the ephemeris for a satellite/HAP constellation can be compressed.

Observation 1: At least for a satellite/HAP constellation belong to the same operator, there is no need to indicate the full ephemeris for each satellite/HAP.

Proposal 1: RAN2 to consider minimizing the size of ephemeris for serving and neighboring NTN cells in a satellite/HAP constellation.

As RAN2 have agreed to individually indicate ephemeris of serving and neighboring cells, a simple and feasible way is to provide the full ephemeris of serving NTN cell as reference, and the ephemeris of its neighboring NTN cell in a relative manner. For instance, a serving HAP #1 can broadcast its full Set 1 ephemeris as absolute position (X1,Y1,Z1) and velocity (VX1,VY1,VZ1), while the ephemeris of its neighboring HAP #2 can be represented as relative position (ΔX2=X2-X1, ΔY1=Y2-Y1, ΔZ1=Z2-Z1) and velocity (ΔVX2=VX2-VX1, ΔVY2=VY2-VY1, ΔVZ2=VZ2-VZ1) to HAP #1, wherein the relative altitude or velocity can be zero and thus omitted. Similarly a serving satellite #1 can broadcast its full Set 2 ephemeris as absolute values (α1, e1, ω1, Ω1, i1, M1, to1), while the ephemeris of its neighboring satellite #2 can be represented as relative values (Δα2=α2-α1, Δe2=e2-e1, Δω2=ω2-ω1, ΔΩ2=Ω2-Ω1, Δi2=i2-i1, ΔM2=M2-M1, Δto2=to2-to1), wherein the relative orbit plane parameters (Δα2, Δe2, Δω2, ΔΩ2, Δi2)can be zero and thus omitted.

Observation 2: The ephemeris of neighboring cells can be represented in a relative manner comparing to the serving cell for Set 1 and/or Set 2, reducing the overhead of broadcasting.

Proposal 2: For a satellite/HAP constellation, ephemeris of neighboring cells can be indicated as relative values to that of the serving cell.

For a satellite/HAP constellation, it is likely that the ephemeris is not frequently updated, e.g. HAPs may operate at fixed locations or velocities, while satellites may operate at fixed orbit(s) or with fixed intervals. In this case it is unnecessary for a UE to repeat acquiring and decoding the ephemeris of the satellite/HAP constellation every time it accesses one in it. A UE can skip obtaining the same ephemeris of a satellite/HAP constellation if there is no update. This can be easily implemented by making the UE aware of the constellation (e.g. a value tag for a group of satellites/HAPs). And the value tag can also be changed upon ephemeris update to indicate UE for updating.

Proposal 3: The UE can skip obtaining or decoding the same ephemeris of a satellite/HAP constellation if there is no update.

Implicit indication of network type/platform via ephemeris

To further assist in mobility, there have been discussions for indication of network types (NTN or TN) and platforms (GEO or LEO or HAP). Here we discuss the option to indicate NTN network type/platform implicitly via ephemeris, without introducing in explicit indicators.

To implicitly indicate network type, UE needs to know the presence of ephemeris no later than SIB1 reception. As a result, either an index indicating presence of ephemeris or the ephemeris itself should be included in SIB1.

Observation 3: To utilize ephemeris for implicit network type indication, either presence of ephemeris is indicated in SIB1, or ephemeris is included in SIB1.

Ephemeris is expected to be used in timing advance pre-compensation where UE can derive its serving link propagation delay based on its location and satellite position. Considering the importance of accurate timing advance when initiating random access, it is expected that serving cell’s ephemeris should be provisioned to UE in SIB1, or at least UE knows how to retrieve serving cell’s ephemeris when receiving SIB1.

Observation 4: To utilize ephemeris for TA pre-compensation, either serving cell’s ephemeris is provisioned to UE in SIB1, or index of serving cell’s ephemeris is provisioned to UE in SIB1 and serving cell’s ephemeris is provisioned to UE in a new SIB.

Another usage of ephemeris in NTN is for cell reselection. While the RSRP/RSRQ variation could be less significant in NTN, ephemeris can help idle/inactive UE to trigger neighboring cell measurement or perform cell reselection using time or distance metric. In this case neighboring cell’s ephemeris can be seen as necessary or supplement information for cell reselection, and should be included in SIB2~SIB5, or at least UE knows how to retrieve neighboring cell’s ephemeris when receiving SIB2~SIB5.

Observation 5: To utilize ephemeris for cell reselection, either neighboring cell’s ephemeris is provisioned to UE in SIB1~5, or index of neighboring cell’s ephemeris is provisioned to UE in SIB1~5 and neighboring cell’s ephemeris is provisioned to UE in a new SIB.

Based on the discussion above the following options could be considered for ephemeris provision via SI:

- Option 1: index of serving cell’s ephemeris in SIB1, index of neighboring cell’s ephemeris in SIB2 (or SIB3~5 depending on the frequency/RAT of NTN), and serving cell’s and neighboring cell’s ephemeris in a new SIB. This option is for large ephemeris size so that the UE could know network type with SIB1, and acquire ephemeris in the new SIB on-demand.

- Option 2: serving cell’s ephemeris in SIB1, index of neighboring cell’s ephemeris in SIB2 (or SIB3~5 depending on the frequency/RAT of NTN), and neighboring cell’s ephemeris in a new SIB. This option is for medium ephemeris size so that the UE could know network type and receive serving cell’s ephemeris with SIB1, and acquire other ephemeris in the new SIB on-demand.

- Option 3: serving cell’s ephemeris in SIB1, and neighboring cell’s ephemeris in SIB2 (or SIB3~5 depending on the frequency/RAT of NTN). This option is for small ephemeris size so that the UE could know network type and receive serving cell’s ephemeris with SIB1, and receive other ephemeris in SIB2 for cell reselection.

Proposal 4: RAN2 to consider one of the options for ephemeris provision in SIB.

- Conclusion

In this contribution, ephemeris provision for serving and neighboring cells is analyzed considering signalling overhead, and the options of implicit indication for network type/platform via ephemeris are discussed. The following observation are given:

Observation 1: At least for a satellite/HAP constellation belong to the same operator, there is no need to indicate the full ephemeris for each satellite/HAP.

Observation 2: The ephemeris of neighboring cells can be represented in a relative manner comparing to the serving cell for Set 1 and/or Set 2, reducing the overhead of broadcasting.

Observation 3: To utilize ephemeris for implicit network type indication, either presence of ephemeris is indicated in SIB1, or ephemeris is included in SIB1.

Observation 4: To utilize ephemeris for TA pre-compensation, either serving cell’s ephemeris is provisioned to UE in SIB1, or index of serving cell’s ephemeris is provisioned to UE in SIB1 and serving cell’s ephemeris is provisioned to UE in a new SIB.

Observation 5: To utilize ephemeris for cell reselection, either neighboring cell’s ephemeris is provisioned to UE in SIB1~5, or index of neighboring cell’s ephemeris is provisioned to UE in SIB1~5 and neighboring cell’s ephemeris is provisioned to UE in a new SIB.

And we propose:

Proposal 1: RAN2 to consider minimizing the size of ephemeris for serving and neighboring NTN cells in a satellite/HAP constellation.

Proposal 2: For a satellite/HAP constellation, ephemeris of neighboring cells can be indicated as relative values to that of the serving cell.

Proposal 3: The UE can skip obtaining or decoding the same ephemeris of a satellite/HAP constellation if there is no update.

Proposal 4: RAN2 to consider one of the options for ephemeris provision in SIB.

- Option 1: index of serving cell’s ephemeris in SIB1, index of neighboring cell’s ephemeris in SIB2 (or SIB3~5 depending on the frequency/RAT of NTN), and serving cell’s and neighboring cell’s ephemeris in a new SIB.

- Option 2: serving cell’s ephemeris in SIB1, index of neighboring cell’s ephemeris in SIB2 (or SIB3~5 depending on the frequency/RAT of NTN), and neighboring cell’s ephemeris in a new SIB.

- Option 3: serving cell’s ephemeris in SIB1, and neighboring cell’s ephemeris in SIB2 (or SIB3~5 depending on the frequency/RAT of NTN).