3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-2110369

1 – 12 November 2021

Agenda item:	8.16.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations for IMS emergency support indicator

WID/SID:	NG_RAN_PRN_enh - Release 17

Document for:	Discussion and Decision

# 1	Introduction

During the previous RAN2 meetings it has been agreed that broadcasting IMS emergency support in SNPNs should be enabled. It remained open if the new indication is per cell or per SNPN. After receiving reply LS from CT1 (C1-215046) this issue could be closed.

# 2	Discussion

CT1 provided the following answer in the reply LS R2-2109306:

In order for a UE, which is in the limited service state and needs to obtain emergency services, to select an SNPN which supports emergency services, if not all SNPNs sharing a cell support emergency services, the UE needs to be aware of which SNPNs support emergency services

Our view is that CT1 requirement highlighted by yellow can only be met if the new IMS emergency support indication is per SNPN.

Proposal 1: The new IMS emergency support indication is per SNPN.

There are at least two options how to add the new indicators to SIB1:

1)	In the top level SIB1 extension, in a way as presented in the example below:

SIB1-v17xx-IEs ::=           SEQUENCE {

ims-snpn-EmergencyList-r17    SEQUENCE (SIZE (1..maxNPN-r16)) OF IMS-SNPN-Emergency-r17    OPTIONAL,    -- Need R

nonCriticalExtension             SEQUENCE {}                                                        OPTIONAL

}

IMS-SNPN-emergency-r17 ::=         SEQUENCE {

ims-SNPN-EmergencySupport-r17       ENUMERATED {true}             OPTIONAL,   -- Need R

}

2)	Into the new extension that contains the other new SIB indicators that are per SNPN, in a way as presented in the example below:

SNPN-AccessInfo-r17 ::=         SEQUENCE {

extCH-Supported-r17                 ENUMERATED {true}             OPTIONAL,   -- Need R

extCH-WithoutConfigAllowed-r17      ENUMERATED {true}             OPTIONAL,   -- Need R

onboardingEnabled-r17               ENUMERATED {true}             OPTIONAL,   -- Need R

ims-SNPN-EmergencySupport-r17       ENUMERATED {true}             OPTIONAL,   -- Need R

}

We think that both options are feasible, and there is no difference in them from overhead perspective. As in case of option 2) all new indicators for SNPNs are in a single location, we think that option 2) makes the SIB1 structure easier to follow.

Proposal 2: Add the new IMS emergency support indicator in the structure where the other new SNPN indicators are located.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: The new IMS emergency support indication is per SNPN.

Proposal 2: Add the new IMS emergency support indicator in the structure where the other new SNPN indicators are located.