3GPP TSG-RAN WG2 Meeting #116                  	R2-2110394

E-Meeting, 1st November– 12th November, 2021

Source:	vivo

Title:	Remaining issues for paging with service indication

Agenda Item:	8.3.4

Document for:	Discussion and Decision

- Introduction

According to the RAN WID of MUSIM, one of the objectives is following:

- Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR [RAN2]. - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

In RAN2#113bis meeting, RAN2 received an Ls from SA2, in which a question is raised about how the UE discriminates the below cases [1]:

SA2 has concluded that the UE shall be able to discriminate the case where it is being paged for non-voice service from the case where it is being paged (for any service) by a RAN node not supporting the Paging Cause, either because the RAN node does not support the Paging Cause feature, or because in case of RAN sharing it is configured to operate without the Paging Cause feature for some of the connected CNs. SA2 would like to ask RAN2 to take into account the above information and provide feedback if necessary.

During RAN2#113bis meeting, RAN2 has made some related agreements, as below.

- 1: RAN2 works to support the MUSIM paging cause feature that SA2 is working on and also addresses the paging cause issue raised by SA2 LS. - 2: RAN2 attempts to reply LS to SA2 once we progress on solution and agree on CR(s) that support/address the above feature/issue. - 5: If RAN2 agrees to add a paging cause value (or any other information that could lead to a specific paging cause) in Uu paging message, RAN2 specifies the relevant UE behavior (i.e. inform or passing to the upper layer) upon its reception in both LTE and NR specifications. - RAN2 does not intend to introduce alternative paging IDs for MUSIM paging (unless requested by SA2).

Recently, CT1 sent a reply LS on NAS-based busy indication [2], and asked RAN2’s feedback on the interaction between the NAS layer and the AS layer when the AS layer receives RAN paging. In the LS, CT1 provided the below two options and mentioned that there could be other possible options that CT1 has not discussed. In addition, CT1 also asked RAN2 if a paging cause, if any, indicated together with indication about RAN paging from AS layer to NAS layer.

Option 1: the AS layer resumes the RRC connection upon receipt of RAN paging and then the AS layer informs the NAS layer an indication that the UE has transitioned to RRC_CONNECTED state and indication about the RAN paging;

Option 2: the AS layer informs the NAS layer an indication about the RAN paging and the AS layer resumes the RRC connection based on a request from the NAS layer to the AS layer to transition to RRC_CONNECTED state (the request from NAS layer is triggered by acceptance of RAN paging or the SERVICE REQUEST message containing the “NAS signalling connection release" indication).

All the above issues have been discussed in the RAN2#115 Post-meeting email discussion, and the below proposals were given in the summary [3].

“Proposal 1: Introduce paging cause by using the ”nonCriticalExtension” in the Paging record.

Proposal 2: No need to study solution proposals based on extending legacy Paging record.

Proposal 3: FFS if B.1 (parallel list with 1 paging cause value “voice”) or B.2 (parallel list with 2 paging cause values “voice, other”) is the preferred ASN.1 coding approach.

Proposal 4: The solution proposal to introduce paging cause in NR will be used for LTE.

Proposal 5: No need to send an LS to SA2 asking to consider a NAS solution to introduce paging cause in LTE.

Proposal 6: For paging reception in RRC_IDLE, UE forwards the paging cause to NAS.  It’s up to NAS whether to accept or reject the paging.

Proposal 7: For the AS-NAS interaction for paging reception in RRC_INACTIVE, FFS Option 2 or Option 3 (i.e. up to UE implementation) is the preferred solution.

Proposal 8: The AS-NAS interaction principles for NR are applied to LTE.

Proposal 9: Introduction of paging cause impacts 38.331 and 36.331 specs; FFS if it impacts stage 2 specs (38.300 and 36.300)”

This paper would like to further discuss the above remaining issues.

- Discussion

2.1 ASN.1 coding approach

During the email discussion [3], parallel list solutions (i.e., network puts the paging causes in a new defined parallel paging record list PagingRecordList-v17xy, which includes the same number of entries, and listed in the same order, as in legacy pagingRecordList) got majority support, however, there is no consensus on whether to have one cause value or two cause values for pagingCause-r17.

“Rapporteur’s proposal3: FFS if B.1 (parallel list with 1 paging cause value “voice”) or B.2 (parallel list with 2 paging cause values “voice, other”) is the preferred ASN.1 coding approach.”

According to the below text in TS 23.502 [4], the UE will be informed about whether its serving CN support paging cause indication for voice feature via NAS signaling for 5GS (The same is true for EPS, as can been seen in TS 23.401).

4.2.2.2.2 General Registration <Omit…> 21. New AMF to UE: Registration Accept (5G-GUTI, Registration Area, [Mobility restrictions], [PDU Session status], [Allowed NSSAI], <Omit…>, [PLMN-assigned UE Radio Capability ID deletion], [WUS Assistance Information], [Truncated 5G-S-TMSI Configuration], [Connection Release Supported], [Paging Cause Indication for Voice Service Supported], [Paging Restriction Supported], [Reject Paging Supported]). <Omit…>

With this core network support indication, a MUSIM UE understands the service from the network as following:

- If its pagingCause-r17 is present, the incoming MT call is voice service (VoLTE or VoNR);

- If CN doesn’t include “Paging Cause Indication for Voice Service Supported” in NAS message, the incoming MT call is any service;

- If CN includes “Paging Cause Indication for Voice Service Supported” in NAS message:

  - If PagingRecordList-v17xy is absent, the incoming MT call is any service;

  - If PagingRecordList-v17xy is present while the UE’s pagingCause-r17 is absent, the incoming MT call is non-voice.

Given above, one cause value for pagingCause-r17 is enough. This solution does not seem to bring much complexity, so it is not worth defining two cause values (i.e., introduce one bit per each paging record) for simplicity. Thus, we propose the following:

- Parallel list with 1 paging cause value “voice” is the preferred solution.

2.2 AS-NAS interaction

According to the below description in TS 23.501 [5] (the same is true for EPS, as can be found in TS 23.401), how the UE distinguishes the Paging from a network that does not support the Paging Cause Indication for Voice Service feature and Paging without the Voice Service Indication will be defined in RAN specification.

5.38.3 Paging Cause Indication for Voice Service <Omit…> UE that supports the Paging Cause Indication for Voice Service feature is capable of differentiation between Paging from a network that does not support the Paging Cause Indication for Voice Service feature and Paging without the Voice Service Indication. How the UE distinguishes the Paging from a network that does not support the Paging Cause Indication for Voice Service feature and Paging without the Voice Service Indication is defined in TS 38.331 [28].

In order to enable the UE distinguishes the Paging from a network that does not support the Paging Cause Indication for Voice Service feature and Paging without the Voice Service Indication, the UE NAS should indicate CN support indication (i.e., whether its serving CN support paging cause indication for voice feature) to the UE AS. Thus, we propose the following:

- UE NAS should indicate CN support indication (i.e., whether its serving CN support paging cause indication for voice feature) to the UE AS.

Moreover, in the email discussion [3], there is a consensus that for CN paging in RRC_IDLE, UE forwards the paging cause to NAS. It’s up to NAS whether to accept or reject the paging. For RAN paging in RRC_INACTIVE, the below options were discussed during the email discussion.

Option 1: the AS layer resumes the RRC connection upon receipt of RAN paging and then the AS layer informs the NAS layer an indication that the UE has transitioned to RRC_CONNECTED state and indication about the RAN paging;

Option 2: the AS layer informs the NAS layer an indication about the RAN paging and the AS layer resumes the RRC connection based on a request from the NAS layer to the AS layer to transition to RRC_CONNECTED state (the request from NAS layer is triggered by acceptance of RAN paging or the SERVICE REQUEST message containing the “NAS signalling connection release" indication);

Option 3: No need to specify AS-NAS interaction, up to UE implementation;

Option 4: Others, please give details if chosen this option.

However, more discussion is needed to decide whether to select option 2 or option 3 [3].

“Rapporteur’s Proposal 7: For the AS-NAS interaction for paging reception in RRC_INACTIVE, FFS Option 2 or Option 3 (i.e. up to UE implementation) is the preferred solution.”

For RRC_INACTIVE, upper layer may know the service type in another network and if it also receives the paging cause from UE AS, upper layer can decide more reasonably whether to accept or reject the paging. Thus, we suggest that the UE AS informs the paging cause to the UE NAS when receiving RAN paging in RRC_INACTIVE, which is also aligned with the UE behavior when receiving CN paging in RRC_IDLE. Thus, we have the following:

- For both RRC_IDLE and RRC_INACTIVE states, if paging cause is indicated by the network, the UE AS forwards the paging cause to UE NAS.

Upper layer may decide not to send busy indication if it judges the UE cannot do this, e.g., upper layer knows the service in another network is delay sensitive. In this case, the RRC connection resume procedure should not be initiated. Otherwise, the UE have to perform the network switching procedure for leaving RRC_CONNECTED, which may cause additional delay for the delay sensitive service in another network. Thus, option 2 seems more reasonable and can be specified to avoid such unexpected case. But for option2, if paging cause will be informed to NAS layer by AS layer, it seems no need to inform RAN paging indication to NAS layer. And if the paging service deduced by AS layer is any service, there is no need to inform RAN paging and/or paging cause to upper layer, that is, the UE can follow legacy procedure. Given above, we propose the following:

- Upon receiving RAN paging in RRC_INACTIVE, AS layer informs paging cause (voice, non-voice) to NAS layer.

- After informing paging cause to NAS, RRC_ INACTIVE UE only resumes RRC connection based on the request from NAS(i.e. the request from NAS is triggered by acceptance of RAN paging or the SERVICE REQUEST message containing the “NAS signalling connection release" indication).

- Reply to CT1 that RAN2’s preference on the AS-NAS interaction for paging with service indication feature.

- Conclusion

According to the LS from SA2 and the enhancements already proposed in RAN2 for paging cause, we have the following observations:

- Parallel list with 1 paging cause value “voice” is the preferred solution.

- UE NAS should indicate CN support indication (i.e., whether its serving CN support paging cause indication for voice feature) to the UE AS.

- For both RRC_IDLE and RRC_INACTIVE states, if paging cause is indicated by the network, the UE AS forwards the paging cause to UE NAS.

- Upon receiving RAN paging in RRC_INACTIVE, AS layer informs paging cause (voice, non-voice) to NAS layer.

- After informing paging cause to NAS, RRC_ INACTIVE UE only resumes RRC connection based on the request from NAS(i.e. the request from NAS is triggered by acceptance of RAN paging or the SERVICE REQUEST message containing the “NAS signalling connection release" indication).

- Reply to CT1 that RAN2’s preference on the AS-NAS interaction for paging with service indication feature.

- References

- S2-2102039, “LS on System support for Multi-USIM devices”, LS to RAN2, Intel Corporation

- C1-214917, Reply LS on NAS-based busy indication

- [Post115-e][236][MUSIM] Paging with service indication, Huawei/HiSilicon

- TS 23.502-h21.

- TS 23.501-h20.