3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2110552

Elbonia, 16 – 27 August 2021

Agenda item:	8.1.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Broadcast Service Continuity

WID/SID:	NR_MBS-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

Support for MBMS was introduced to E-UTRAN in Release 9 via single frequency network mode of operation (MBSFN), which could be provided on a same frequency layer as regular unicast. In NR multicast services are to be supported as well as broadcast services, but these are not provided via MBSFN

in RAN2#114-e we agreed:

One-to-one mapping between G-RNTI and MBS session is supported in NR MBS. Other mappings FFS

One-to-one mapping between G-CS-RNTI and MBS session is supported in NR MBS. Other mappings FFS.

A UE can support multiple G-RNTIs/G-CS-RNTIs, It is FFS whether this depends on UE capability. Inform RAN1 of this agreement.

Multiple MBS QoS flows corresponding to the same MBS session can be mapped to one or more than one MBS radio bearers.

MCCH is mapped to the DL-SCH for NR MBS delivery mode 2.

MTCH is specified for PTM transmission of NR MBS.

MTCH is mapped to the DL-SCH.

DTCH is reused for PTP transmission of NR MBS.

FFS if there is a need to have specific LCID spaces for the used channels.

Multiplexing/de-multiplexing of different logical channels associated with the same G-RNTI is supported for NR MBS.

FFS if Multiplexing/de-multiplexing of different logical channels associated with the same G-CS-RNTI is supported for NR MBS.

Multiplexing/de-multiplexing of different logical channels associated with the C-RNTI is supported for NR MBS.

For NR MBS delivery mode 2, LTE SC-PTM DRX scheme is used as baseline.

FFS whether For PTM transmission of NR MBS, DRX scheme is independent of DRX for unicast transmission, e.g. supported on a per G-RNTI basis

FFS whether For PTP transmission, DRX operation for unicast transmission is reused

MBS specific SIB is defined to carry MCCH configuration.

MCCH contents should include information about broadcast sessions such as G-RNTI, MBS session ID as well as scheduling information for MTCH (e.g. search space, DRX). L1 parameters that need to be included in MCCH are pending further RAN1 progress and input.

Postpone the discussion on whether dedicated MCCH configuration is required until RAN1 makes progress on BWP/CFR for MCCH.

Indication of an MCCH change due to modification of an ongoing session’s configuration (including session stop) is provided with an explicit notification from the network  (provided that RAN1 confirms a separate bit for this purpose can be accommodated in the MCCH change notification DCI, in addition to a bit for session start notification). FFS on whether this notification can be reused for modification of other information carried by MCCH, if any.

FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation.

At least in case RAN1 decides to utilize RNTI other than MCCH-RNTI for MCCH change notification, MCCH change notification is sent in the first MCCH monitoring occasion of each MCCH repetition period.

We support single MCCH (in this release)

# 2	Discussion on MCCH/MTCH for broadcast

## 2.1	BWP and Interest Indication

In LTE MBMS solution one has single MCCH and this is logical as a UE camping on a carrier will receive wide carrier bandwidth. But in NR UE’s camping on a cell will only currently receive initial BWP where the UE can receive BCCH and PCCH. When considering providing MBMS service in NR it will be difficult or basically almost impossible to provide appropriate service level for UEs only receiving initial BWP due to capacity limitations.

Similarly when considering MCCH bandwidth one would have similar problem of scheduling MCCH only on CORESET#0 – Already now the capacity is issue on CORESET#0 and limiting the MCCH scheduling could be disastrous. But as this issue is more in RAN1 domain we would consider that decision on bandwidth (e.g. same CFR as MTCH) should be decided in RAN1.

If the MTCH could reside on BWP not containing initial BWP then we have bit of challenge how the UE will be able to perform regular idle activities e.g. paging, BCCH updates and emergency service reception as well as possible updates to broadcast services available (e.g. indicated on MCCH). Basically, we would have few options possible to ensure that these actions are possible while receiving MTCH:

- BWP where MTCH is sent fully contains initial BWP

- UE would be indicated updates on services required in IDLE while receiving MTCH on same BWP where MTCH is sent. The BWP may not overlap with initial BWP.

- We would allow UE to retune receiver to get initial BWP in order to perform required idle activities

Let’s discuss these options in more detail:

| BWP where MTCH is sent fully contains initial BWP | UE would be indicated updates on services required in IDLE while receiving MTCH on same BWP where MTCH is sent | We would allow UE to retune receiver to get initial BWP in order to perform required idle activities |  |
| --- | --- | --- | --- |
| Pros: - Simple - no RF retuning - no overhead Cons: - Require unnecessary wide BWP to be used to receive MTCH thus causing extra power consumption for the UE. - Also if initial BWP MCCH is used to indicates changes then UE would need to regularly check MCCH to check updates to broadcast services. - Still only limited capacity of a cell available to broadcast | Pros: - No need for RF retuning - BWP could be optimized for each service Cons: - Extra overhead to provide indication to UE | Pros: - No overhead Cons: - RF retuning required causing added latency to broadcast and possibly interruption in broadcast reception due to UE specific timing |

Once UE enters RRC_CONNECTED state UE will apply initial BWP until configured with wider BWP with RRCSetup and RRCResume

Observation 1: NW may change BWP of UE at RRCSetup/Resume

If NW does not have information about MBS interest prior reconfiguring BWP one could reconfigure BWP to such that MBS reception is not possible and causing service interrupt. One should avoid this kind of interrupt if possible:

Observation 2: If NW does not have information about MBS interest prior reconfiguring BWP there will be service interruption

Simple way would be not to reconfigure any UE BWP prior NW gets knowledge whether UE is interested in broadcast reception. UE which will not send interest indication would not indicate NW about its lack of interest for MBS services thus NW needs to have its own implementation for handling such UEs e.g. timer running in the NW after which it would definitely know whether UE is not interested in MBS service. This approach would impact both MBS supporting UEs not indicating interest and UEs not supporting MBS.

Another simple alternative is to never configure BWP to be not overlapping with CFR(s) used for MBS services. This would restrict possible BWP usefulness drastically as it would limit BWP configuration for UEs not interested in MBS services, also UEs not supporting MBS.

Observation 3: To avoid MBS service interruption NW cannot configure any UE BWP prior NW knows UE is not interested in MBS service(s)

From system performance point of view better approach would be to have network information about MBS interest prior possibility to reconfigure BWP.

SA3 is providing LS response in S3-213623 where they indicate privacy concern when  explicitly indicating service identities. They also indicate just having information regarding MBS frequency/priority does not have privacy concern.

In simplest form UE would indicate early whether it is receiving any MBS broadcast services. This way NW can avoid not being able to change BWP of non-MBS UEs. And NW would know that it should configure BWP in such a way that MBS reception can continue but this would have limitation that NW cannot optimize BWP to be specific for a service. If UE could provide more detailed information e.g. CFR of MBS service then NW could optimize configured BWP. Although this raises another concern on privacy that can a “attacker” map CFR to specific service. This would depend on deployments i.e. if each (or limited set) service have a dedicated CFR then it would be possible to map CFR to service.

Observation 4: Just providing frequency (e.g. CFR) information can cause privacy concerns as one could map CFR to service

But if this is no issue it would be beneficial to provide such a information to the NW together with priority.

Proposal 1: UE should provide interest to receive broadcast service(s) prior it is possible to reconfigure BWP for the UE. Whether CFR/priority is provided is subject to possible privacy concerns.

## 2.2	Reselection prioritization

## 2.2.1	Multicast cell specific prioritization

There is open issue:

- Editor’s note: FFS if there is a need to prioritize a frequency with multicast support for idle/inactive UEs that monitor multicast activation notification.

General principle in 3GPP has been that a UE should camp on the best (strongest) cell (even if non-MBS cell) in order to avoid massive interference to stronger cell when moving to connected state. We think this principle should be kept as interference will be a major issue. Additionally, even in non-MBS cell you can page the UE (for multicast session activation) and thus there is no issue regarding service continuity.

Observation 5: Generally UE is supposed to camp on strongest cell to avoid inteference

If for some reason NW still wants to UE to operate from non strongest cell when receiving multicast NW can always move UE to non-strongest cell if that is really desired by the NW. Thus we see no need to introduce cell specific prioritization for multicast services

Proposal 2: Do not introduce cell specific prioritization for MBS

## 2.2.2	Broadcast cell reselection prioritization

In the draft 38.304 we have various open issues although we do not think none of those are actually very critical:

- Editor’s note: FFS whether there is a seperate capability for MBS Broadcast and MBS Multicast or a single MBS capability.

We assume there can be UEs supporting either/or multicast and broadcast. Thus having separate capability seems fine but it is not clear what would be NW need for having information whether UE supports broadcast prioritization.

- Editor’s note: FFS whether UE needs to read the SIBx of the candidate cell before cell reselection. As an alternative, UE may determine whether the reselection candidate cell is broadcasting SIBx based on whether the scheduling info of SIBx is present in SIB1 of the reselection candidate cell or not.

In our view it would be sufficient to check whether SIBx is scheduled – This is in fact required behaviour if we have on demand SIBs. It would not make sense for UE to request transmission of SIBx and needing to wait it to be transmitted prior UE can perform reselection to the cell

Proposal 3: It is sufficient that SIBx is scheduled in the cell in order to perform MBS specific reselection behaviour

- Editor’s note: FFS whether UE should stop to prioritize the frequency if SIBx is not scheduled on the serving cell(i.e. reselected cell) anymore.

It seems to be a use case that network does not want more UEs to prioritize camping on MBS frequency but NW would still continue MBS service in the frequency. If now stopping SIBx transmission would cause all the UEs to reselect away from the frequency would make the MBS broadcast rather useless. Thus we propose

Proposal 4: Stopping SIBx transmission does not trigger UE to “deprioritize” MBS frequency for camping

- Editor’s note: FFS whether frequency in USD should also be checked when One or more IDs (e.g. SAI) of that frequency are indicated in SIBy of the serving cell.

  - SIBy is not broadcast in the serving cell and that frequency is included in the USD of this service.

- Editor’s note:	FFS whether the UE can prioritize the frequency indicated in USD when SIBy is broadcast but does not provide the mapping for the concerned service.

Regarding prioritization of frequencies from SIBy, we discussed that the UE shall prioritize the frequency indicated for the service/SAI from SIBy without checking if the candidate cell on that frequency broadcast SIBx.

Generally we think RAN information overrides USD information i.e. if RAN SIBy indicates mapping then UE can prioritize camping on the frequency. And on the other hand if USD indicates the service but it is not indicated by the RAN then it seems logical that RAN is broadcasting the service and prioritization should not occur but in this case it can be that UE is camping currently on non-MBS cell (e.g. release 16) which is not capable of broadcasting SIBx or SIBy. In this scenario it makes sense that UE could follow prioritization based on purely USD information but UE would need to check SIBx from tha candidate cell in order to prioritize camping on that one.

Proposal 5: UE should prioritize camping based on USD if the current (serving cell) does not provide SIBy after UE has checke that target cell provides SIBx

On the other hand if serving cell provides SIBy (and SIBx) one should be able to trust that RAN provides correct information and there is no need to check SIBx in this scenario from the target cell.

Proposal 6: If SIBy indicates interested service on a frequency then UE shall prioritize camping on that frequency without needing to check if target cell provides SIBx

# 3	Conclusion

In this paper we discussed IDLE/INACTIVE operation for multicast and broadcast services and came to following observations:

Observation 1: NW may change BWP of UE at RRCSetup/Resume

Observation 2: If NW does not have information about MBS interest prior reconfiguring BWP there will be service interruption

Observation 3: To avoid MBS service interruption NW cannot configure any UE BWP prior NW knows UE is not interested in MBS service(s)

Observation 4: Just providing frequency (e.g. CFR) information can cause privacy concerns as one could map CFR to service

Observation 5: Generally UE is supposed to camp on strongest cell to avoid inteference

and proposals:

Proposal 1: UE should provide interest to receive broadcast service(s) prior it is possible to reconfigure BWP for the UE. Whether CFR/priority is provided is subject to possible privacy concerns.

Proposal 2: Do not introduce cell specific prioritization for MBS

Proposal 3: It is sufficient that SIBx is scheduled in the cell in order to perform MBS specific reselection behaviour

Proposal 4: Stopping SIBx transmission does not trigger UE to “deprioritize” MBS frequency for camping

Proposal 5: UE should prioritize camping based on USD if the current (serving cell) does not provide SIBy after UE has checke that target cell provides SIBx

Proposal 6: If SIBy indicates interested service on a frequency then UE shall prioritize camping on that frequency without needing to check if target cell provides SIBx