3GPP TSG-RAN2 #116-e	R2-2110709

Elbonia, November 01 – 12, 2021

Agenda item:		8.12.2.1

Source:	LG Electronics

Title: 	Discussion on reduced capabilities

Document for:	Discussion and Decision

1.	Introduction

In this contribution, we discuss remaining FFS on definition of RedCap UE type and reduced capabilities.

2.	Discussion

Support of CHO related capabilities

In RAN2#115-e, RAN2 had the following conclusion regarding applicable capabilities for RedCap UEs.

- DAPS and CAPC related capabilities are not applicable for RedCap UE; [8/20] FFS on CHO. FFS on how to capture this in the specification;

We think CHO is applicable to RedCap UEs because CHO provides robustness and the functionality will be useful for some RedCap use cases such as Wearables. Considering some other use cases such as IWSN and video surveillance, robustness of mobility may not be so essential because the RedCap UEs may be deployed in a fixed location or have low mobility in a particular area. So, we think CHO can be optionally supported by RedCap UEs.

No specification update is needed because non-RedCap capabilities that are not applicable for RedCap UE is clarified in TS 38.306.

Proposal 1. CHO related capabilities are applicable for RedCap UEs as optional.

Number of DRB

- The number of DRBs supported by RedCap UEs is less than legacy value (which is 16). There will be a single mandatory value (FFS if 4 or 8). FFS if it will be possible to have an optional capability

- Maximum 8 DRBs is mandatory supported by RedCap UEs.

As RAN2 agreed that RedCap UEs support 8 DRB as mandatory, we think no additional number of DRBs needs to be specified. Although RedCap UEs are targeted for various range of use cases, the main category identified is mid-range use cases according to justification in WID [1]. Therefore, support of 8 DRBs would be enough for RedCap UEs.

Proposal 2. No additional optional capability to indicate the number of DRBs for RedCap is specified.

3.	Conclusions

In this contribution, we discuss a remaining FFS on definition of RedCap UE type and reduced capabilities, and we have following proposals:

Proposal 1. CHO related capabilities are applicable for RedCap UEs as optional.

Proposal 2. No additional optional capability to indicate the number of DRBs for RedCap is specified.

4.	Reference

# [1] RP-210918	Revised WID on support of reduced capability NR devices