3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-2110715

Online, 01 – 12 November 2021	revision of R2-2107508

Agenda item:	8.13.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Logged MDT and other enhancements

WID/SID:	NR_ENDC_SON_MDT_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we address remaining aspects of Logged MDT in context of avoidance of Signalling based MDT overriding and unbalanced UL/DL coverage.

In addition, this contribution discusses SNPN impacts on PLMN check for MDT reports that are worth considering for improving Logged MDT and RLF report retrieval in Rel-17.

# 2	Discussion

## 2.1	Signalling-based Logged MDT prioritization

RAN2#113bis-e made the following agreements:

Agreements:

3	Option 3 (R2-2104434) is used for logged MDT in EN-DC, i.e., do not introduce SN configuration for logged MDT (neither for camping nor for non-camping/ EMR specific frequencies).

4	UE provides assistance by which network can avoid overwriting of an MDT configuration.

5	Introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration.

RAN2#114-e further agreed:

Agreements:

1	In order to avoid overwriting of signalling-based logged MDT, UE-assisted and network-based solution, which relying on network implementation through UE providing assistance, is introduced.

RAN2#115-e further agreed:

Agreements:

Signaling based logged MDT override protection is applicable in the following scenarios:

1)	Signaling based Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

2)	Signaling based Logged MDT configuration is stopped (i.e. the expiry of T330), but UE still has un-retrieved results that would be discarded upon accepting a new configuration

4	Include an indicator to indicate the signaling based logged MDT configuration availability in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

FFS: Implicit (flag indicating T330 is running or not) vs explicit indication

The  introduction of information on logged MDT type into MDT configuration will provide the UE with a value to be repeated (under certain conditions) together indication of any logged data availability.   In case, the UE transitions to RRC_IDLE and reconnects to the network (including cells other than stored the UE context), the UE should inform it that there is one prioritized MDT session ongoing. However, since Management based MDT is more statistical and data loss from this kind of session is not that critical (compared to Signaling based MDT) it does not seem necessary to provide the Logged MDT type with value pertaining to Management based MDT.:

Observation 1: The UE assistance information on involvement in Management based MDT is not necessary.

Proposal 1: LoggedMeasurementsConfiguration is extended with “Logged MDT type” IE, which explicitly indicates involvement in Signaling based MDT only (no Management based MDT).

The agreed conditions on indicating MDT overriding protection distinguish three cases:

- Logged MDT is configured but there are no data available

- Logged MDT is configured, there are data available and T330 is running (MDT session is ongoing)

- Logged MDT is configured, there are data available and T330 expired (MDT session is finished but data unretrieved)

Observation 2: If the UE assistance information on involvement in Signaling based MDT is based on the Logged MDT type presence it covers all the three scenarios (no available data, T330 running but data unretrieved, T330 expired and data unretrieved).

To simplify conditions and UE procedures, we propose the following:

Proposal 2: No separate flag is introduced to indicate Signaling based MDT configuration during running T330.

Proposal 3: The UE includes availability bit on involvement in Signaling based MDT only upon checking “Logged MDT type” IE presence (provided in LoggedMeasurementsConfiguration).

## 2.2	Logged information for UL/DL coverage unbalanced

Based on  the email discussion in RAN2#113bis-e reported in R2-2104536, UL/DL coverage imbalance issue could not be concluded:

UL/DL coverage imbalance

Proposal 13A: FFS how to identify and solve the problem about UL/DL coverage imbalance.

As further we discussed in [Post115-e][896][SON/MDT] Logged MDT (Nokia), detection of UL coverage issues by the NG-RAN nodes was proposed to be achieved with three Options, out of which two got some support:

Option 1: To record DL signal state during an UL outage, RLF report is extended with “DL quality” information.

The field setting is assumed to be determined based on events around RLF (e.g. detecting a suitable cell, RRC Re-establishment attempt failure, losing cell signal).

Option 3: UL coverage can be estimated with the following conditions:

- Max UE power is higher than P_max or

- P_compensation in S-criteria is not equal to zero

For Option 3 we note it is claimed (R2-2108648) it is insufficient to estimate UL coverage with the current report. To identify UL/DL coverage imbalanced, it is proposed to estimate UL coverage based on the following conditions:

- Max UE power is higher than P_max or

- P_compensation in S-criteria is not equal to zero

However, the UL coverage as such is not in the focus here. The assumption is that RLF is caused due to UL outage detected by exceeding max number of RLC retransmissions. The addressed issue is to answer whether DL is affected, too, or it is an imbalance issue where only UL coverage is affected. The proposed elements are not suitable to provide the needed information.

The needed information could be, for instance, provided with a “Downlink Quality” field in the RLF report. The framework could be determined as follows:

-	If an RLF occurs with the RLF-Cause “T310 expiry” or “T312 expiry” detecting out-of-sync for the serving cell then the field is set to “downlink disappeared”. Of course, for this case the failure is obvious, and the new field is redundant. But field is relevant in case of UL coverage issue as shown in the following.

-	If an RLF occurs with the RLF-Cause “rlc-MaxNumRetx,” or “randomAccessProblem,”, then the UE sets the field to “downlink always available” by default indicating a pure UL coverage issue.

UE starts re-establishment process:

-	If UE detects a suitable cell for the re-establishment, the field remains set to “downlink always available”.

-	If the re-establishment fails, but UE still measures DL reference signals, irrespective whether from the RLF cell or new cell, and starts to new connection setup attempts, the field keeps the value “downlink always available”. If connection setup was successful, the field indicates a pure UL issue

-	If the re-establishment fails, and UE is also no longer able to measure DL reference signals, the UE changes the field to “downlink unclear”.

-	If UE starts another connection setup attempt after a certain time period indicates that DL signal was detected after short outage. Irrespective whether connection setup fails or is successful, the field remains set “downlink unclear”.

-	If however the UE is out-of-sync and not able to measure DL reference signals for a long period (and, for instance, already during re-establishment process), the value of the field is set to “downlink disappeared”.

Due to the new information the NG-RAN knows whether DL was also no longer measurable after the UL issue or available during the re-establishment and re-connection phases. For instance, if  there were 2 failed CE attempts, i.e. DL was at least partially received by the UE, but the UL was not working. If the out-of-sync periods (DL not measurable) lasts for long time, UE can finally set “downlink disappeared” in RLF, indicating that DL coverage needs to be corrected too.

For more precise identification of the DL coverage quality during the UL coverage outage we propose to extend the RLF report with “DL quality” information for better characterization of the DL signal during an UL outage:

Proposal 4: RLF report is extended with “DL quality” information for better characterization of the DL signal during an UL outage.

# 3	SNPN impact on PLMN check for MDT

## 3.1 PLMN check for MDT reports

When the UE is requested to report some stored information such as logged measurements or RLF report, it first checks if the current PLMN is the same that the one that requested the measurement.

The check concerns all the operations related to Logged MDT and RLF report, starting from logging initiation through storage of the data till the eventual reporting (association of the logged data with PLMN id, storing PLMN id, checking PLMN id before data retrieval, sending MDT data only to a RPLMN if it matches with a stored PLMN id).

Observation 3: UE checks the RPLMN when initiating sending the stored MDT data to avoid sending MDT data to wrong PLMN.

In particular, we note that RLF report is a mandatory feature in the UE.

Observation 4: UE mandatorily checks the RPLMN when initiating sending the stored RLF report data to avoid sending MDT data to wrong PLMN.

## 3.1 SNPN check for MDT reports

In Rel-16, the feature developed in WI “NG_RAN_PRN-Core”(RP-191563), allows the deployment of SNPN (Standalone Non Public Network). The selection of the network provider will be made using Network ID (NID). More precisely the combination of a PLMN ID + NID identifies an SNPN.

SNPN support was developed in NR, in parallel to the first release of NR MDT. When SNPN is used, the UE may not be connected to PLMN as such, but to another kind of service provider.

For MDT, by default, the UE should provide the stored information only to the relevant Network. Due to co-existence of SNPN and regular PLMN, the UE behaviour on data collection in case selected PLMN-Identity becomes NPN, remain ambiguous:

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityList or npn-IdentityInfoList fields included in SIB1.

After connecting to a cell and selecting PLMN id in case of NPN, the procedure leaves ambiguous interpretation on how the UE should proceed. The basic PLMN check for MDT reports does not support SNPN storage, thus actually it leads to a failure scenario in case of SNPN.

Observation 5: Lacking support of MDT PLMN check for SNPN may lead to failure scenario.

To avoid the failure scenarios for this mandatory feature, the Rel-17 UE should perform any selectedPLMN-Identity check before reporting RLF report. For this purpose, the UE needs to support storage and maintenance of SNPN through the same procedural steps, that apply to regular PLMN check in RLF reporting, i.e., the UE should check the Registered SNPN the same way it checks the RPLMN.

For RLF report this feature is more critical, but we believe the enhancement would also benefit Logged MDT, as it secures any logged report reporting to a right network. However, for the sake of timely completion of the WI, we propose the following:

Proposal 5: Logged MDT and RLF report supporting SNPN check is postponed to Rel-18.

# 4	SNPN impact to cell identification

## 4.1	SNPN presence in Cell Identity

Rel-16 NR SON and MDT reports use the IE CGI-Info-Logging-r16 in several places to identify the cell, where relevant SON or MDT event took place.

With the co-existence of SNPN, we believe it is important to consider the identification of a cell being used as reference information in logged measurements, RLF and CEF reports. If the cell, which becomes visible to the UE turns out to be SNPN cell, for consistent reporting the Cell Global Information used for MDT and SON reports,  will need to be enhanced to allow SNPN ID. Otherwise, we limit the implementation options to run SON and MDT features.

Proposal 6: The cell identification in MDT and SON messages should allow the use of SNPN.

# 5	Conclusion

In this contribution we have made the following proposals:

Proposal 1: LoggedMeasurementsConfiguration is extended with “Logged MDT type” IE, which explicitly indicates involvement in Signaling based MDT only (no Management based MDT).

Proposal 2: No separate flag is introduced to indicate Signaling based MDT configuration during running T330.

Proposal 3: The UE includes availability bit on involvement in Signaling based MDT only upon checking “Logged MDT type” IE presence (provided in LoggedMeasurementsConfiguration).

Proposal 4: RLF report is extended with “DL quality” information for better characterization of the DL signal during an UL outage.

Proposal 5: Logged MDT and RLF report supporting SNPN check is postponed to Rel-18.

Proposal 6: The cell identification in MDT and SON messages should allow the use of SNPN.