3GPP TSG-RAN WG2 Meeting #116-e															R2-2110826

Electronic Meeting, November 1 – 12, 2021

Agenda item:	8.11.7

Source: 	Qualcomm Incorporated

Title: 	Remaining issues for Positioning Reference Units

Document for:		Discussion and Decision

# 1.	Introduction

At RAN2#115-e the RAN1 LS on "Positioning Reference Units (PRUs) for enhancing positioning performance" [1] was discussed [2], and the following agreements were made [3]:

Agreements:

Proposal 1 (modified): For purposes of RAN2 discussion, the PRU functionality as described in the RAN1 LS can be considered as UE with known location (to some degree of accuracy) at least (16/17).

PRU modelled as a gNB can be discussed in RAN3 (no RAN2 action).

Agreement:

RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer.

In this contribution we discuss the impacts of PRUs on the positioning operations, the RAN2 specification impacts, and propose a LS to SA2.

# 2.	Positioning Reference Units (PRUs)

According to the RAN1 LS [1], a Positioning Reference Unit (PRU) at a known location can perform DL positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, etc.) and report these measurements to a location server. In addition, the PRU can transmit UL-PRS (SRS) to enable TRPs to measure and report UL positioning measurements from devices at known location (e.g., RTOA, UL-AoA, gNB Rx-Tx Time Difference, etc.). The PRU measurements can be compared with the "true" measurements expected at the known device position to determine correction terms for other nearby UEs. The DL- and/or UL location measurements for other UEs can then be corrected based on the previously determined correction terms. This principle is well known from e.g., Differential-GNSS operation.

The PRUs perform positioning measurements just like a normal UE, but at an a-priori known location. Therefore, the PRU-terminated positioning protocols could be the same protocols as used for normal UE positioning.

Observation 1:	The PRU-terminated positioning protocols (e.g., LPP, RRC, MAC) can be the same protocols as used for normal UE positioning.

# 3.	UE Positioning Operations

The general UE positioning operation is summarized in clause 5.2 of TS 38.305 [4] and clause 6.11 of TS 23.273 [5] and is shown in Figure 1 below. At Step 1, either some entity in the 5GC (e.g., a GMLC), or the UE, or the AMF itself (e.g., to locate the UE for an emergency call) requests some location services from an AMF. The AMF then transfers the location request to an LMF at Step 2 in Figure 1. The LMF may then invoke one or more LPP and/or NRPPa positioning procedures with the UE and/or NG-RAN at Step 3. Once the LMF has determined the UE location based on the measurement information received in Step 3, the LMF provides the location estimate to the AMF at Step 4, which then provides the UE location to the requesting client at Step 5.

A prerequisite for Step 3 is that a LCS Correlation identifier and the AMF identity has been passed to the LMF by the serving AMF, as described in TS 23.273 [5] (e.g., clause 6.11.1). The LCS Correlation ID and the AMF ID are provided by the AMF to the LMF in the Location Service Request at Step 2 in Figure 1. When the LMF then instigates Step 3 using a service operation towards the AMF with a LPP or NRPPa PDU included, the LMF also includes the LCS Correlation ID for this location session, together with the AMF ID which indicates the AMF instance serving the UE. The LMF uses the AMF instance to forward LPP/NRPPa messages to the UE/NG-RAN.

Figure 1: Location Service Support by NG-RAN (according to TS 38.305 [4]).

The PRU location measurements are used by an LMF to correct the measurements for target UEs. That is, the consumer of the PRU location information is an LMF, and therefore, Steps 1 and 2 in Figure 1 would not happen for PRUs.

This essentially means, the LMF becomes an "LCS Client" for PRUs and need to be enabled to instigate location sessions with a PRU in absence of Steps 1/2 in Figure 1.

Observation 2:	For PRUs, an AMF/LMF would not receive a location request from an LCS client. Instead, the location client for PRU measurements would be the LMF itself.

# 4.	PRU Positioning Operations

As a consequence of Observation 2, Steps 1/2 in Figure 1 would have to be replaced by a "PRU Registration" procedure to make an LMF aware of PRUs in a network such that the LMF is enabled to instigate a LPP or NRPPa session with the desired PRU or NG-RAN serving the PRU whenever reference measurements are needed by an LMF, as illustrated in Figure 2.

The "PRU Registration" procedure at Step 1 in Figure 2 should enable an LMF to instigate the positioning procedures at Step 3 in the same way as currently specified for target UEs.

Figure 2: Location Service Support for PRUs.

Once a PRU is registered/known to an LMF, the LPP and NRPPa procedures can be executed like for normal UEs.

Observation 3:	Some form of "PRU Registration" at an LMF is required to enable an LMF to instigate a location procedure with PRUs.

# 5.	RAN2 Specification Impacts

For the PRU registration at an LMF there could be multiple options as described in [6]. However, this is mostly outside RAN2's responsibility and should be discussed in SA2 first.

From RAN2/3 point of view, if a PRU is considered as a UE from LMF perspective, the existing LPP, NRPPa, RRC, MAC positioning procedures can be equally applicable to PRUs (Observation 1) and all defined LPP positioning methods can be supported by a PRU, dependent on PRU capabilities.

Observation 4:	Dependent on PRU capability, all LPP positioning methods can be supported by a PRU.

According to the RAN1 LS [1], a PRU need to provide its own location coordinates and antenna orientation information to an LMF (in addition to the normal UE positioning measurements). Dependent on how Step 1 in Figure 2 is realized, this can be achieved without any LPP, and other RAN2 protocol changes as also discussed in [6]. For example, all PRU specific information can be exchanged during PRU Registration which would avoid e.g., overloading LPP with functionality not required for normal UE positioning.

Observation 5:	Dependent on how a "PRU Registration" is realized by SA2, PRUs could be supported without RAN2 specification changes.

Therefore, before RAN2 adds potential PRU specific signalling to e.g., LPP the overall LCS framework for PRUs should be defined by SA2 first.

Observation 6:	Before determining any RAN2 specification impacts for PRUs, the LCS framework for PRUs need to be defined by SA2 first.

# 6.	Summary

In this contribution, we discussed some remaining issues for Positioning Reference Units (PRUs). The following Observations were made:

Observation 1:	The PRU-terminated positioning protocols (e.g., LPP, RRC, MAC) can be the same protocols as used for normal UE positioning.

Observation 2:	For PRUs, an AMF/LMF would not receive a location request from an LCS client. Instead, the location client for PRU measurements would be the LMF itself.

Observation 3:	Some form of "PRU Registration" at an LMF is required to enable an LMF to instigate a location procedure with PRUs.

Observation 4:	Dependent on PRU capability, all LPP positioning methods can be supported by a PRU.

Observation 5:	Dependent on how a "PRU Registration" is realized by SA2, PRUs could be supported without RAN2 specification changes.

Observation 6:	Before determining any RAN2 specification impacts for PRUs, the LCS framework for PRUs need to be defined by SA2 first.

Therefore, it is proposed:

Proposal 1:	Enable an LMF to act as an "LCS Client" for PRUs, i.e., enable an LMF to instigate location procedures (e.g., LPP, NRPPa procedures) with PRUs without location requests from an AMF.

Proposal 2: 	All LPP positioning methods can be supported by a PRU, dependent on PRU capabilities.

Proposal 3: 	Since the specification details for Proposal 1 are in the realm of SA2, agree on the LS to SA2 in R2-2110827 [7].

Proposal 4:	Before discussing specification changes for PRUs in RAN2, RAN2 should wait for the SA2 LCS Stage 2 support of PRUs.

# References

[1]		R2-2106920 (R1-2106326), "LS on Positioning Reference Units (PRUs) for enhancing positioning 					performance", RAN1.

[2]		R2-2108940, "[AT115-e][610][POS] PRUs (CATT)", CATT.

[3]		R2-2108835, "Report from session on positioning and sidelink relay", Session Chair (MediaTek).

[4]		3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[5]		3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[6]		R2-2108386, "Signalling and Procedures for supporting Positioning Reference Units", Qualcomm Incorporated.

[7]		R2-2110827, "[draft] Response LS on Positioning Reference Units (PRUs) for enhancing positioning 					performance", Qualcomm Incorporated.