3GPP TSG-RAN WG2 #116-e	Tdoc R2-2110886

Online, November 1th – 12th 2021

Agenda Item:	8.4.3

Source:	Ericsson

Title:	On Local Routing and Type 2/3 RLF Handling

Document for:	Discussion, Decision

# 1	Introduction

RAN2#114e made the following agreements for topology adaptation enhancements:

Assume that the IAB-donor will configure (alternative) egress links that can be used at local re-routing (at least with same destination, FFS same routing ID).

Local re-routing based on flow control feedback is allowed based on certain value of available buffer size. FFS further details. (Current hbh fc is for DL traffic).

The trigger to generate a type 2 RLF indication is at RLF detection. FFS whether for both: single and dual connection cases.

The trigger for type 3 RLF indication transmission is successful recovery after BH RLF. FFS whether for both: single and dual connection cases.

Type 2 and Type 3 BH RLF Indications are transmitted via BAP Control PDU.

Upon reception of the type-2 indication, the IAB node does not initiate RRC re-establishment.

If an IAB node with dual parents (via DC) receives type-2 BH RLF indication from one parent, IAB-node may trigger a local re-routing to the other parent. The detail of local re-routing and whether/how the action on type-2 indication is configurable is FFS.

This paper discuss the above issues, and presents our point of views as well as possible solutions.

# 2	Local Re-routing for Congestion Mitigation

IAB Rel-16 specs support local re-routing for RLF case using an alternative egress link in IAB node(s) routing tables (choose by the IAB-DU locally) for the BAP destination address carried the packet headers that are affected by RLF of their intended egress link. If this procedure is adopted for the re-routing due to local link congestion, then IAB-donor-CU does not need to configure specific alternative egress link. Rather the local IAB node can select a proper alternative/backup egress link among the links matched to the BAP destination address (in the routing table) carried in the packets that needed to be re-routed due to congestion.

If IAB Rel-16 mechanism for local re-routing (due to RLF) is adopted for link congestion scenario, then IAB-donor-CU does not need to configure specific alternative egress link to be used for local congestion mitigation.

Since local re-routing will be used to handle temporary congestion, RAN2 does not need to define a mechanism that requires additional specification work, i.e., use the existing re-routing mechanism (for RLF case) also for local link congestion case.

RAN2 agree to adopt the IAB Rel-16 re-routing mechanism for local link congestion case,  i.e. the alternative link is selected among the entries in the routing table matching the BAP destination in the BAP header.

When it comes to activating local re-routing, it was agreed in RAN2#115-e that the IAB node should be configured with a threshold on the available buffer size indicated in the DL HbH control feedback. If the available buffer size is smaller than such a threshold the IAB node may perform local routing.

From RAN2#115-e A configured threshold of available buffer size based on flow control feedback is used to determine the congestion, for the purpose of local re-routing.

Since the above configured threshold has to be provided by the CU to the IAB node-DU, RAN3 should take into account the above agreement in RAN3 specifications, i.e. TS 38.473

RAN2 to ask RAN3 to introduce a threshold on the available buffer size for the purpose of local re-routing, that may be provided by the CU to the IAB node DU.

The IAB node may enable local re-routing if the available buffer size is below the configured threshold.

In the unfortunate case in which all the dowlink links are congested, it is argued in [1] that the IAB node may suspend DL transmissions. How to deal with DL scheduling is a network task. The IAB node may decide to suspend DL transmissions, but this solution might not be always good. For example, the IAB node may enable AQM policies, or it may just suspend non-latency critical traffic. Additionally, RAN3 has also introduced a DL channel congestion notification from DU to CU, so the situation of all DL links being congested should be extremely rare.

How to deal with the case in which all links in the DL are congested is left to the IAB node DL scheduler implementation.

Since the DL flow control can be either per BH RLC channel ID, or per BAP routing ID, it seems reasonable that local re-routing can be enabled per BH RLC channel ID and BAP routing ID.

Local routing can imply re-routing of congested BH RLC channel ID(s) or of congested BAP routing IDs.

Related to the UL local routing based on local congestion, RAN2 has discussed it at length during the past meetings, but benefits are still not clear. In particular, we believe that if RAN2 wants to introduce UL local routing, there is no need to have the UL HbH flow control. The child can simply determine by its UL buffer status whether UL congestion is occurring. There is no need to receive an UL HbH flow control from the parent , because the child can already evaluate locally the UL congestion status. That is inherently different from the DL scenario, because in that case obviously the parent cannot know the DL congestion status at the child.

If RAN2 intention is to enable some sort of local routing at the child for the UL, that is certainly doable, but it is just enough to act on the buffer status at the child, e.g. the CU can configure rules on the UL buffer status at the child to enable local routing in the upstream.

It is not clear what is the benefit of the UL HbH flow control, given that the UL congestion status can already be evaluated locally by the child node.

RAN2 does not consider UL HbH flow control which was already downpriorized in RAN2#113e.

If RAN2 intention is to enable local routing in the upstream, it would be enough to introduce rules on the UL buffer status at the child, e.g. based on implementation. No need for the UL HbH flow control for the sake of UL local routing.

# 3	UL Local routing upon type 2/3 RLF Notification

For Rel-17 IAB WI, RAN2 has agreed to support the following RLF notification messages that an IAB node can send to its children nodes:

Type 2 - “Trying to recover”: Indication that BH link RLF is detected by the IAB-node, and the IAB-node is attempting to recover from it.

Type 3 - “BH link recovered”: Indication that the BH link successfully recovers from RLF.

Related to type-2 and -3 RLF the following agreements have been already reached by RAN2:

- From RAN2#113 RAN2 to support type-2/3 RLF indication (FFS specified behavior(s) TS impact, FFS details). Type-2 RLF indication may be used to trigger local rerouting Type-2 RLF indication may be used to trigger deactivation of IAB-supported in SIB Type-2 RLF indication may be used to trigger deactivation or reduction of SR and/or BSR transmissions From RAN2#114 The trigger to generate a type 2 RLF indication is at RLF detection. FFS whether for both: single and dual connection cases. The trigger for type 3 RLF indication transmission is successful recovery after BH RLF. FFS whether for both: single and dual connection cases. Type 2 and Type 3 BH RLF Indications are transmitted via BAP Control PDU. Upon reception of the type-2 indication, the IAB node does not initiate RRC re-establishment. If an IAB node with dual parents (via DC) receives type-2 BH RLF indication from one parent, IAB-node may trigger a local re-routing to the other parent. The detail of local re-routing and whether/how the action on type-2 indication is configurable is FFS.

One thing discussed in [1] is whether local routing triggered by the type-2 RLF should be configurable or not, either at the parent node or at the child. Since the type-2 RLF may trigger different action at the child node (see the agreements above from RAN2#113), whether local routing is possible or not should be configured, since that may trigger topology change:

Whether type-2 RLF can trigger local routing at the child node is configured by the CU to the child IAB node.

Another issue discussed in [1] is the granularity of the type-2 RLF indication. Some companies argue that the type-2 RLF should include the BAP Routing IDs of the traffic affected, whereas some others believe that the type-2 RLF should just be per BH link, as the legacy type-4 RLF indication.

Which granularity to choose strictly depends on the parent node behaviour when it is configured with dual connectivity. In our view, whenever there is a failure in any of the two links, the IAB node can perform local routing and route the traffic from the problematic link to the “still-available” link. It is not clear why the parent node which is dual connected should not perform local routing when there is a failure in one of the two links. In our view, this implies that the type-2 RLF should be transmitted by the parent node to the child node only when both links in the upstream are unavailable due to RLF.

When a dual-connected parent IAB node experiences an RLF in one of the two upstream links, it can perform local re-routing of the traffic from the problematic link to the other available link.

For a dual-connected parent IAB node, the type-2 RLF should be transmitted to the child IAB node only when both upstream links are unavailable due to BH RLF.

The above proposal greatly simplifies the type-2 RLF indication which does not need to indicate the BAP Routing IDs of the traffic affected. Rather, it can be similar as the legacy type-4 RLF indication, i.e. applicable by the child IAB node to the whole BH link.

The granularity of the type-2 RLF indication is per BH link, as the type-4 RLF.

Upon reception of such an indication, it will be up to child node implementation how to properly handle the affected traffic. As agreed in RAN2#113-e, the child IAB node may use the type-2 RLF reception to “trigger local routing” itself, to “trigger deactivation of IAB-supported in SIB”, to “trigger deactivation or reduction of SR and/or BSR transmissions”. In the email discussion [1], it was discussed suspending UL transmissions upon type-2 RLF reception, e.g. when there is no alternative link. Receiving type-2 RLF from both links does not necessarily imply that both links will be unable to recover. Suspending the transmissions blindly might just cause interruption, especially if the parents will be able to recover. Note also that it should be very unlikely that both parents get an RLF more or less at the same time, and both of them are unable to recover.

In RAN2#113-e, it was agreed that use may perform different actions upon reception of the type-2 BH RLF, e.g. trigger local routing, trigger deactivation of IAB-supported in SIB, trigger deactivation or reduction of SR and/or BSR transmissions, etc.

Upon reception of the type-2 RLF indication, it is up to the child IAB node implementation how to handle the affected traffic, also in case of no available alternative link.

For a situation where the children IAB nodes have dual parents (for instance, IAB8 in Figure 1), the reception of Type-2 from one of the parent nodes can cause a local re-routing of traffic impacted by RLF via the BH link with the other parent node. As proposed in Proposal 1 for the case of local congestion, also for the case of local routing due to BH RLF or type-2 RLF reception, there is no need for any specification work. That is because, Rel-16 already gives the possibility to select an alternative link.

If the IAB node performs local routing upon reception of type-2 RLF or BH RLF, Rel-16 re-routing principles are used,  i.e. the alternative link is selected among the entries in the routing table matching the BAP destination in the BAP header.

Figure 1. Example of IAB network with multiple paths for IAB node 3

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	If IAB Rel-16 mechanism for local re-routing (due to RLF) is adopted for link congestion scenario, then IAB-donor-CU does not need to configure specific alternative egress link to be used for local congestion mitigation.

Observation 2	It is not clear what is the benefit of the UL HbH flow control, given that the UL congestion status can already be evaluated locally by the child node.

Observation 3	When a dual-connected parent IAB node experiences an RLF in one of the two upstream links, it can perform local re-routing of the traffic from the problematic link to the other available link.

Observation 4	In RAN2#113-e, it was agreed that use may perform different actions upon reception of the type-2 BH RLF, e.g. trigger local routing, trigger deactivation of IAB-supported in SIB, trigger deactivation or reduction of SR and/or BSR transmissions, etc.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 agree to adopt the IAB Rel-16 re-routing mechanism for local link congestion case,  i.e. the alternative link is selected among the entries in the routing table matching the BAP destination in the BAP header.

Proposal 2	RAN2 to ask RAN3 to introduce a threshold on the available buffer size for the purpose of local re-routing, that may be provided by the CU to the IAB node DU.

Proposal 3	The IAB node may enable local re-routing if the available buffer size is below the configured threshold.

Proposal 4	How to deal with the case in which all links in the DL are congested is left to the IAB node DL scheduler implementation.

Proposal 5	Local routing can imply re-routing of congested BH RLC channel ID(s) or of congested BAP routing IDs.

Proposal 6	RAN2 does not consider UL HbH flow control which was already downpriorized in RAN2#113e.

Proposal 7	If RAN2 intention is to enable local routing in the upstream, it would be enough to introduce rules on the UL buffer status at the child, e.g. based on implementation. No need for the UL HbH flow control for the sake of UL local routing.

Proposal 8	Whether type-2 RLF can trigger local routing at the child node is configured by the CU to the child IAB node.

Proposal 9	For a dual-connected parent IAB node, the type-2 RLF should be transmitted to the child IAB node only when both upstream links are unavailable due to BH RLF.

Proposal 10	The granularity of the type-2 RLF indication is per BH link, as the type-4 RLF.

Proposal 11	Upon reception of the type-2 RLF indication, it is up to the child IAB node implementation how to handle the affected traffic, also in case of no available alternative link.

Proposal 12	If the IAB node performs local routing upon reception of type-2 RLF or BH RLF, Rel-16 re-routing principles are used,  i.e. the alternative link is selected among the entries in the routing table matching the BAP destination in the BAP header.

# 4	References

- R2-2107251, Summary of [Post114-e][075][eIAB] Open Issues on Re-routing, Huawei, HiSilicon