3GPP TSG RAN WG2 Meeting #116-e                                                        	R2-2110940

e-Meeting, November 1 – November 12, 2021

Agenda item:	8.15.3

Source:	Samsung

Title:	Resource pool configuration and selection of resource selection mechanism

Document for:	Discussion and Decision

# 1. Introduction

Resource allocation enhancement is part of Rel-17 NR SL enhancement WI scopes. In this contribution, we would like to raise some initial RAN2 discussion on the introduction of partial sensing based resource selection and random resource selection.

# 2. Discussion

In addition to random resource selection, RAN1 has introduced two types of partial sensing mechanisms. One is periodic based partial sensing and another is contiguous partial sensing. In our understanding, periodic based partial sensing is similar to LTE partial sensing although there would be small detailed differences. Contiguous partial sensing is a kind of sensing with the shorter sensing window duration compared to the sensing mechanism in Rel-16. Also note contiguous partial sensing can be performed once resource selection is triggered at n, i.e. post-sensing mechanism. Details are still under the discussion in RAN1. However, RAN2 can have some initial discussion on the resource pool configuration, resource pool selection, and selection of partial sensing mechanism.

In LTE V2X communication, a resource pool can be configured with the following options.

- A resource pool configuration allowing full sensing based resource selection

- A resource pool configuration allowing partial sensing based resource selection

- A resource pool configuration allowing random resource selection

- A resource pool configuration allowing partial sensing based resource selection or random resource selection.

In NR, we basically have four resource allocation (except the consideration of inter-UE coordination based resource selection), i.e. full sensing based resource selection, periodic partial sensing based resource selection, contiguous partial sensing based resource selection and random resource selection. Note RAN1 also allows combined partial sensing (periodic partial sensing AND contiguous partial sensing).  Then possible resource pool configurations would be as follow:

- A resource pool configuration allowing:

  - Full sensing {enabled}

  - Periodic partial sensing {enabled}

  - Contiguous partial sensing {enabled}

  - Periodic partial sensing AND contiguous partial sensing {enabled}

  - Random selection {enabled}

In LTE, selection of appropriate sensing mechanism is done in more semi-static manner (e.g. whether UE is configured for P2X transmission and UE capability) (see 36.331). For NR, it is not clear now how selection of appropriate sensing mechanism is done. Some can propose it is done based on dynamic conditions (e.g. traffic pattern, required QoS for each TB or remained battery status, etc.). To us, we do not think there would be sufficient time for the discussion before WI completion. Note this meeting is the last meeting for stage 2 discussion. Thus we would like to propose selection of appropriate sensing mechanism is done in more semi-static manner (like LTE). Note we can leave it to UE implementation for some cases (like LTE).

[Proposal 1]: RAN2 is asked to confirm a resource pool configuration allowing the following options.

  - Full sensing {enabled}

  - Periodic partial sensing {enabled}

  - Contiguous partial sensing {enabled}

  - Periodic partial sensing AND contiguous partial sensing {enabled}

  - Random selection {enabled}

[Proposal 2]: RAN2 is asked not to consider selection of a sensing mechanism based on dynamic conditions considering no time for discussion before WI completion.

# 3. Conclusion

In this contribution, we have discussed resource pool configuration and selection of a sensing mechanism aspects, and asked RAN2 to agree with the following proposals.

[Proposal 1]: RAN2 is asked to confirm a resource pool configuration allowing the following options.

  - Full sensing {enabled}

  - Periodic partial sensing {enabled}

  - Contiguous partial sensing {enabled}

  - Periodic partial sensing AND contiguous partial sensing {enabled}

  - Random selection {enabled}

[Proposal 2]: RAN2 is asked not to consider selection of a sensing mechanism based on dynamic conditions considering no time for discussion before WI completion.

# 4. Annex: latest RAN1 agreements

RAN1#105:

1) For the set of Preserve values in periodic-based partial sensing,

- If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList.

- Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList

  - This is per mode 2 Tx resource pool (pre-)configuration

  - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset

    - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx

      - FFS whether the monitoring can be mandatory

2) In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources.

- The processing time restriction includes Tproc,0SL  and Tproc,1SL.

- Aspects relating to sensing during SL DRX are to be discussed separately

Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately

- FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking

3) For the k value in periodic-based partial sensing for resource (re)selection,

- By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction.

- If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value

  - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included.

  - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions)

  - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring.

- FFS relationship between periodic-based partial sensing occasions and SL-DRX

- Note:

  - This is for the case when the resource (re)selection triggering slot n is expected by UE

4) For random resource selection,

- Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation.

- SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any)

  - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected.

FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions).

- Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

5) In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative

- TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc)

  - FFS details

- FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc).

RAN1#106:

1) In periodic-based partial sensing, UE monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection.

2) Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met:

- L1 is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool

  - FFS: When the trigger will be received by L1

- The resource pool is (pre-)configured to enable partial sensing

- Partial sensing is configured by higher layer in the UE

3) For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing,

- Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking

  - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc

- Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively

  - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable.

    - FFS: If additional enhancements are needed for enabling/disabling

- The triggering of re-evaluation and pre-emption checking is as in R16.

4) When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n,

- The resource selection window (RSW) is [n+T1, n+T2] where T2 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4

  - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- On the sensing window [n+TA, n+TB] for CPS,

  - Details of TA and TB values based on the agreements from previous RAN1 meetings

  - FFS whether and how to define a minimum CPS window size, including (pre-)configurability and the case when TB - TA is smaller than the minimum CPS window size

  - FFS whether and how to define a maximum value / upper bound for TB with respect at least to the minimum RSW size and the remaining PDB, including (pre-)configurability

- FFS how a set of candidate resource (SA) is initialized considering candidate single-slot resources, including

  - Whether and how to define a minimum size for the RSW (e.g., Rel-16 T2min), including (pre-)configurability

  - Whether the set SA is confined within a set of Y candidate slots within the RSW

- UE performs resource exclusion from the set SA based on at least all available sensing results and based on step 6) and 7) of Rel-16 TS 38.214 Sec. 8.1.4

- Note, re-evaluation and pre-emption checking in a resource pool with periodic reservation for another TB (sl-MultiReserveResource) disabled is considered separately.

- FFS: Details on T1

5) For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e

- Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed

  - Note, lower value means higher priority

  - FFS whether resource pool partitioning can be additionally applied

- Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI

  - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement,

  - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or

  - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement.

- Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking

- Option 12: No special consideration

6) When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n,

  - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4

    - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints

    - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure

    - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

7) When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n

  - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS

    - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA)

      - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings

- Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled

RAN1#106bis:

1) In the agreement from RAN1#105-e, the working assumption is confirmed and the FFS bullet (in RED) is closed without any agreement.

Agreement from RAN1#105-e: - For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE

2) When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings:

  - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots.

    - By default, M is 31 unless (pre-)configured with another value.

3) For the periodic sensing occasion(s) (PSO(s)) that a UE needs to additionally monitored in PBPS, it shall be (pre-)configured jointly for all Preserve values.

- The UE is not required to monitor PSOs earlier than n–T0 if the UE is triggered to do resource (re)selection in slot n, where T0 is (pre-)configured