3GPP TSG-RAN WG2 #116e	Tdoc R2-2110953

Electronic meeting, 2021-11-01 - 2021-11-12	Revision of R2-2108454

Agenda Item:	9.2.3

Source:	Ericsson

Title:	User plane aspects of NB-IoT and LTE-M in NTNs

Document for:	Discussion, Decision

# 1	Introduction

The Rel-17 SI on “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)” was documented in the TR 36.763 [1] and RAN#92-e decided the SI was completed and a new LTE WI on “NB-IoT/eMTC support for Non-Terrestrial Networks” was approved with a WID in [2].

In the justification the following is noted:

This Work Item intends to reuse the conclusions and recommendations of FS_LTE_NBIOT_eMTC_NTN study item, and the NR_NTN_solutions Work Item agreements and conclusions.

The WID list the assumptions:

Enhancements shall be specified as described hereafter with the following assumptions:

- Standalone deployment for NB-IoT / eMTC (i.e. operating in carrier(s) used only for NB-IoT NTN (resp. eMTC NTN)) for support in Rel-17 timeframe will be prioritized.

- GNSS capability in the UE is taken as a working assumption for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

- NB-IoT/eMTC design for terrestrial networks shall be reused as much as possible.

- Transparent payload

Further the WID has the following RAN2 objectives:

All cellular IoT features specified up to Rel-16 are supported for IoT NTN unless problems are found.

Specify the following enhancements re-using NR_NTN_Solutions WI agreements as a baseline, according to Section 8 in TR 36.763:

-	User Plane:

-	Enhancements to ra-ResponseWindowSize, mac-ContentionResolutionTimer, HARQ RTT timer, UL HARQ RTT timer, and sr-ProhibitTimer.

-	Enhancements to RLC t-Reordering timer.

-	Others:

-	Provisioning of ephemeris

Specify the following IoT NTN specific enhancements not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763:

-	Architecture:

-	Support for EPC

-	Mobility and Tracking Area:

-	Enhancements to tracking area management using the earth-fixed TA concept, considering both hard-switch and soft-switch options, where in the soft-switch option the network may broadcast more than one Tracking Area Code per PLMN.

-	Support of legacy (Rel-16) cell selection/reselection mechanisms without major enhancements. Minor adjustments to existing mobility mechanisms, such as a new parameter values, change to timing etc. can be considered to adapt functionality to NTN.

-	Support of legacy (Rel-16) Handover and RLF/reestablishment mechanisms without major enhancements. For eMTC, Rel-16 LTE CHO procedure can be considered without major enhancements. Minor adjustments to existing mobility mechanisms, such as a new parameter values, change to timing etc. can be considered to adapt functionality to NTN.

-	Others:

-	Support of discontinuous coverage without excessive UE power consumption and without excessive failures / recovery actions. Minor enhancements to the existing power saving mechanisms e.g. DRX, PSM, eDRX, relaxed monitoring, and (G)WUS can be considered, and if found needed, specified, to support discontinuous coverage;

Further RAN2#115 made the following user plane agreements:

Start of ra-ResponseWindow is delayed by an offset. Postpone discussion on the offset value until further agreements regarding RACH are made in RAN1.

If the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN.

Start of mac-ContentionResolutionTimer is delayed by an offset, (assumed equal to UE-eNB RTT). This can be revisited if RAN1 decides something that requires to change this.

If the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the mac-ContentionResolutionTimer for IoT NTN.

From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to eNB implementation to ensure sufficient time on UE side for the Msg3 transmission for IoT NTN.

RAN2 assumes that TA information (FFS what) reporting by the UE on network enabling will be needed in IoT NTN. Expect RAN1 need to progress on this, and can maybe reuse NR NTN progress. FFS in which message this is provided.

UE-eNB RTT is taken into account when calculating the (UL) HARQ RTT timer.

RAN2 assumes that sr-ProhibitTimer need to be extended. Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

From RAN2’s perspective, delayed start of pur-ResponseWindowTimer with UE-eNB RTT can be supported. This can be revised if RAN1 finds issues to support PUR that are not small.

pur-ResponseWindowSize is not extended for IoT NTN.

SPS is supported without modification for IoT NTN.

RAN2 confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN.

Do not extend the PDCP discardTimer for NB-IoT over NTN.

FFS whether to extend the PDCP discardTimer for eMTC over NTN.

Do not extend PDCP t-Reordering for IoT NTN.

In this contribution, we provide an overview of the necessary user plane changes when adapting NB-IoT and LTE-M to support NTNs.

# 2	Discussion

As noted in the TR and in the NR NTN WI, the most significant impact of NTN comes from the long round trip time. RAN1 have the objective to specify the time and frequency synch as well as timing relationships, see the WID.

Here we assume that RAN1 will specify LTE parameters Koffset and Kmac (or a similar mechanism to make scheduling possible) like the NR Koffset and Kmac, as well as specifying how the UEs can estimate the UE to eNB round trip time (UE-eNB RTT).

## 2.1 MAC issues

The LTE “HARQ RTT Timer” corresponds to the NR drx-HARQ-RTT-TimerDL which was extended by the UE-gNB RTT for HARQ processes with HARQ feedback.

The HARQ RTT Timer length is increased by the UE-eNB RTT.

The LTE “UL HARQ RTT Timer” corresponds to the NR drx-HARQ-RTT-TimerUL which details have not been concluded on but will most likely be extended by the UE-gNB RTT.

The UL HARQ RTT Timer length is increased by the UE-eNB RTT.

The LTE drx-RetransmissionTimer corresponds to the NR timer drx-RetransmissionTimerDL that was decided to not be extended in the NR NTN WI. The TR recommended to not extend the number of HARQ processes and to not have disabled HARQ feedback, thus there is no reason to change drx-RetransmissionTimer.

The drx-RetransmissionTimer is not changed.

The LTE drx-ULRetransmissionTimer corresponds to the NR timer drx-RetransmissionTimerUL that has not been agreed yet. The TR recommended to not extend the number of HARQ processes and not have disabled HARQ feedback, thus there is no reason to change drx-ULRetransmissionTimer.

The drx-ULRetransmissionTimer is not changed.

The other DRX timers have not been updated in the NR NTN WI, thus no updates shall be necessary for these in LTE.

## 2.2 RLC issues

For LTE-M and NB-IoT with two HARQ processes the following values are available for RLC t-Reordering:

T-Reordering ::=					ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, ms1600-v1310}

While a t-reordering of 1600 ms might be able to cover a lot of use cases, it is risky to have such a large gap between the largest value and the second largest value if for instance voice is to be supported for LTE-M NTN. The details of the corresponding parameter RLC t-Reassembly in NR NTN have not been concluded but it has been discussed to extend it up to 2200 ms. The timer value and the method to extend t-reassembly can be reused for IoT NTN.

The RLC timer t-Reordering is extended with higher values. IoT NTN can reuse the NR NTN solution.

## 2.3 PDCP issues

For LTE-M the current discardTimer has the following values:

discardTimer						ENUMERATED {

ms50, ms100, ms150, ms300, ms500,

ms750, ms1500, infinity

}															OPTIONAL,			-- Cond Setup

In NR NTN it was agreed that the timer can be extended to include the value 2000ms:

Agreements:

- ...

- Introduce a new discardTimerExt-r17 IE with a new value ms2000 and several spare bits for future extension.

- ...

We think that this could be reasonable for IoT NTN as well. We propose that similarly as in NR NTN that we extend the discardTimer with a value 2000 ms.

This could for instance be done in a simple manner by introducing discardTimer-r17 with a single value and then if the -r17 version is included the legacy discardTimer is ignored:

PDCP-Config information element

-- ASN1START

PDCP-Config ::=						SEQUENCE {

discardTimer						ENUMERATED {

ms50, ms100, ms150, ms300, ms500,

ms750, ms1500, infinity

}															OPTIONAL,			-- Cond Setup

...,

[[

discardTimer-r17					ENUMERATED {ms2000}		OPTIONAL, 		-- Need FFS

]]

}

-- ASN1STOP

PDCP-Config field descriptions

...

discardTimer Indicates the discard timer value specified in TS 36.323 [8]. Value in milliseconds. Value ms50 means 50 ms, ms100 means 100 ms and so on. If discardTimer-r17 is configured the discardTimer is ignored.

...

Extend the PDCP discardTimer with the single value 2000ms.

# 3	Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The HARQ RTT Timer length is increased by the UE-eNB RTT.

Proposal 2	The UL HARQ RTT Timer length is increased by the UE-eNB RTT.

Proposal 3	The drx-RetransmissionTimer is not changed.

Proposal 4	The drx-ULRetransmissionTimer is not changed.

Proposal 5	The RLC timer t-Reordering is extended with higher values. IoT NTN can reuse the NR NTN solution.

Proposal 6	Extend the PDCP discardTimer with the single value 2000ms.

# 4	References

TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”, RAN1/RAN2, v17.0.0, June 2021

RP-211573, “NB-IoT/eMTC support for Non-Terrestrial Networks”, MediaTek Inc., Eutelsat S.A., RAN#92-e, June 2021