3GPP TSG-RAN WG2 Meeting #116 electronic		R2-2110964

Online, 1st –  12th November, 2021

Title:	[DRAFT] LS Reply on beam information of PUCCH SCell in PUCCH SCell activation procedure

Response to:	R2-2109360 (R4-2115339)

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	Samsung [To be RAN2]

To:	RAN4

Cc:	RAN1

Contact Person:

Name:	Seungri Jin

E-mail Address:	seungri.jin@samsung.com

# 1. Overall Description

RAN2 would like to thank RAN4 for the LS on beam information of PUCCH SCell in PUCCH SCell activation procedure. RAN2 added the answers for the RAN4’s questions as below.

Q1: Whether UE can report CSI (e.g. L1-RSRP) of the target being-activated PUCCH SCell belonging to secondary PUCCH group by configuring CSI report setting (e.g. CSI-ReportConfig) on any active serving cells belonging to primary PUCCH group

A1: CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included. RAN2 signalling perspective, UE can report CSI (e.g. L1-RSRP) of the target being-activated PUCCH SCell onto the active serving cell belonging to primary PUCCH group which CSI-ReportConfig is included if the CSI-ReportConfig is configured on a certain active serving cell belonging to primary PUCCH group and refers the carrier as the target being-activated PUCCH SCell belonging to secondary PUCCH group which the CSI-ResourceConfig is included.

Q2: Whether the above observation is correct, i.e. the identified four cases are not supported by the current RAN1 and RAN2 specification

A2: Based on A1, since CSI report (e.g. L1-RSRP) can be reported within a serving cell belonging to primary PUCCH group, all 4 cases above can be supported. Even for the cases of unknown PUCCH SCell activation with an invalid TA for both FR1 and FR2, based on the CSI report as in A1, the UE can receive PDCCH order triggering CFRA for acquiring valid TA from UE side.

Q3: Whether the above identified cases can be supported by RAN1 and RAN2 spec updates within Rel-17 timeframe.

A3: For RAN2 perspective, the above identified cases are already supported in current specification. However, it is not clear the new capability to support this feature is required or not.

2. Actions:

To RAN4

ACTION: 	RAN2 respectfully asks RAN4 to take the above information into account in their further work

3. Date of Next Meetings:

TSG-RAN WG2 Meeting #116bis-e 	17 – 25, January, 2022			e-Meeting

TSG-RAN WG2 Meeting #117-e 	21 February – 03 March, 2022			e-Meeting