3GPP TSG-RAN WG2 Meeting #116 electronic                                            　    　　　　　　 R2- 2111150

Online, Nov, 01st - 12th 2021

Source: 	KDDI Corporation

Title:	System Information and supporting for RedCap UEs

Tdoc Type:	Discussion

Agenda Item:	8.12.2.2 Identification, access and camping restrictions

Document for:	Discussion and decision

Release:	Rel-17

Introduction

In [AT115-e][104][RedCap][1] email discussion, “Should system information provide information whether neighboring cells accept RedCap UE access” was discussed, without reaching a consensus. In this paper, we would like to share our views on the proposal 5 and discuss some details of which system information should be provided for redcap.

- Summary proposal 5: [To discuss, 2nd prio] [14/12] Discuss whether system information should provide information on which cells accept RedCap UE access, and if, what this information should include (e¸g. support, barring?) and in which form (e.g. NCell, allow-list, exclude-list)

Discussion

RedCap support status of neighbor cells

Regarding whether to indicate Redcap support status of neighbor cell, we understand some companies concern that the features of different base stations provided is not supported in current specification and some RAN3 discussions are needed if including them. However, in our view, without the indication, the UE has no acknowledge whether the cell can accommodate redcap UEs, before the UE read SIB1 of the cell. It results that the UE may measure the cell which cannot accommodate Redcap UEs and in that case the UE read SIB1 and recognize the cell cannot be a candidate cell for redcap UEs and mark it as a barred cell for some time, after that the UE reverts back to the original cell or other candidate cells.  If the network provide the indication, the UE can exclude the cell not supports redcap form the candidate cells in advance, before starting measurement. It makes the UE to avoid search cell not supporting Redcap, and contributes to battery saving.

Therefore, We share our views on proposal5 below.

Proposal1: RAN2 agree to indicate RedCap support status of neighbor cell

Cell reselection relevant parameters

Next discussion is whether redcap UE uses the same cell reselection parameters (q-RxLevMin and others) as normal UE or not. In our understanding in terms of the coverage, redcap UE usually has the same capability as normal UE, so no motivation to prepare another sets of cell reselection parameters dedicated for redcap UE.

Proposal2: RAN2 confirm that redcap UE uses the same cell reselection relevant parameters as normal UE

Cell reselection priority

Operators need to update their gNBs to have redcap capability. So, at the early stage of redcap commercialization, there are some deployment where there are not enough redcap gNBs. In that cases, operators may want to differ their redcap UE cell resection priority from the priority for normal UE. So, we propose to introduce another cell reselection priority information dedicated for redcap UE.

Proposal3: RAN2 agree to introduce another sets of priority information dedicated for redcap UE

Current SIB or new SIB

To indicate RedCap support status of neighbor cells and priority information dedicated for redcap UE, there are two solutions, using current SIB or introducing new SIB. In the case of using current SIB, it is difficult to separate reselection priority and other parameters between normal UE and RedCap UE. But, in the case of using new SIB, we can provide easily flexibility between the UE’s information. So, we propose the following

Proposal4: RAN2 agree to introduce new SIB to carry the information dedicated for redcap UE

Conclusion

In this conclusion, we make the following proposals.

Proposal1: RAN2 agree to indicate RedCap support status of neighbor cell

Proposal2: RAN2 confirm that redcap UE uses the same cell reselection relevant parameters as normal UE

Proposal3: RAN2 agree to introduce another sets of priority information dedicated for redcap UE

Proposal4: RAN2 agree to introduce new SIB to carry the information dedicated for redcap UE.

References

[1] R2-2108892 – [AT115-e][104][RedCap]-Final_summary.docx

[2] 3GPP TS 38.304