S3GPP TSG-RAN WG2 Meeting #116-e	R2-210xxxx

# Online, November 1-12, 2021

Agenda Item: 	5.4.3

Source: 			Huawei, HiSilicon

Title: 	Summary of [AT116-e][003][NR15] UE Capabilities I

Document for: 	Discussion and decision

- Introduction

This document summarizes the following offline discussion.

[AT116-e][003][NR15] UE Capabilities I (Huawei)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2109310, R2-2110969, R2-2110970, R2-2110971, R2-2110972,

Intended outcome: Report, agreed CRs if applicable

Deadline: Schedule 1

- Contact from companies

| Company | Email |
| --- | --- |
| Ericsson | lian.araujo@ericsson.com |
| Nokia | amaanat.ali@nokia.com |
| Apple | naveen.palle@apple.com |
| OPPO | qianxi.lu@oppo.com, duzhongda@oppo.com |
| vivo | xiao.xiao@vivo.com |
| Qualcomm Incorporated | mkitazoe@qti.qualcomm.com |
| CATT | zhangxiangdong@catt.cn |
| ZTE | li.wenting@zte.com.cn, liu.jing30@zte.com.cn |
| Samsung | Sb07.kim@samsung.com |
| Intel Corporation | seau.s.lim@intel.com |
| Sequans | omarco@sequans.com |

- Discussion

  - Part 1: Intended to determine agreeable parts

Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilties

R2-2109310	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilties (R1-2108378; contact: ZTE)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2	Cc:RAN4

Rapporteur understands this LS has already been taken into account in the last meeting in [AT115-e][017][NR15] UE Capabilties III (ZTE), so no further discussion is needed in this meeting.

Clarification on intraAndInterF-MeasAndReport capability

R2-2110969	Clarification on intraAndInterF-MeasAndReport capability	Huawei, HiSilicon	CR	Rel-15	38.306	15.15.0	0655	-	F	NR_newRAT-Core

R2-2110970	Clarification on intraAndInterF-MeasAndReport capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0656	-	A	NR_newRAT-Core

According to current TS 38.306, the capability of intraAndInterF-MeasAndReport is mandatory supported by UE for NR MCG. The network could see this feature as mandatory supported without checking with UE capabilities in NR SA, NR-DC and NE-DC. However, it is also described that this capability applies to NE-DC when configured, which should be interpreted as mandatory with capability signalling. Thus the requirement for intraAndInterF-MeasAndReport capability in NE-DC is not clear in the current 38.306, it should be clarified to avoid possible misunderstanding between the UE and the network.

Q1 Do companies agree with the intention of the CRs above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes (proponent) |  |
| Ericsson | Yes but | We should clarify also the NR-DC case, which does not seem clear at the moment either. Hence we propose the following: Indicates whether the UE supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. This field only applies to NE-DC and SN configured measurement when (NG)EN-DC is configured. For NR MCG, For NR SA, and MN and SN configured measurement when NR-DC or NE-DC is configured, this feature is mandatory supported. |
| Nokia | Yes | Agree but the formulation from Ericsson should be corrected as in NE-DC only the NR MCG part is relevant. |
| Apple | Yes |  |
| OPPO (Qianxi) | Yes | Same view as Nokia |
| vivo | Yes | Share Nokia’s view. So perhaps the original version of the change should be OK instead? |
| Qualcomm Incorporated | Yes, but | We actually have the same understanding as Ericsson. Currently there is no capability parameter for SCG of NR-DC, which to us indicate it is mandatory today. |
| CATT | Yes |  |
| ZTE (LiuJing) | Yes, but | For NE-DC, we agree this capability is not applicable. But for SN configured measurements in NR-DC, we think this capability is applicable. So we propose the following (based on Ericsson’s version): Indicates whether the UE supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. This field only applies to NE-DC and SN configured measurement when (NG)EN-DC or NR-DC is configured. For NR MCG, For NR SA, and MN configured measurement when NR-DC or NE-DC is configured, this feature is mandatory supported. Regarding QC’s comment, we think the field description already says: this field only applies to…”. For the last sentence, it should be interpreted as “this feature is always supported for NR SA and MN configured measurements regardless of the value of this field”. |
| Samsung | Yes | Since it is already mandatory for NR MCG, need not mention that "This field only applies to NE-DC". |
| Intel | Yes | NE-DC seems redundant here with NR MCG mentioned. |
| Sequans | Yes | Agree it is no applicable for NE-DC. |

12/12 companies agree with the intention of the CR, but 3 companies among them indicate the further clarification for NR-DC and NE-DC. It seems that it is the consensus that for NR SA, MN configured measurement in NR-DC and NE-DC, the feature is mandatory supported. The controversial part is for SN configured measurement in NR-DC, whether feature is also mandatory supported. This can be further discussed in phase 2. The moderator understands that all companies agree with the intention of the CR, so the CRs R2-2110969 and R2-2110970 can be pursued and the details will be discussed in phase 2.

Proposal 1: The CRs R2-2110969 and R2-2110970 are pursued.

Miscellaneous corrections for Rel-15 UE capabilities

R2-2110971	Miscellaneous corrections for Rel-15 UE capabilities	Huawei, HiSilicon	CR	Rel-15	38.306	15.15.0	0657	-	F	NR_newRAT-Core

R2-2110972	Miscellaneous corrections for Rel-15 UE capabilities	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0658	-	A	NR_newRAT-Core

1) pdsch-ProcessingType1-DifferentTB-PerSlot defines the UE supported number of unicast PDSCH TB(s) multiplexed in time domain within the same slot per CC per serving cell for processing time capability 1, in current TS38.306, the counted unicast PDSCH include the PDSCH scrambled with C-RNTI, TC-RNTI or CS-RNTI. However, MCS-C-RNTI, which is also used for unicast should be included as well. Otherwise, the PDSCH TB scrambled with MCS-C-RNTI (if supported by UE) scheduled in a slot may beyond UE capability.

2) timeDurationForQCL defines minimum number of OFDM symbols required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [12] clause 5.1.5. It is unclear about the starting and ending point for the above highlight time durations and how to count the time durations. For example, whether the last symbol or/and the first symbol are counted in the duration. Therefore, we propose to clarify the boundary of time duration.

beamReportTiming indicates the number of OFDM symbols between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report.

beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission. The number of OFDM symbols is measured from the last symbol containing the indication to the first symbol of CSI-RS.

Q2 Do companies agree with the intention of the CRs above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes (proponent) |  |
| Ericsson | Yes |  |
| Nokia | Yes | The changes are non-essential and are purely editorial. These are editorial changes so could be merged to the rapporteur’s CR? |
| Apple | Yes |  |
| OPPO (Zhongda) | Yes |  |
| vivo | Yes |  |
| Qualcomm Incorporated | No | The first proposal looks reasonable. The second proposal essentially reduces one symbol from the UE reporting delay. Say the last symbol of the indicating is ‘n’ and the first symbol of CSI-RS is ‘m’, the value of beamSwitchTiming should be m-n. But the proposal makes it m-n-1. |
| CATT | Yes |  |
| ZTE | FFS | The first proposal is OK to us, but the modification to the beamReportTiming/beamSwitchTiming, we need to confirm which understanding is right e.g. m-n or m-n-1 in Qualcomm’s example. We think m-n is more aligned with RAN1’s understanding. |
| Samsung | Yes |  |
| Intel | Yes | The first change looks fine. For the second change to the timeDurationForQCL, these seem to be aligned to the R1 feature list text as ‘Time duration is defined as counting from end of last symbol of PDCCH to beginning of the first symbol of PDSCH.’. Hence, we are fine with this addition. We are also fine to align beamReportTiming and beamSwitchTiming with this. This can be merged with the previous CR since these changes are not that essential. |

10/11 companies agree with the intention of the CR, 2 companies have different understanding on the change 2) the time for beamReportTiming/beamSwitchTiming, i.e. m-n or m-n-1. The moderator understands that the majority of companies agree with the intention of the CR, so the CRs R2-2110971 and R2-2110972 can be pursued, and the issue on value “m-n” or “m-n-1” for beamReportTiming/beamSwitchTiming needs to be discussed in phase 2.

Proposal 2: The CRs R2-2110971 and R2-2110972 are pursued, the issue on value “m-n” or “m-n-1” for beamReportTiming/beamSwitchTiming will be discussed in phase 2.

  - Part 2: Discussion on details and CRs

Clarification on intraAndInterF-MeasAndReport capability

Based on the phase 2 discussion, regarding capability intraAndInterF-MeasAndReport, companies have consensus on the following aspects:

- For NR SA, MN configured measurement in NR-DC and NE-DC, the feature is mandatory supported

- For SN configured measurement in (NG)EN-DC, the feature is applicable

The controversial part is:

- For SN configured measurement in NR-DC, whether the feature is applicable or mandatory supported?

Q3-1 For SN configured measurement in NR-DC, which option do companies support, capability intraAndInterF-MeasAndReport is applicable or mandatory supported?

| Company | Applicable or mandatory supported? | Comments |
| --- | --- | --- |

Q3-2a If “capability intraAndInterF-MeasAndReport is applicable” is supported in Q3-1, do companies agree with the following changes? Besides, do companies think additional capability signalling needs to be introduced for SN configured measurement in NR-DC?

intraAndInterF-MeasAndReport Indicates whether the UE supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. This field only applies to NE-DC and SN configured measurement when (NG)EN-DC is configured. For NR MCGNR SA, and MN configured measurement when NR-DC or NE-DC is configured, this feature is mandatory supported.

| Company | Yes or No for the change? | Comments (including additional capability signalling for SN configured measurement in NR-DC) |
| --- | --- | --- |

Q3-2b If “capability intraAndInterF-MeasAndReport is mandatory supported” is supported in Q3-1, do companies agree with the following changes?

intraAndInterF-MeasAndReport Indicates whether the UE supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. This field only applies to NE-DC and SN configured measurement when (NG)EN-DC is configured. For NR MCGNR SA, MN and SN configured measurement when NR-DC is configured, and MN configured measurement when NE-DC is configured, this feature is mandatory supported.

| Company | Yes or No? | Comments |
| --- | --- | --- |

Q3-3 If “capability intraAndInterF-MeasAndReport is mandatory supported” is supported in Q3-1, do companies agree that the similar changes should applied to capability eventA-MeasAndReport?

eventA-MeasAndReport Indicates whether the UE supports NR measurements and events A triggered reporting as specified in TS 38.331 [9]. This field only applies to SN configured measurement when (NG)EN-DC is configured. For NR MCGNR SA, MN and SN configured measurement when NR-DC is configured, and MN configured measurement when NE-DC is configured, this feature is mandatory supported.

| Company | Yes or No? | Comments |
| --- | --- | --- |

Minimum number of OFDM symbols

timeDurationForQCL

Based on the feedback in phase 1, it seems companies agree with the correction. Companies are invited to confirm it.

Q4-1 Do companies agree with the following changes for timeDurationForQCL, or prefer to send an LS to RAN1 is needed?

timeDurationForQCL Defines minimum number of OFDM symbols required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [12] clause 5.1.5. The number of OFDM symbols is measured from the end of the last symbol of the PDCCH reception to the start of the first symbol of the PDSCH reception. UE shall indicate one value of the minimum number of OFDM symbols per each subcarrier spacing of 60kHz and 120kHz.

| Company | Yes or No or LS? | Comments |
| --- | --- | --- |

beamReportTiming / beamSwitchTiming

Based on the feedback in phase 1, it seems companies have different views on the value “m-n” or “m-n-1”. The understanding for “m-n” or “m-n-1” are illustrated as below

Q4-2 Which option do companies support for beamReportTiming / beamSwitchTiming, value “m-n” or “m-n-1” or sending an LS to RAN1?

| Company | “m-n” or “m-n-1” or LS? | Comments |
| --- | --- | --- |

- Conclusions

- References

R2-2109310	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilties (R1-2108378; contact: ZTE)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2	Cc:RAN4

R2-2110969	Clarification on intraAndInterF-MeasAndReport capability	Huawei, HiSilicon	CR	Rel-15	38.306	15.15.0	0655	-	F	NR_newRAT-Core

R2-2110970	Clarification on intraAndInterF-MeasAndReport capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0656	-	A	NR_newRAT-Core

R2-2110971	Miscellaneous corrections for Rel-15 UE capabilities	Huawei, HiSilicon	CR	Rel-15	38.306	15.15.0	0657	-	F	NR_newRAT-Core

R2-2110972	Miscellaneous corrections for Rel-15 UE capabilities	Huawei, HiSilicon	CR	Rel-16	38.306	16.6.0	0658	-	A	NR_newRAT-Core