3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-21xxxxx

Online, November 1-12, 2021

Agenda item:	6.1.4.1.1

Source:	vivo

Title:	Draft-Summary of [AT116-e][010][NR16] Connection Control III (vivo)

Document for:	Discussion and Decision

1 Introduction

This document is a report on the following email discussion:

[AT116-e][010][NR16] Connection Control III (vivo)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2110523, R2-2110524, R2-2110525, R2-2110526, R2-2109346, R2-2110685, R2-2110686, R2-2111037, R2-2111200

Intended outcome: Report, Agreed CRs if applicable

Deadline: Schedule 1

The deadline Schedule 1 for this email discussion is copied from Chair notes:

- A first round with Deadline for comments Thursday W1 Nov 4 1200 UTC to settle scope what is agreeable etc

- A Final round with Final deadline Thursday W2 Nov 11 1200 UTC to settle details / agree CRs etc.

- Additional deadlines check points etc if needed are defined by the Rapporteur. In case some parts of an email discussion need more time, doesn’t converge, need on-line treatment etc Rapporteur please contact chair.

2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| vivo(rapporteur) | Xiaodong Yang | yangxiaodong5g@vivo.com |

3	Discussion

3.1 NR SA to ENDC Handover

This topic is from the following four contributions.

[1] R2-2110523	Discussion on the Timing Reference of PSCell SMTC Configuration	vivo	discussion	Rel-16	NR_newRAT-Core

[2] R2-2110524	Clarification on the Timing Reference of PSCell SMTC Configuration	vivo	CR	Rel-16	38.331	16.6.0	2836	-	F	NR_newRAT-Core

[3] R2-2110525	Define the UE capability for PSCell SMTC configuration	vivo	CR	Rel-16	38.306	16.6.0	0652	-	F	NR_newRAT-Core

[4] R2-2110526	Clarification on the Timing Reference of PSCell SMTC Configuration	vivo	CR	Rel-16	36.331	16.6.0	4735	-	F	NR_newRAT-Core

In [1],  the company identifies that it’s unclear whether the UE applies the PSCell SMTC configuration based on the timing reference of NR PCell or target EUTRA PCell for the case of NR SA to EN-DC HO with PSCell addition. According to the proposal in [1], there are basically the following options to address such an ambiguous UE behavior:

- Option 1: The target NR PSCell SMTC configuration is provided through MobilityFromNRCommand based on the timing reference of source NR PCell;

- Option 2: The target NR PSCell SMTC configuration is provided through targetRAT-MessageContainer based on the timing reference of target EUTRA PCell.

### Question 1: For the Timing Reference of PSCell SMTC Configuration in NR SA to ENDC Handover, which option(s) do you prefer?

- Option 1: based on the timing reference of source NR PCell;

- Option 2: based on the timing reference of target EUTRA PCell;

- Both Option 1 and Option 2;

- Other Option(s), please specify.

| Company | Option(s) | Comments |
| --- | --- | --- |

### Question 2-1: If Option 1 is preferred to Question 1, do companies agree with the intention of 38.331 CR in [2] i.e., clarify that the target NR PSCell SMTC configuration through MobilityFromNRCommand is based on the timing reference of source NR PCell?

| Company | Agree or not? | Comments |
| --- | --- | --- |

### Question 2-2: If Option 1 is preferred to Question 1, do companies agree with the intention of 38.306 CR in [3] i.e., define a new UE capability for PSCell SMTC configuration?

| Company | Agree or not? | Comments |
| --- | --- | --- |

### Question 3: If option 2 is preferred to Question 1, do companies agree with the intention of 36.331 CR in [4] i.e., clarify that the target NR PSCell SMTC configuration through targetRAT-MessageContainer is based on the timing reference of target EUTRA PCell?

| Company | Agree or not? | Comments |
| --- | --- | --- |

3.2 UP security policy update

In the incoming LS from RAN3 [5], the following action is required:

To RAN1 : 	RAN3 kindly requests RAN2 to provide feedback whether the enabling/disabling of ciphering or integrity protection of one or multiple DRBs can be achieved by intra-cell handover within one RRC reconfiguration message.

There are several contributions related to this issue as follows.

[6] R2-2110685	Discussion on UP security policy updated by intra-cell handover	Ericsson	discussion	Rel-16	TEI16

[7] R2-2110686	[Draft] Reply LS on UP security policy updated by intra-cell handover	Ericsson	LS out	Rel-16	TEI16	To:RAN3	Cc:SA3

[8] R2-2111037	Discussion on Ls on UP security update	vivo	discussion	Rel-16	TEI16

[9] R2-2111200	Discussion on UP security policy updated by intra-cell handover	China Telecommunications	discussion

Referring to contributions above,  all companies express similar views, i.e., enabling/disabling of ciphering or integrity protection of one or multiple DRBs can be achieved by intra-cell handover within one RRC reconfiguration message. The rapporteur shares the same view with these companies,  according to current TS 38.331, clause 5.3.1.2, integrity protection and ciphering for all DRBs related to the same PDU session are always activated together. A DRB can be reconfigured using RRCReconfiguration message according to the TS 38.331 clause 5.3.5.3.

In order to form a common view to reply to RAN3’s question, companies are encouraged to answer the following question.

### Question 4: Do companies agree that enabling/disabling of ciphering or integrity protection of one or multiple DRBs can be achieved by intra-cell handover within one RRC reconfiguration message？

| Company | Agree or not | Comments |
| --- | --- | --- |

4 Conclusion

TBD.

5 Reference

[1] R2-2110523	Discussion on the Timing Reference of PSCell SMTC Configuration	vivo	discussion	Rel-16	NR_newRAT-Core

[2] R2-2110524	Clarification on the Timing Reference of PSCell SMTC Configuration	vivo	CR	Rel-16	38.331	16.6.0	2836	-	F	NR_newRAT-Core

[3] R2-2110525	Define the UE capability for PSCell SMTC configuration	vivo	CR	Rel-16	38.306	16.6.0	0652	-	F	NR_newRAT-Core

[4] R2-2110526	Clarification on the Timing Reference of PSCell SMTC Configuration	vivo	CR	Rel-16	36.331	16.6.0	4735	-	F	NR_newRAT-Core

[5] R2-2109346	LS on UP security policy updated by intra-cell handover (R3-214464; contact: China Telecom)	RAN3	LS in	Rel-16	TEI16	To:RAN2	Cc:SA3

[6] R2-2110685	Discussion on UP security policy updated by intra-cell handover	Ericsson	discussion	Rel-16	TEI16

[7] R2-2110686	[Draft] Reply LS on UP security policy updated by intra-cell handover	Ericsson	LS out	Rel-16	TEI16	To:RAN3	Cc:SA3

[8] R2-2111037	Discussion on Ls on UP security update	vivo	discussion	Rel-16	TEI16

[9] R2-2111200	Discussion on UP security policy updated by intra-cell handover	China Telecommunications	discussion