3GPP TSG-RAN WG2 Meeting #116 electronic		R2-2111525

Online, 1 - 12 November 2021, 2021

Title:	LS on UE Power Saving

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN2

To:	SA2, CT1, RAN3, RAN1

Cc:

Contact Person:

Name:	Yanhua Li

E-mail Address:	liyanhua1@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 discussed UE paging subgrouping as part of the Rel-17 work on UE power saving enhancement (see RP-200938). Two subgrouping approaches have been agreed, namely CN-assigned subgrouping and UEID-based subgrouping. The following provides the detailed agreements below for the 2 approaches:

Assume that one subgroup indication refer to either CN assigned subgroups or UE-ID based subgroup (no overlapping)

Both UE ID based and CN based subgrouping can be supported simultaneously in a cell, it is allowed to just support one of them.

The total number of CN-assigned subgroups that is used is not fixed can be configured up to 8 (e.g. OAM). No specification impact.

RAN introduces a new parameter Nsg-UEID to indicate its support for CN assigned subgrouping and/or UE-ID based subgrouping. FFS for the signalling details.

RAN does not support any type of subgrouping if its configuration for subgrouping is either absent or nullified (e.g. subgroupsNumPerPO is either absent or set to zero). FFS for the signalling details.

RAN2 also discussed the UE capability, and agreed to the following:

To be added.

RAN2 also discussed the UE PEI assistance information, and agreed to the following:

To be added based on [035, CMCC].

RAN2 also discussed whether to restrict PEI to the last used cell, and agreed to the following:

To be added based on [046, Ericsson].

2. Actions:

To SA2, and CT1:

RAN2 respectfully asks SA2, and CT1 to take the above information into account for their future work, and provide further information on the following issue once concluded:

- Add if there is any.

To RAN3:

RAN2 respectfully asks RAN3 to take the above information into account for their future work, and provide further information on the following issue once concluded:

- Signalling between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE.

To RAN1:

RAN2 respectfully asks RAN1 to take the above information into account for their future work, and provide further information on the following issues once concluded:

- List of the parameters for PEI subgrouping configuration.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #116bis   17 January – 25 January, 2022	online meeting

TSG RAN WG2 Meeting #117        21 February – 3 March, 2022	             online meeting