3GPP TSG-RAN WG2 #116-e	R2-21xxxxx

Electronic, 1st – 12th Nov, 2021

Agenda Item:	5.4.1

Source:	R2 Chair (MediaTek inc)

Title:	[AT116-e][049][TEI17] TEI17 NR proposals (Chairman)

Document for:	Discussion, Decision

Introduction

This document is to kick off the following email discussion:

[AT116-e][049][TEI17] TEI17 NR proposals (Chairman)

Scope: Collect comments on selected NR TEI17 proposals
Intended outcome: Report
Deadline: Tuesday W2

The intention with this offline discussion is to collect comments to identify proposals that could be agreeable.

Chair on TEI proposals

A TEI item shall have a limited scope, it should be possible to complete the work in 1 quarter (given sufficient attention and focus). The work should be limited to one WG (small exceptions are allowed).

TEI proposals are usually judged differently according to novelty - in a range, e.g.

- Corrections not implemented in a previous release, small proposals that should obviously/reasonably have been implemented in a previous WI but was missed for some reason.

- Well known earlier WI proposals with some support but were not done e.g. due to lack of time. Small features that were implemented in earlier system.

- New items, giving better performance, or enabling a new use case etc.

Corrections or almost corrections are typically judged similarly to corrections, e.g. the motivation for the full story is assumed pre-known. Discussions can be quite simple, straightforward opinions on impact vs gain and the bar for acceptance is usually medium (higher or somewhat higher than for pure corrections).

New features most often require a more comprehensive analysis and understanding, sometimes similar to judging new WI proposals at Plenary. Understanding justifications vs impact/possibility to deploy etc is important. Operator input is sometimes helpful to verify validity of justifications. The bar for acceptance is usually quite high.

Other aspects are usually considered, e.g. proposals that has recently been rejected would be considered again if the situation has changed somehow, but not otherwise. Proposals that were rejected for an ongoing WI should generally not be considered for TEI.

As usual and always, for all kinds of proposals, technical sanity check is fundamental. Does the proposal work? Is it feasible? Does the proposal address the intended issue / intended case.

Please consider these aspects when you provide comments in this discussion so there can be a balanced result.

Opinions and Comments

Please provide opinions. It is appreciated that you give a concise motivation. You can refer to other company’s motivation if your’s is the same. You can also ask questions, and make comments that you think may impact the perception of the proposal.

Opinions will be interpreted as follows:

Support = Support the proposal, think it is useful

Not Support = Don’t support the proposal, not useful etc. Could be acceptable.

Not Acceptable = This is objected to.

Unclear = Don’t know yet, asking some questions, may decide later if there are replies.

Updating this document

This is a big document so collision updates may happen. When naming your file update, please:

1) Increase the revision one step compared to your baseline version.

2) Keep the previous editor company name and add your company name last (i.e. two company names)

E.g. CATT revision based on Nokias: 
[AT116-e][049][TEI17] TEI17 NR proposals_v12_Nokia_CATT.docx

Contact Information

| Company | Email address |
| --- | --- |
| R2 Chair | Johan.johansson@mediatek.com |
| LG | ssunyoung.lee@lge.com |
| Ericsson | Ritesh.shreevastav@ericsson.com, antonino.orsino@ericsson.com |
| ESA | florin-catalin.grec@esa.int |
| Nokia | benoist.sebire@nokia.com |
| CATT | pierrebertrand@catt.cn |
| CMCC | chenningyu@chinamobile.com |
| Huawei, HiSilicon | zhaoyang@huawei.com |
| Apple | yuqin_chen@apple.com |
| OPPO | Shicong@oppo.com |
| Vodafone | chris.pudney@vodafone.com |
| Lenovo, Motorola Mobility | hchoi5@lenovo.com |
| MediaTek | chunfan.tsai@mediatek.com |
| Xiaomi | Yangxing1@xiaomi.com |
| ZTE | Dong.fei@zte.com.cn |
| Futurewei | Hao.bi@futurewei.com |
| Vivo | Yangxiaodong5g@vivo.com |
| China Telecom | liujiaxiang6@chinatelecom.cn, linp@chinatelecom.cn |
| NEC | hisashi.futaki @nec.com |
| Samsung | bh14.jung@samsung.com |
| Google | frankwu@google.com |

Discussion

Undecided proposal (has been treated no decision)

CGI Report extension

CGI Report Extension Proposal

R2-2110981	On the support of NG-based handover using CGI report	Huawei, HiSilicon, CMCC, China Telecom, China Unicom	discussion	Rel-17	TEI17

R2-2109716	CR to 38.331 on support of NG-based (i.e. via CN) handover based using CGI report	China Telecom, Huawei, HiSilicon	CR	Rel-17	38.331	16.6.0	2816	-	F	TEI17

Some Comments has already been provided in the following tdoc

R2-2110856	On using RAN3 based solution for unsupported SCS+BW of neighbor cell	Ericsson	discussion

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Support(Proponent) | Issue: unmatched capability of SCS/BW between UE and target cell leads to HO failure Two options on the table: a) RAN2 solution: add SCS and BW into CGI report; b) RAN3 solution (proposed by Ericsson): enhance the ’cause’ values in the HANDOVER FAILURE message to reflect unsupported SCS and/or BW We think RAN2 solution is better. For UEs configured to report CGI, SCS/BW info of target cell is already obtained by the UE and the UE will send the CGI report anyway. No big effort to include extra known fields into the CGI report. Besides, the HO failure can be prevented. Drawback of RAN3 solution: 1) The HO has to be failed first to know the SCS/BW is(/are) unsupported; 2) Even though the source node know the reason for failure, it is not aware about the exact the SCS/BW of target node, thus future failures can happen again. |
| Nokia, Nokia Shanghai Bell | Not accept | As proposed, this seems to require source to do the same checks as target already does. - When source sends HO request to target, the target cell will check the UE capabilities and current RRC configuration. If they do not match the cell, target will reject the HO request. - If there are multiple HO failures to a cell, that would likely be recognized as a bad HO candidate and blocked. In any case, this is really a RAN3-only issue and should be solved there. Huawei Response: we have explained the problem if it is left to RAN3. It seems that there is same understanding here that this would lead to potential handover failures, thus we think it is worth discussing which solution is better in RAN2. |
| CMCC | Support | As one of the proponent companies, we are supportive to enhance CGI reporting to facilitate NG-based handover. Source gNB still has no idea of the SCS/BW of the target gNB. So the failure may happen again. |
| Apple | Not support | Our understanding is the channel bandwidth, and SCS of one frequency should be rather stable configurations. It is not likely that for one spectrum, different cells are deployed with different channel bandwidth. With this logic, isn’t it better to solve the problem by OAM? Relying on UE reporting seems an overkill to us. |
| OPPO | NSupport | If network based solution can solve the issue as Ericsson‘ paper said, we would like not to impact UE. |
| vivo | Support | From our perspective, a proactive approach is preferred as it is beneficial in avoiding the potential failure events. The UE reporting SCS+BW information is straight-forward in this sense. The CR looks good to us, but some minor editorial issues should be fixed, e.g., „5> include the scs-SpecificCarrier-listUL and set it as scs-SpecificCarrierList obtained from uplinkConfigCommon of the concerned cell” “ [[ reportSCSAndBandwidth-r17 ENUMERATED {true} OPTIONAL, -- Need N ]]“ |
| China Telecom | Support | RAN3 based solution needs to reject HO request first which could be avoided by adding SCS and BW into CGI report. Thus, we think RAN2 based solution can solve this issue directly. |
| NEC | NSupport | First, we would like to know how much „such cell‘ is important for handver. If the cell is important, then it is to be known by the network (source gNB). Second, it could be assumed that SCS would be common within a frequency, i.e. not per cell basically. This means the OAM approach (well-known in RAN3) will be feasible. |
| Samsung | Support | We support the proposals and agree not to introduce additional information on the CGI report. |
| Google | NSupport | We think this can be solved by the network based solution without any impact to the UE. |
| Docomo | Support | We support the motivation of this proposal. Deployment and migration of the spectrum sometimes takes multiple steps and each step could be several years, leading to different BWs for the same band. As for a solution, although the target gNB could validate the UE caps and refuse the handover, preventative approach would be better. |

Location Privacy in RRC

Location Privacy in RRC

R2-2110047	User preferences to control location information sharing	Apple, Samsung, Google, Xiaomi, Vivo, BT Plc, Rakuten Mobile, MediaTek Inc	discussion	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Nokia, Nokia Shanghai Bell | NSupport | This is outside 3GPP: The intent of "available" for location information was that UE indicates the information if it UE has it at the time of reporting, but not that UE has to do location update to obtain the information. This seems like a UE implementation matter and not something RAN2 should discuss. User consent is already taken into account for MDT (once user consent is provided it ensures privacy and legal obligations are fulfilled), so it's unclear what the problem is. The proposed solution (by statement in RRC specification) does not actually help either, as this would be still implementation issue on how the RRC layer gets this “User Preference”. This is some application layer-level information and actual perception of the user preference information into RRC still remains implementation specific. Furthermore, we believe the intention does not differ from what is already captured in the current RRC: “The UE may not succeed e.g. because the user manually disabled the GPS hardware, or due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.” |
| CATT | NSupport | We can understanding the requirement and intention to introduce User Preferences, but this may be out of RAN2 scope, this issue coudl be left to other groups to confirm or discuss, not RAN2. |
| CMCC | NAccept | We have shared our opinion in last meeting. This is unacceptable for us. The reasons are as follows: - Firstly, anything related with security or privacy should be discussed in SA3 or SA5, not in RAN2. RAN2 is not responsible to discuss on privacy unless any issue is identified by SA3. - The current user consent mechanism is sufficient. If the UE sign the user consent, it means the UE is willing to share MDT reporting containing location info. It looks strange if UE still don’t prefer to report location. If UE don’t want to report location or not trust operator, the UE can simply not sign the user consent. - For SON reporting, e.g., RLF-reporting, CEF-reporting, they are one shoot reporting. With UE ID anonymization, the UE cannot be tracked. - UE ID is anonymized by the gNB. For SON/MDT related reporting, the UE ID is removed by the gNB from the record before send it to OAM. So there won’t be any privacy issue. |
| Huawei, HiSilicon | Not a RAN2-led TEI | If the proposal is referring to the user consent, we understand there is already discussion in other groups. RAN2 is not the leading group to discuss this. |
| ZTE | NSupport | According to current specs UE can decide its preference on location report by turn on/off GPS module, and by user consent. And operator would offer special discount to UE who have signed the user consent, so if UE is allowed to refuse to report the location info even when it has already signed the user consent, the efforts from operators are wasted. Anyhow, we don’t see a need to discuss this in RAN2, if there are concerns then SA3 will initiate the discussion. |
| OPPO | Support |  |
| Lenovo, Motorola Mobility | unclear | We agree with the intention but think a note is sufficient. Adding the phrase “allowed by the user” in the normative text looks strange in AS specs. |
| MediaTek | Support | At least, RAN2 should confirm that location information may not be provided to newtork due to user preference. CR content could be further discussed. |
| Apple | Support | We would like to address the points raised above, specifically: - Some companies mention user consent; however, TS 32.422 is very clear – user consent is not defined for SON (only for MDT). - Some companies mention this is not in 3GPP/RAN2 scope; however, the notion of user preferences is not new to RAN2, see for example clause 5.5.3.1 and clause 5.6.15.4 in TS 36.331. - Some companies mention this should be discussed in SA3; however, SA3 have already discussed it and acknowledged the issue (see S3-211338). - Some companies claim UE ID anonymization solves the issue; however, with proliferation of machine learning re-identification techniques this is no longer the case. - Some companies believe the user can turn GNSS off to preserve privacy. This is rather radical, don’t you think? Regarding how to capture this in the specs – we are open to discuss the actual CR text. |

| Futurewei | NSupport | This should be discussed in other WGs first, and RAN2 can follow up upon their progress, if needed. It looks like more straightforward to clarify the provacy requirement of SON in TS 32.422. |
| --- | --- | --- |
| NTTDOCOMO | See comments | There is no quesiton user consent should be taken into account before network acquire UE location info. We agree observation4 in the paper that current user consent in 3gpp spec applies to MDT only, not to SON. So we prefer to clarify that gNB should confirm user consent through core network signaling like MDT before configuring UE to acquire location info in SON report (i.e. RLF report, SCGFailureInfomation) |
| vivo | Support | Currently user consent defined in 3GPP specifications applies to MDT only, not to SON. In some cases UE may be required to report user location when GNSS is otuned n, but user may not be willing to share his location info with the network. In such case user should be allowed to able to share his location based on his prefence or not. |
| NEC | NAccept | This had been discussed long time ago (Rel-10) but not agreed. The reason was that 3GPP (at least RAN2) does not specify something related to or involving „user“ of the UE. |
| Samsung | Support | We support the proposals to introduce user preference concept for SON, which is similar to user consent defined in MDT. |
| Google | Support | A user always decided whether to report the location information and the current text should be clarified. |

System Information Scheduling

System Information Scheduling Proposal

R2-2111248	On the need of providing explicit SI start position for SI Scheduling	Ericsson, Verizon, Deutsche Telekom, Softbank, Swift Navigation, ESA, T-Mobile USA	discussion	Rel-17

Some comments has already been provided in the following tdoc

R2-2110799	SIB and posSIB scheduling constraints	MediaTek Inc.	discussion	Rel-17	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Ericsson | (Proponent for R2-2111248) | Some comments on R2-2110799. It is difficult to solve (avoid collision and have more rooms for SI) just by means of changing parameters in deployment. Reducing SI window length implies: - Reducing coverage - decreased Transport Block Size; which may increase latency for PWS SI which then would have to be provided in very small segments - Not possible to have large number of beam sweeps. Each beam needs to have the SI information and if the SI window length is small; NW can’t provide large number of beams for UE beam sweeping procedure Increasing SI periodicity implies: - Increased latency. Longer time for UE to preform cell selection and cell reselection which will also impact how quickly a UE can access a cell for RACH procedures etc. Can consume more UE power. For DSS: - Even in a legacy deployment the current solution is not good and there might be a need to introduce e.g. more MBSFN subframes to counter for the legacy SIBs. However, without a future proof solution for NR new SIBs (MBS, UE power savings in rel-17 may introduce new SIBs) and posSIBs we see a high risk that there will not be possible to support new functionality together with DSS without deteriorating the performance. For Positioning SIBs: Also, R2-2110799 analysis show need of at least 9 SIs for positioning. - One version of RTK (~5 SI messages) - GNSS assistance data for one constellation (~3 SI messages) - DL positioning (1 SI message) Even with 80ms offset solution; we will not be able to schedule 9 positioning SIs. Pls note that these offsets based will anyway have the same constraints as mentioned in Observation Observation 1: If the shortest SI periodicity is x*si-WindowLength, the SI scheduling mechanism can only accommodate x SI messages. That is as 80ms SI needs to be repeated and hence we will be able to accommodate only 7 positioning SIs at maximum. It would become x-1 in fact. Further in Rel-17, there will be further new posSIBs (around 10) |
| ESA | Support | We agree with Ericsson´s analysis. The number of posSIBs is already high and it is expected to increase even more in Rel17. There is need to find a way to be able to schedule more posSIBs. |
| CMCC | Support | The identified issue is valid for the current SI mechanism in NR system. We are open if the solution can be backward compatible and future proof for more SIBs in the future. |
| CATT | unclear | The observations in R2-2110799 are quite objective. We need to figure out what the real issue is with the deployment, and how serious the issue is at first. CATT share the similar understanding as MTK that the SI scheduling issue mainly comes from positioning SI. This issue is valuable to further discuss since the posSI becomes larger and larger in Rel-17. We need further analysis before we jump into some solution, such as a new scheduling mechanism. |
| Huawei, HiSilicon | NAccept for Rel-16 Nsupport/unclear for Rel-17 | For Rel-17, we are not sure whether there will be such cases in real deployment. The major issue here is based on the assumption that quite a lot SIs are deployed for positioning and no 80ms SI are scheduled in SIB1, which we think is a corner case. This proposal leads to a fundamental change on current SI scheduling mechanism, and also requires the UEs to adapt to such new scheduling mechanism. The network usually can use proper implementation scheudling planning to avoid such corner cases. In any case, we cannot accept this mechanism extending to Rel-16 SIBs, which would impact legacy UEs. |
| Apple | Support with comments | We agree that the current posSI-scheduling is problematic when the minimum SI periodicity is not 80ms, we support to find a way to fix this issue and allow more posSIBs to be accommodated without causing conflict. |
| OPPO | NSupport | It’s also not clear whether such cases may happen in real field and we have concern on the proposed mechanims which may have big impacts on the Ues. |
| MediaTek | See comment | We see merit in Ericsson’s analysis; we think the scenario described in their paper is a bit too pessimistic, but indeed the problem can occur that systems need to either exclude some SIBs or modify their SI scheduling, especially where posSIBs are concerned and especially for DSS deployments. We think this could be addressed first by fixing the “80 ms only” issue with offsetToSI-Used, which would allow more flexibility in scheduling (and we consider that it could be done with the magic sentence, so that a Rel-16 UE need not be “locked out” of the posSIBs). Beyond that, let’s discuss how severe the residual problem is and whether it’s necessary to introduce a new scheduling mechanism to avoid the costs of longer SI intervals and/or shorter windows. We appreciate the operators’ involvement in this discussion and think it helps to clarify the need for a solution. |
| vivo | NSupport | At least RAN2 RAN2 should confirm the SI scheduling problem, i.e. whether more SI messages are really needed than possible so far. Moreover, The new SI scheduling mechanism is non-backward compatible. The performance of SI scheduling should be also considered, such as SI reception latency. In our understanding, SI deployment can be accommodated with a suitbale minimum SI period and suitbale window size. |
| Samsung | Support | We agree with the first proposal to discuss the justification of the problem in the operator perspective first, and if that was justified, then we suggest to start considering the change only in posSI offset modifications. |

C-DRX enhancements for 5G applications

R2-2109730	C-DRX enhancements for 5G applications	vivo, CMCC, China Telecom, China Unicom, Spreadtrum, Guangdong Genius	discussion	Rel-17	TEI17	R2-2107416

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| LG | NSupport | At the beginning of NR, RAN2 has discussed this issue, i.e., Active Time is not well aligned with frame boundary or Active Time does not incldue sufficient PDCCH Monitoring opportunity. However, it was considered difficult to keept the PDCCH-subframe concept in NR because of various numberologies. Thus, we are not in favor of introducing PDCCH-subframe like concept to NR at this moment (option1) Given that DRX cycle is defined in an absolute value, we are not sure how solution2 solves this problem. Our understanding is that solution3 would be the today’s implementation, i.e., no need to specify. |
| Nokia | NSupport | Agree with LG and wondering how that would work with dynamic patterns. |
| CATT | NSupport | This is quite a big change at this late stage and should rather be discussed in XR WI. Solution 3 (longer onDurationTimer) is enough for R17. |
| CMCC | Support | The issue can happen for some specific DRX configuration. We are open for solutions. |
| Huawei, HiSilicon | NSupport | We think such enhancements should be considered in Rel-18 relevant discussion, no duplicated discussion here. |
| Apple | NSupport | Agree with LG. This issue was discussed and NR at the early stage and also discussed in LTE previously, and RAN2 agreed the current after thorough discussion. |
| OPPO | Nsupport | It’s understood in the early NR discussion, this case was brought up however, due to the complexity of the UE monitoring duration, it’s decided to use absolute time for controlling the monitoring behaviour. In general, if network configure UL heary frame structure, it mean more UL traffic is expected compared with DL traffic, in order to balance the power saving and the low latency requirement as pointed by this contribution, one way is that netowkr can configure proper configured grant while keeping the DRX related timers in a reasonable duration. Thus, we dont think for now we need to specially handle this case, and we can keep the R15 design as it is. |
| Lenovo, Motorola Mobility | NSupport | Very likely DRX enhancement to support applications like XR will be discussed in Release 18, wherein similar issues, e.g., dynamic DRX to better fit non-integer traffic period, are considered. Optimization as such would be more appropriate to discuss in Release 18. |
| MediaTek | Unclear | We agree with the observation in this document, i.e. the current DRX framework does not take TDD operation very well into account, effectively forcing the use of longer DRX timers – leading to higher power consumption. However the solution being proposed isn’t very clear. Is it option 2, and if so, does option 2 mean that DRX timers disregard UL slots/symbols? Which DRX timers would this be applicable to? |
| ZTE | Nsupport | First of all, we think we can discuss it in R18 and no redundant discussion here, and, as NW vendor, we’ll try to cover at lease one DL slot for each onduration period when the NW configuring the DRX configuration to the UE. |
| Vivo | Support (Proponent) | It is true that there was some discussion on DRX cycle and timers at the begining of NR, i.e. ms/slot vs. PDCCH monitoring occasion. But considering the complexity, it is hard for NR to use unit of PDCCH monitoring occasion. Current unit of ms/slot, without considering TDD frame flexibility, has no issue in early deployment of NR, where long DRX cycle is actually configured. But with the deployment of 5G, more applications have requirements of latency, and also need power saving, e.g. XR, or cloud gaming. For these applications, short DRX cycle will be configured to seek for the balance of power saving and latency. In order to achieve power saving, shorter drx-onDurationTimer will be configured. In case several subframes are configured as UL, there will be case that the running DRX timer duration may (or partially) fall into the UL subframes. In this case, UE will have no (or less) opportunities to monitoring PDCCH. In this way, the latency requirements for the traffic would not be guaranteed. Regarding option 2 (reply to @LG): one potential solution is DRX timer could be adapted if falling into UL slot. We are open with the solutions. As in last RAN2 meeting, some companies confirm the problem, but think it should be included in Rel-18 XR WID. We think more time is needed for companies check after the discussion on Rel-18 XR scope stable. Thus, we could accept to postpone the discussion for this TEI by now. |
| China Telecom | Support | This issue may happen for some specific DRX configuration. We are fine to discuss the solutions and find potential enhancement. |
| NEC | NSupport | As the issue is only for TDD, it is not sure why network cannot take into account TDD config (DL/UL) in DRX configuration? |
| Samsung | NSupport | Agree with LG and we do not think the possible misalignment of DRX period and UL subframes as an urgent problem. |
| Docomo | See comments | Tend to prefer to tackle this in Rel-18 for more consistent solution with e.g. RAN3 and/or SA. |

New Proposals (has not been treated yet for R17)

EPS Fallback

EPS Fallback

R2-2110485	EPS fallback enhancements for UEs in IDLE/INACTIVE	Huawei, HiSilicon, CMCC, China Telecom, China Unicom, LG Uplus	discussion	Rel-17	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Support (Proponent) | Following legacy procedure, when an IDLE/IACTIVE UE is paged for voice, the EPS falllback latency includes 1) UE response paging and establish/resume RRC in NR Cell, 2) the HO/redirection procedure triggered by the NR cell blindly or based on UE measurement reporting, 3) UE initiates access to LTE cell to get voice service. The EMR based solution proposed by vivo is trying to save the time of measurement configuration and reporting time in NR side. While the key point in this contribution is that after UE receives the paging message indicating EPS fallback, UE can directly select and access to the LTE cell, which can save the time of all procedure in NR side. This also omits the measurement reporting, while we understand the real scenario in the field is largely blind HO/redirection scenarios, assuming LTE coverage is better than NR. |
| Nokia | NSupport | Decreasing latency is interesting for voice fallbacks in general. We are wondering what is different compared to LTE CS fallback – this seems analogous to that. And secondly, we wonder why focus on MT cases? Shouldn’t one also consider MO like in CS fallback that UE establishes connection directly in LTE(or wherever)? Additionally at least in the past there has been strong concerns on adding new IEs in the paging message as it would decrease the paging capacity. We are wondering why this would be different now? Huawei response: we understand in LTE, CS fallback is triggered by UE, i.e. UE request to do CS fallback in NAS message, based on which MME will inform RAN to perform the corresponding AS procedures. In NR, it was assumed UE and CN would support IMS voice, while gNB can decide if EPS fallback is needed according to support of VoNR etc [this is described clearly in 23.502]. So we think even for IDLE/INACTIVE UE, it still should be the gNB to make the decision on EPS fallback. Then for paging capacity, in LTE, CS/PS indication is included in paging, and here EPS fallback is just a similar one-bit indication, would not impact capacity much. |
| CATT | Unclear | Some issues on the below enhancements for EPS fallback is to be clarified, - When the paging message indicates voice service, the UE sets the NR RRC establishment cause as voice instead of mt-access. [CATT]if paging cause is set to voice but actually the voice is VONR,UE will still set RRC establishment cause as voice, then it seems gNB will falsely treat is as EPS fallback. what is the impact? [Huawei] we are not sure we understand the comments on “gNB falsely treat as EPS fallback”, as EPS fallback can only trigger by gNB. The cause value only indicate the UE access to gNB for voice service, the gNB will determine if it will provide UE voice service via VoNR or trigger EPS fallback. - The gNB can include EPS fallback indication in paging message, the UE selects an E-UTRA cell to establish the RRC connection, and sets the E-UTRA RRC establishment cause as voice. [CATT] based on EPS fallback indication in paging message ,the UE select a LTE cell and initiate RRC connection on LTE cell autonomously without involving gNB and 5GC. We are wondering whether it is still EPS fallback. EPS fallback procedure contains interaction between gNB and 5GC, and between CN network entities, according to TS23.502. [Huawei] in our solution, it is like the UE receives a NR RRC redirection message (i.e. “or AN Release via inter-system redirection to EPS” as you coted below) to E-UTRA cell. After UE accesses to E-UTRA cell, it will perform TAU which trigger EPC retrieve the UE context (including voice service) from 5GC. So we think this is consistent with the legacy 5GC-EPC interaction principle. //23.502 4. NG-RAN responds indicating rejection of the PDU Session modification to setup QoS flow for IMS voice received in step 2 by PDU Session Modification Response message towards the PGW-C+SMF (or H-SMF+P-GW-C via V-SMF, in the case of home routed roaming scenario) via AMF with an indication that mobility due to fallback for IMS voice is ongoing. The PGW-C+SMF maintains the PCC rule(s) associated with the QoS Flow(s) and reports the EPS Fallback event to the PCF if PCF has subscribed to this event. 5. NG-RAN initiates either handover (see clause 4.11.1.2.1), or AN Release via inter-system redirection to EPS (see clause 4.2.6 and clause 4.11.1.3.2), taking into account UE capabilities. The PGW-C+SMF reports change of the RAT type if subscribed by PCF as specified in clause 4.11.1.2.1, or clause 4.11.1.3.2.6. When the UE is connected to EPS, either 6a or 6b is executed |
| CMCC | Support | This is an efficient solution to reduce EPS fallback delay. |
| Apple | Unclear | We have several questions and comments on this matter: 1) Does the EPS fallback indication come from gNB directly or from AMF? If it’s decided by AMF, CT1 and SA2 should be involved. 2) We also wonder why only MT call matters? Why not considering MO calls? 3) We have some concerns regarding the reliability of using paging message to indicate the EPS fallback. In legacy EPS fallback procedure, UE is already in RRC connected state thus the EPS fallback indication from NW can get to UE for sure. Using CCCH channel instead without knowing in which cell the UE actually locates, might be a big concern. 4) The capacity and security of using paging message to indicate EPS fallback should also be considered. |
| OPPO | Unclear | We understand the motivation, however, it’s not clear for us for the following aspects: - Whether the enhancement on the paging for MUSIM can be resused by this mechinams If 1 is confirmed, we understand the key enhancement is to enhance the paging message in from gNB to the UE, i.e., based on the paging mesage with voice indicatation, the UE can perform RRC connection to E-Utra. However, it’s not clear to us why the UE need to set the RRC establishment cause as voice when performing E-UTRA RRC establishment. |
| Vodafone | Unclear | We support the motivation and need for TEI-17 work to reduce EPS fallback latency/improve reliability, however, we suspect that there are ‘systems aspects’ that mean that this proposal is incomplete. |
| Lenovo, Motorola Mobility | unclear | Regarding EPS fallback in inactive the RAN Paging message is used like RRC release message with redirection command to LTE/EPS. However, we wonder whether UE can trust the received paging message, e.g. in view of fake base stations or man-in-the-middle attack. Therefore, there may be security issues with this enhancement. Regarding the solution of setting the NR RRC establishment cause as voice (mo-VoiceCall) instead of mt-access, it will face the risk that the establishment request could be rejected by network. For the solution of EPS fallback indication, if EPS fallback indication is proposed to be included in paging message, we need to evaluate the load of current paging messgae. It may impact the current paging capacity. |
| MediaTek | Unclear | It is not so clear that how much latency could be reduced by this proposal (e.g. 1% or 10% of call setup time). We understand the connection establishment time is not so high within the total call setup time. It is also unclear that why the UE has to change the establishment cause from mt-access to mo-VoiceCall, is it really TRUE that networks will prioritize the mo-voiceCall over mt-access call? For the assistant E-UTRAN frequencies broadcasted in SI, is it up to UE implementation to selection which frequency to perform the MT call? Is the UE request to measure this neighbor frequecies before receving the paging? |
| vivo | Support | There is indeed a need on the reduction for the latency of EPS fallback, we think the paging enhancement for EPS fallback could help. However, regarding the first direction for EPS fallback, i.e.“ When the paging message indicates voice service, the UE sets the NR RRC establishment cause as voice instead of mt-access“, we think directing UE to perform early measurement when receives paging can save more time. |
| China Telecom | Support | To reduce the EPS fallback delay is important for both the network and the UE. We support to have some enhancement on this issue to make better uesr exprience. We think this solution is efficient without too much spec impact. |
| NEC | NSupport | From network point of view, it is confusing about what happens at UE side. |
| Samsung | Support | We support to discuss how to reduce the EPS fall back latency in case that UE is paged in RRC_IDLE/INACTIVE. |
| Docomo | unclear | We share the motivation to shorten the latency of EPS fallback, but we wonder if the paging message in the proposal should be integrity-protected. |
| LG Uplus | Support | When the SA user uses the VoLTE, silence may occur due to EPS fall back delay. To avoid this situation we adapted some solution, but it is not enough to solve silence issue. So, We support how to reduce the EPS fall back latency. |

UL Skipping Control

UL Skipping Control

R2-2110198		Fast Control of UL Skipping	NTT DOCOMO INC., Ericsson, CMCC, Verizon	discussion	Rel-17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| LG | NAccept | P2 has been discussed in RAN2#115 and not pursued, hence it should be excluded in this discussion. Regarding P1: Such dynamic on/off may complicate the UE behaviour because the generation of the MAC PDU may need to depend on the timing of receiving such MAC CE. For example, sudden change to skipping ON while the UE is already preparing a MAC PDU or sudden change to skipping OFF while the UE has already generated a MAC PDU. We already have a similar experience, e.g., CSI reporting considering sudden Active Time or sudden non-Active Time in DRX, which is complex even today. In addition, we don’t think the SINR situation is so dynamically change and requires very dyanmic on/off of skipping. Lastly, for false detection case, the UE ignores the received grant for the skipped transmission. So, we don’t agree with the view that the gNB will have problem with soft combining issue or the UE may use this wrong grant for UCI multiplexing. |
| Nokia | Unclear | LG’s concerns are relevant. |
| CATT | NSupport | It’s not necessary to introduce the optimization. |
| CMCC | Support |  |
| Huawei, HiSilicon | NSupport /Unclear | It is noted that the network is free to disable UL skipping in advance whenever needed, since from LTE. It is unclear whether there is any real problem for reusing this mechanism on NR. This may also lead to impact on the LCP, i.e. the feasibility for UE timing during LCP if this becomes a fast control via MAC CE. |
| OPPO | Nsupport | Currenlty the UL skipping is already supported by RRC configuration, it’s not clear why we need MAC CE based UL skipping control. |
| Apple | NSupport | We understand the proposals might reduce complexity at the gNB side. However, the solution seems to address a corner case. A false negative (or misdetection) of a TB transmitted by UE is not necessarily related with UL skipping. A false positive detection of an UL transmission is an error at the gNB side and a corner case. It was agreed in the last RAN2 meeting not to adopt the specification for this (e.g., ‘retransmission of configured grant with empty buffer’). Moreover, we are not convinced proposal 1 is essential. The problem mitigated here is rather minor. It was discussed in last R2 meeting that the gNB can avoid UL skipping together with TB repetition as part of the scheduling. In our understanding it is sufficient to rely on RRC reconfiguration for overall configuration control, also the latency (with respect to cell edge etc.) seems sufficient. We also support the comments made by LG. Proposal 2 encompasses that the UE disables UL skipping following reception of a DCI with K>1. This is not acceptable as it requires cross-layer interaction in the time-critical period of UL grant processing. In summary, the change agreed in the last RAN2 meeting (e.g., the RRC change to consider the REPETITION_NUMBER per the MAC spec) is sufficient in our view. |
| MediaTek | NSupport | Our concerns are on keeping the UE and the gNB in sync. LG raises a valid point on the complexities on when to apply skip/don’t skip commands, which would be similar to the issues surrounding CSI reporting in DRX. Furthermore, MAC CEs do not have the same level of reliability as RRC signaling, and a loss of the MAC CE can lead to significant interoperability issues. |
| ZTE | Nsupport | We already have two enable flag in RRC to activate/deactivate the UL skipping |
| NTTDOCOMO | Support | Proponent. Since UL skipping does not work well in low SINR condition, it is reasonable for network to swtich off the function. Although RRC configuration based switching on/off is supported in rel-16, it is slow which may take 10~100ms. Therefore, we believe it should be enhanced with a faster mehtod e.g. MAC CE or DCI. |
| vivo | Nsupport | We are not convinced whether the miss-detection probability would indeed increase in the mentioned case. Basically, for low SINR case, the NW would schedule a UL grant with a more robust MCS (i.e. lower MCS index with lower SE), where more PRBs will be allocated for the UL grant. Then, the miss-detection probability of PUSCH DM-RS can still be guaranteed, as shown in the following simulation plots (e.g. with more PRB allocated via lower MCS index, miss-detection probability can be kept at 10^-3 level even when the SINR is decreased from -4.4dB to -7.7dB). Take one step back, with proper beam management and NW implementation (e.g. setting a reasonable detection threshold), we don’t think the radio link quality will change frequently and dramatically (also note that the miss-detection is generally a very very corner case), in this sense, there is no need to introduce MAC CE for fast control. The existing mechanism is sufficient. In conclusion, we fail to see the motivation of using MAC CE control for UL skipping enabling/disabling. |
| NEC | unclear | It is not sure how much this is critical. Is there any problem found in the field? The solution does not seem to solve the issue (at least obervation 1 which should be the main motivation). |
| Samsung | NSupport | We prefer RRC configuration based UL skipping to increase the reliabiility of network operations. L2 signaling does not guarantee the successful transmission, and we do not think more dynamic control is needed. |

SRS in Dormancy

SRS in Dormancy
Had some support in R16 but wasn't done in the end

R2-2110836	Periodic SRS in SCell dormant BWP	Qualcomm Incorporated, ZTE Corporation, Futurewei	discussion	Rel-17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Nokia | unclear | To us having very seldom SRS is hardly helping NW in assessing quality of the cell. But RAN1 also indicated in R16 that this could be useful. This seems quite simple addition, so we are neutral on having this if UEs are willing to send it for dormant BWP. We do not see the need to change the PHR behavior though as proposed in P3 since it should be very sporadic transmissions without impacting other cells much and no close loop power control for the dormant BWP. |
| CATT | Support |  |
| OPPO | NSupport | It is already discussed in R16 DCCA and it is not agreed. If RAN2 wants to support it in R17 TEI. RAN1 should be involved first. I also think it is RAN1 scop to decide whehter to support it or not. |
| Lenovo, Motorola Mobility | NSupport | It has been already discussed in R16 and not agreed. |
| MediaTek | NSupport | This is one of controversial issue in R16 and R2 decided not to support this due to no consensus. We see some benefit to have SRS in dormant SCell but does not think the gain is high enough to re-discuss this again. If simple solution could be introduced, we are acceptable to this. |
| Futurewei | Support (Proponent) | During Rel-16, many companies had concern on the long wake-up delay from the dormancy. RAN1 and RAN4 acknowledged the concern and suggested long-periodic SRS in dormancy is a compromised solution to minimize the wake-up delay from dormancy while maintain low power consumption. We support to adopt this solution to reduce the transition delay when a UE is out of dormancy. |
| NEC | unclear | No strong view but we assume the main impact is in RAN1, although signaling should support in RAN2. |
| Samsung | NSupport | We believe the Dormant BWP is introduced for power saving and transmitting periodic SRSs within the dormant BWP may reduce the efficiency of power saving. |

Skip RACH on Data Arrival

Skip RACH on Data Arrival

R2-2111161	Skipping RACH upon data arrival	NTT DOCOMO, INC.	discussion	Rel-17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
|  | NSupport | We have some sympathy to the intention that the network may want to poll the BSR rather than the UE by itself always trigger the BSR and consequently SR/RA. Currently, the only way to prevent BSR trigger by UE is not to allocate a LCG. However, it prevents BSR report as well because BS is reported per LCG. R2-2111171 has proposed to allow skipping RA for this case, which we don’t think is the only solution. For example, we could enhace BSR so that BSR is not triggered by UE itself. Therefore, we are open to discuss more but not limited to RA skip. |
| Nokia | Nsupport | There are already means to prevent triggering SR like logicalChannelSR-DelayTimer and logicalChannelSR-Mask |
| CATT | NSupport | It is not essential. |
| Huawei, HiSilicon | NSupport | We understand by existing mechanism, the network can configure SR mask to prevent the UE from triggering SR-RA in case it thinks SR-RA is not needed. It seems to be sufficient and a bit unclear to us why new mechanism is needed. |
| OPPO | NSupport | It seems this contribution assumes the UE is always reacheable if the beam turns to the UE in FR2. In this case, by requesting aperiodical CSI, the UE is assumed to have the opportunity to send the BSR and even the ul data (if any). However, we think this is not always the case, because UE should be supposed to work with DRX in order for power saivng, in this case, UE is not always reachable in the case when UE is not in Active Time. Then, SR can be used to trigger the UE entering into the Active Time when UL data is available. Thus the statement in the contribution „This means that the gNB can receive UL data from the UE without interruption when the gNB requires BSRs frequently enough. In such an operation, scheduling requests can be stopped by configuring in RRC“ seems not alwasy work especially considering the DRX configuration. |
| Lenovo, Motorola Mobility | NSupport | This is presumably not good from system/ network’s perspective since due to Beam sweeping the amount of unused resources (just now put in the PDCCH to send a possible BSR) will be too high. |
| MediaTek | NSupport | As commented by Nokia and Huawei, there is already mechanism to solve this. It seems that no further enhancement is needed. |
| vivo | NSuport | In ideal case, it can save RACH procedure by transmit BSR/data using the grant that is issued for CSI MAC CE. However, it could be that the gNB does not have DL data and thus does not need CSI at all, which means that the gNB probably does not schedule the UE for CSI report. In this case, the UL data is blocked if RACH-SR is not allowed. |
| NEC | NSupport | This goes opposite direction compared with CG. This may cause unnecessary PDCCH at every time of beam sweeping with corresponding beam for a UE. |
| Samsung | NSupport | We believe the current specification already supports RA cancellation for SR triggered for BSR. |

Fast RLF

Fast RLF

R2-2110055	Discussion on Fast RLF recovery	Apple, Verizon	discussion	Rel-17	TEI17

R2-2110056	38.331 CR to introduce fast RLF recovery (Option 1)	Apple, Verizon	draftCR	Rel-17	38.331	16.6.0	B	TEI17

R2-2110057	38.331 CR to introduce fast RLF recovery (Option 2)	Apple, Verizon	draftCR	Rel-17	38.331	16.6.0	B	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Ericsson (Tony) | NSupport | We think that this is an optimization that basically bring no benefits. With the proposed solution(s) we think that gaining e.g., around 20-30ms, is not really essential taking into account that the reestablishment procedure itself can last several second (from the moment to when is initiated to the moment in which the RRCReestblishmentRequest is sent). Therefore, we gain few milliseconds in a procedure that last several seconds. A further doubt that we have is that, sometime, the RLF may be due to a reconfiguration error and thus restoring the previous configuration it will bring yet to another RLF (in this case the proposed solutions do not help at all). On top of this, when a UE performs reestablishment, the network may also want to give a different configuration at the UE for e.g., load balancing reasons, and this will basically void the small benefits given by the proposed solutions. Also, we are wondering if the case that is describing (UE with temporary out-of-coverage) can be simply resolved by extending the timer T310 (also simply set the maximum allowed time). For all these reasons, the benefits for restoring the previous RRC configuration during reestablishment it seems to be very limited. |
| Nokia | NAccept | It is not clear if this really reduces any delay. RRC reestablishment is already supported and UE does not release RRC configuration as stated in discussion paper. It seems only aspects is to keep PSCell configuration in case of RLF but that hardly seems to bring any benefit. |
| CATT | NSupport | RRC re-establishment may be triggered due to RRC reconfiguration failure, e.g. inability to comply with RRCReconfiguaration message. In this case, the previous configurations cann’t be used. Thus, the benefit of fast RLF recovery is not clear. |
| Huawei, HiSilicon | NSupport | The proposal is apparently specific to the case when the UE re-establishes to the same cell after RLF. This is rather unlikely to occur. Besides, the CRs provided are not simple and may require a lot of discussions. Thus we are not convinced on this proposal. |
| Apple (Proponent) | Support | We think it is quite frequent that UE will select the same PCell (which is the serving cell before the failure) during RRC Establishment procedure triggered by RLF or HOF. In this case, it is reasonable to allow UE to continue to use its prior configuration w/o let NW to reconfigure the UE. This can help reduce the signaligng overhead and latency of the recovery. We have presented two options to address this problem, one is to extend the CHO configuratuon to cover the source cell. The other is to adopt a procedure simialr to RRC resumption when the select cell is the same prior PCell. |
| Lenovo, Motorola Mobility | NSupport | If the cause of declaring RLF is T310 expiry, the network has reserved the sufficient time for recovery of channel. If the channel can not be recovered during T310, it is almost impossible that UE can reselect the same cell during T311. If the cause of declaring RLF is reconfiguration failure, the proposed solution can not work. Actually, we already have some enhanced solutions to speed up the recovery e.g fast MCG link recovery and CHO recovery. |
| MediaTek | NSupport | We understand that the proposal is to optimize the case that re-establish to the previous serving cell. We are not sure this is common scenario. As RLF should already be low possibility in normal deployment, we don’t think it is necessary to enhance on this particular case. |
| Xiaomi | Support | RAN2 has supported UE could try HO in case selected cell is one of the CHO target cell during re-establishment. UE is expected to store the source configuration during handover. So, it follows the same logic for UE to resume RRC connection with source cell if selected during re-establishment. |

| Futurewei | NSupport | It is an optimizaiton useful only for an unlikely scenario, and the specs impact don’t look very straightforward. |
| --- | --- | --- |
| vivo | NSupport | 1. The probability that UE selects the same PCell during cell selection We think the time from UE judges and declares the RLF to cell selection completion before sending the RRC Reestablishment request is very short. it is with low probability for UE to select the same PCell during cell selection in such a short time. Unless the T311 is configured to a large value, e.g., the default value of T311 is the 30s. So, maybe it is better to have a specific simulation analysis to indicate it is with a very high probability. Besides, 26ms is ok, and the low latency requirement is more suitable for URLLC. 2. Regarding the specific option 1) Option 1 We think the RACH and RRCReconfigurtionComplete may not be needed if the network does not detect UE’s RLF, considering the time for the network to detect UE’s RLF is longer than that of UE. So, if UE selects the same PCell which it was camped on prior to RLF or HO Failure, UE can restore the pre-stored configuration by implementation and the user plane data is resumed. Then the impact for the current spec is low. 2) Option 2 The RRC Reestablishment procedure is similar to the RRC resume procedure. According to the previous meeting and spec, for the RRCReestablishement procedure, UE can only resume SRB2 and all DRBs after receiving the RRCReconfiguration message rather than the RRCReestablisment message. We think it is not so suitable for two RRC procedures to be so similar which in fact are defined as two different procedures in the spec. |
| NEC | NSupport | No strong view, but if this is only within the same PCell, then may be acceptable. Firstly would like to check this point. |
| Samsung | NSupport | In our understanding, RLF declaration itself also has the time duration to check the link status such as T310, or some consecutive number of failures in RLC and RACH. If link has failed for this duration, it is thought better to find another cell. There is no need to adjust UE behavior only for the specified scenario in this Tdoc. Moreover, we wonder if the current spec prohibits the case that current Pcell can be given as candidate target cell. If this is allowed, the UE can use CHO recovery scheme for the failed cell further without proposed spec modification. |
| Google | NSupport | We understand this proposal addresses the radio link failure scenario only. However, it also complicates the UE implementation because the UE has different handlings for the radio link failure scenario and other scenarios triggerring the RRC re-establishment. In addition, the user does not perceive the time saved by the proposal. We don’t see a need to optimize it. |

Idle / Inactive Measurements w SUL

Measurements

R2-2109773	Idle/Inactive state measurement enhancement for UEs supporting SUL	OPPO, Spreadtrum Communications, Qualcomm	discussion	Rel-17	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Nokia | NSupport | This would bring at most negligible UE power saving gain for SUL UEs if any. And setting this parameter and optimizing it for NW will be challenging. |
| Huawei, HiSilicon | NSupport/ unclear | It is unclear how much gains we can get from such enhancements, considering large number of existing UEs, we think the gain is marginal. |
| Apple | Nsupport | Generally speaking, the configuration on those parameters would not be that extreme low, otherwise the mobility performance may get compromised. |
| OPPO | Support (proponent) | Having separate measurement start threshold for SUL capable UEs can help these UEs to save power, i.e. these UEs only need to perform neighbour cell measurement when they are near the SUL‘s coverage edge, which is different from NUL’s coverage edge. |
| Lenovo, Motorola Mobility | Nsupport | It might be good if proponents provide some quantitative analysis on the gains wrt UE power consumption that can be achieved with the proposed enhancement. |
| MediaTek | NSupport | We understand the intention is for power saving. Although the proposal is acceptable to us, we doubt that network vendor will really configure this parameter. As mentioned by Nokia, it will request some study on how to set the value properly. We thus that think the enactment is not a must. |

| Futurewei | NSupport | The potential gain is not clear, while the impact on network implementation may no be trivial. |
| --- | --- | --- |
| NEC | NSupport | We do not see benefit which can justify undesirable complexity and overhead. |
| Samsung | NSupport | In order to modify SIntraSearchP and SnonIntraSearchP in SIB2, the SUL UE’s large power consumption due to existing parameter values must be studied and verified first. |

RMSI alignment and HARQ granularity

Miscellaneous

R2-2110558	RMSI alignment and HARQ granularity	Nokia, Nokia Shanghai Bell	discussion	Rel-17	TEI17, NR_unlic-Core

Note that this document has two proposals that should be considered individually:

RMTC: Enhance RMTC-Config to allow RSSI measurements to be contained in gNB idle periods.

HARQ: Allow more granular configuration of PDSCH HARQ processes for UE in Rel-17.

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Nokia | Support both (proponent) |  |
| CATT | For RMTC, unclear For HARQ，unclear | For RMTC and HARQ, from the view of Ran2, we are fine to accept it, but we think the RMTC part should be discussed in RAN4 firstly, since it relate to a more meaningful measurement in idle time of every Nth gNB FFP, the benefits shoudl be confirmed by RAN4. The HARQ part should be discussed in RAN1 to see whether it has the potential impact to HARQ procedure and the UE chip. |
| Huawei, HiSilicon | NSupport /unclear | It is a bit unclear what the issue 1 is, and hence not clear what is needed to be done. For issue 2, sensible UE implementation already takes care of the HARQ buffer management, and it has been acknowledged by the UE vendors. In this sense, not sure any enhancement is needed. |
| Apple | For RMSI unclear | Regarding RMTC one, so far we don’t see a clear motivation yet. Besides, we are wondering would it be possible to use RSRQ by network implementation to derive RSSI? e.g., use SSB based RSRQ but indicate the RSSI symbol locations, and then use RSRP divided by RSRQ network may still get rough RSSI during certain symbol level period (not exactly the LBT BW). It’s not so starightforward like RMTC configuration, but I think it still can work roughly. |
| OPPO | Nsupport | We agree the issue may exist, but the impact can be limited especially when a smart UE uses a proper algorithm. To me, it is not very essential, there is no need to enhance RMTC-Config at this late stage. |
| MediaTek | RMSI – Unclear HARQ – No strong view | For RMSI, we are not so sure about the motivation and it looks more like a R4 Topic. |
| vivo | NSupport | Both proposals are like optimizations. Regarding HARQ: more granular configuration of PDSCH HARQ processes is not essential. Current granularity {n2, n4, n6, n10, n12, n16} is enough. |
| NEC | NSupport | For HARQ one, it is acceptable. |
| Samsung | NSupport | Although the first proposal makes some sense, we beilieve that the existing RMTC-config already covers the scneario (mostly). For the second issue, we understand that RAN2 already discussed and concluded that allocation of 16 HARQ processes from the UE side is not an issue (as in Rel-15 and Rel-16). |

UE assistance information configuration in RRCResume

Miscellaneous

R2-2109474	UE assistance information configuration in RRCResume message	OPPO	discussion	Rel-17	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Nokia | NSupport | If we understand correctly this would allow to send otherConfig in resume message without needing separate RRC reconfiguration message. So this could save one RRC message in case where the delay is not really issue so hard to see motivation for this change. |
| CATT | NSupport | It is not urgent to report UE assistance information to the network when the UE transfer from RRC_INACTIVE to RRC_CONNECTED. |
| Huawei, HiSilicon | NSupport | We understand this is an enhancement for UAI configuration to reduce one more RRCReconfiguration message. But we are not sure if it really reduces the signaling overhead, the NW may still need to send RRCReconfiguration message for other configurations in addition to UAI configuration. So we don’t think this enhancement is essential. |
| Apple | Neutral | We think this is good to have but no strong opinion. |
| OPPO | Support | Proponent |
| Lenovo, Motorola Mobility | NSupport | We think that this is a minor optimization. |
| MediaTek | NSupport | We understand there is no urgency to provide OtherConfig. The proposal is just a minor optimization. |

| Futurewei | NSupport | This seems a very minor optimzation, compared to other enhancements for consideration. |
| --- | --- | --- |
| NEC | NSupport | We do not see any need of this optimization. |
| Samsung | NSupport | The proposal to include otherConfig IE in RRCResume msg is not a necessary modification. |

Efficient UL pre-scheduling

R2-2110759	Efficient UL pre-scheduling operation	MediaTek Inc., Qualcomm Inc.	discussion	Rel-17	TEI17	R2-2109019

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| LG | NSupport | We wonder why the network provides such useless UL grant axcessively. More safe and helpful way would be to report BSR=0 in this case so that the network does not provide more UL grant until the UE requests so. |
| Nokia | NSupport | Agree with LG. Also, this cannot be introduced as mandatory behaviour for backward compatibility reasons (with legacy gNBs). Explicit configuration of the feature will be required and overall operation will still have to rely on smart gNBs. |
| CATT | NSupport | P1 can be a vast topic and, in our understanding, is part of R18 XR scope. P2 requires that the MAC entity is configured with enhancedSkipUplinkTxConfigured and the problem is that if the UE receives a dynamic grant for new transmission, it starts the CGT, so it cannot go to sleep as it may receive a dynamic grant for a ReTx. So the issue raised by LG in R2-2111170 should be addressed first. |
| Huawei, HiSilicon | NSupport /unclear | We are not sure about the complexity for checking LCP result whenever handling the inactivity timer, and felt the potential benefit is marginal. |
| OPPO | unclear | We understand the motivation of the proposals. From UE’s perspective, it would be beneficial for UE power saving if UL pre-scheduled grant is unused due to no UL data at UE side. However, we are not sure whether this intensive UL pre-scheduling operation is widely used in real field, maybe we need to check with NW vendor. |
| Lenovo, Motorola Mobility | NSupport | For P1 “Introduce assistance information to indicate that whether current UE application requires low latency transmissions”: We think the need should not arise since due to QoS negotiation and information available at gNB (from CN) it would know what latency requirement is for a DRB; and secondly, the proposal can be misused by UEs. For P2 “Support of enhanced DRX inactivity timer operation to reduce power consumption when UE has no UL data to transmit“: Drx-inactivitytimer is directing UE to go to ActiveTime for reception of DL and UL DCI. Even though UE may not have UL data, there may be still DL data for the UE pending. Furthermore, gNB may also ask for e.g. aperiod CSI etc. Therefore, we don’t think that this is a good solution. |
| Apple | Support the intention / Unclear | 1) We do not think P1 is required. The network should know whether there is any DRB with a low latency requirement, and BSR is available as well. 2) We are ok to discuss P2/P3 in Rel-17 and think changes to address this problem can be useful. However, we do not agree to taking the earlier QC proposal as baseline (P4) at this stage. 3) We also support the view from OPPO. |
| ZTE | Nsupport | - For proposal 1, we also have no idea why NW will pre-schedule the UL grant for a UE when there is no any SR/BSR received, in our understanding , NW always schedule UL grant according to the SR or BSR. Assuming NW’d like to do some pre-schedule things, UL skipping can be utilized. - We tend to agree with lenovo, the drx-inactivityTimer is not only used for burst coming data but also for UE to receive the DL transmission. |
| vivo | NSupport | Our concern is that this optimization will increase the risk of DRX misalignment between UE and gNB and complicate UE and NW implementation with limited power saving gains. To address the misalignment, new mechanisms will be evaluated and introduced. However, from the perspective of service QoS guarantee and system efficiency, UE should start/restart inactivity timer upon UL grants reception. If further power saving effect should be pursued in this scenario, some implementation solutions can be considered, e.g. configuration of a shorter inactivity timer. |
| NEC | unclear | Looking at TP, it changes the legacy behaviour (by mistake?). We are open for discussions but would like to confirm some aspects: 1. no impact to legacy 2. how NW can manage the assistance information on whether current UE application requires low latency transmissions, when multiple DRBs are configured? |
| Samsung | NSupport | Regarding P1, it seems not essential, as SPS/CG is already utilized without having such assistance even today. Regarding P2-4, we still think that it is not needed as discussed in Rel-16. Since network already knows the UE behavior from the specification, (sensible) network should refrain providing such unsolicited UL grants if it configures DRX to the UE. |

Multi-TB CGs on licensed bands

R2-2109652	Enabling Multi-TB CGs on licensed bands	CATT	discussion	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| LG | NSupport | Using HARQ formula when cg-RT is not configured but multi-TB CG is configured, it will allocates the same HPID to all HARQ processes within the CG period. In unlicensed, it is of not problem because the intention was to allow pending data transmission by using the same HPID. However, for licensed, retransmission of pending data is not an issue. If multi-TB CG is for transmitting new data in licensed, different HPID needs to be allocated, which we think is a specification impact. Also, multiple CG configuraiton would provide similar CG occasions, hence see not much need to support multi-TB CG in licensed. |
| Nokia | NSupport | This was discussed in RAN1 in Rel-16 NR-U WI and concluded as not supported for licensed. |
| CATT | Support (Proponent) | To LG: Note that even without this proposal, multi-TB CGs are already currently supported when cg-RetransmissionTimer is not configured, since it is allowed, in R17 to not configure cg-RetransmissionTimer in shared spectrum. In other words, multi-TB CGs are already supported when the same HPID is allocated to all CGs within the CG periodicity. And that does not raise any problem, on the contrary it can serve other IIOT purposes such as addressing traffic jitter as discussed in above contribution. So the intention here is just to extend this behavior to licensed bands, because, if the mechanism works in UCE (w/o CGRT configured), there is no technical reason to prevent it from being used in licensed bands as well. |
| Huawei, HiSilicon | unclear | Is it RAN1-led or RAN2-led? |
| OPPO | Unclear | In unlicensed band, we understand the reason to introduce multi-TB CG is to increase the transmission opportunity when the UE occupies the channel, however, it’s not clear the motivation to introduce Multi-TB CG on licensed band, thus we think there is no need to copy the unlicensed mechanisms to licensed band. |
| Apple | NSupport | In our understanding multi-TB scheduling is only considered useful when LBT-failures can happen (i.e., when cg-RetransmissionTimer is configured). We are also not convinced an extension of multi-TB transmissions to licensed bands has no additional specification impact. |
| MediaTek | NAccept | We do not find a strong justification for this change. It is suggested that this is a means to deal with jitter for IIoT traffic. However, we have repetitions available as a solution to deal with jitter since Rel-15. The NW can configure the UE to use any of the individual repetition occasions for initial transmission. |
| ZTE | Nsurppot | As per the intention mentioned in contribution, we think multiple CG is introduced for dealing with jitter issue. We do not see any more enhancements for improving the same issue. |
| Futurewei | NSupport | Should this be discussed in RAN1 first? |
| vivo | NSupport | In NRU, it is beneficial to use multiple slot scheduling since UL DL switch means additional LBT and could suffer losing of channel occupation. However, in licensed carrier, there is no such issue. Multiple TB CG could hard to match the varity of the burst size. Its pros and cons needs to be investigated. |
| NEC | NSupport | Actually there is no strong motivation to apply this in lincensed spectrum. |
| Samsung | NSupport | We do not see a strong motivation of Multi-TB CGs for lincensed band. In the licensed band, multiple CG configurations can be used. |

Pending empty PDUs

R2-2109651	Handling of pending empty PDUs after UCI multiplexing	CATT, Lenovo, Motorola Mobility	discussion	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| LG | NSupport | We have symphathy to the intention and are open to discuss. However, flushing the buffer is not sufficient because CGT is started and transmission using this CG will be blocked until CG expiry. We think CGT and CGRT should not be started for this empty PDU and HARQ process status should be kept as not pending regardless of LBT failure indication. |
| Nokia | unclear | We are fine with the intention but current text impacts legacy behaviour. New behaviour must be limited to the cases where Rel-16 features like autoTx, CG retx timer or Rel-16 UL skipping is/are configured. |
| CATT | Support (Proponent) | We acknowledge the LG’s observation regarding CGT (and BTW, we support your proposal), but we think it is another problem: we address the useless continuation of the retransmission attempts of this empty PDU while you address the blocking of the HP associated with this transmission by the CGT. |
| OPPO | NSupport | So the scenario described in the contribution is that when UCI mutlipexing is performed due to overlapping between PUSCH and PUCCH, the UE may transmit the MAC PDU multiplexed with UCI in the PUSCH even if the MAC PDU is empty. Then, it’s not necesasry to perform retransmission due to the empty MAC PDU. We think keeping the MAC PDU in the buffer does not casue any issue because network is not aware there the MAC PDU is empty or not, by flushing the buffer does not save any UL grant resources. Thus we don’t think it is an critial issue to be resolved.. |
| Lenovo, Motorola Mobility | Support (Proponent) | The main point of the proposal is to avoid autonomous retransmissions of an empty MAC PDU. Regarding LG’s comment, we would be open to discuss the starting of CGT/CGRT timer. |
| Apple | Support the intention | Autonomous transmission/retransmission does not make sense for an empty MAC PDU. We agree to the problem and the general approach to flush the HARQ process or to stop the CGT/CGRT. We are okay to even add a fix in Rel-16, however, as this was not agreed in R2#115e we can address a solution in Rel-17. If companies are willing to tackle the problem, then solutions can be discussed (there are multiple ways to address this problem). At this stage we do not agree to the changes proposed, but we are fine to address / evaluate the problem / find ways to avoid autonomous transmission for empty MAC PDUs. |
| MediaTek | NSupport | It is unclear how the NW will deal with retransmissions in this case. If the UE has flushed the HARQ process and then receives a retransmission grant from the NW, the UE sends new data to the NW – which can cause issues with soft-combining at the NW side. |
| ZTE | Nsupport in TEI17, discuss in R17 NRIIOT instead. | We can understand the intention here. The padding PDU will occupy another one transmission occasion so that new transmission is blocked due to the previous transmission with UCI is failed. However, the R17 NRIIOT (e.g URLLC/NRIIOT) is still under discussion for rel-17, we understand this issue is somewhat in the scope of the R17 NRIIOT since any possible outcome from this issue may cause some potential impacts on the ongoing discussion. |
| vivo | NSupport | Although the MAC PDU carries pure pending bits, the gNB does not know whether there are data carried by the PUSCH. If the gNB has not decoded the PUSCH correctly, it is natural that the gNB shall schedule the corresponding retransmissions. If the UE flushes the HARQ buffer, the retransmission procedure cannot continue and the gNB will continue schedule further retransmisions, which consumes more resources, and cause negative link adaptation behaviors in gNB. |
| Samsung | NSupport | We believe that retransmission of UCI-only TB can be allowed and there’s nothing broken. Also this issue was discussed during Rel-16. |

QoS Flow to DRB Mapping for MDBV Enforcement

R2-2109851	Adaptation of QoS Flow to DRB Mapping for MDBV Enforcement	Futurewei	discussion	Rel-17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Nokia | unclear | Agree with the issue (challenges with MDBV enforcement) but would prefer controlling the bit rate where the bits are allocated for transmission i.e. during LCP. |
| CATT | NSupport | Dynamic flow to DRB mapping (switching) depending on whether MDBV is met/not met. It is unclear which entity controls the switch. Sounds like a big change for TEI. Could be discussed in R18 (XR)? |
| OPPO | Unclear | We understand the QoS parameters for signled 5QI will only be transmitted to gNB, UE may not be aware of the related QoS parameters inlcuding MDBV and PDB of the signalled 5QI. The proposal shows that SDAP can perform dynamic switching between two configured DRBs for a QoS flow based on the MDBV requirement, we wonder how UE switch the DRB without knowing MDBV. |
| Apple | NSupport | Some tools to utilize shorter latency techniques already exist. For example, the network can reconfigure LCP mapping restrictions via RRC, configure type 2 CGs to be used on demand, rely on DGs. We think that more evaluation would be needed, thus we’d prefer to analyze this in Rel-18. Besides the current 5G QoS framework cannot support mapping a single QoS flow to multiple DRBs, although it could be said that each QoS flow continues to be mapped to a single DRB at a time. |
| MediaTek | NSupport | We do not find a strong enough justification to introduce such a complicated method to enforce MDBV. QoS has always been enforced by MAC in the RAN (with the leaky bucket concept) and introducing further QoS mechanisms in PDCP is undesirable from our point of view. |
| ZTE | NSurppot | Implementation based solution seems sufficient. The reflective QoS can be used to enable the dynamic mapping between DRB and QoS flow |
| Futurewei | Support (Proponent) | @Nokia, Apple, MediaTek – MAC/LCP based schemes to enforce MDBV were discussed in Rel-16, yet there were concerns on their efficay as follows – - Adjusting operating parameters dynamically can be challenging, as LCP task is performed under tight processing timeline; and - These enhancements work only for the special case when a single QoS flow is mapped to a DRB, but not in general scenarios when multiple QoS flows are mapped onto a DRB. @Oppo – the QoS flow to DRB mapping/switching is configured by gNB, and gNB can set parameters according to the MDBV/PDB received in the QoS flow’s QoS profile. UE just needs to follow the configured threshold for switching of QoS flow to DRB mapping. @ZTE – not sure an implementation can rely on the condition that DL user packets happen to arrive at the time that UL QoS flow to DRB mapping should be switched. |
| vivo | Unclear | Perhaps relying on the bit rate control based on NW scheduling. |
| Samsung | Unclear | The proposed dynamic QoS flow to DRP mapping method could already be possible via SDAP implementation. |

Activation/Deactivation of QoS Flow to DRB Mapping for SMBR Enforcement

R2-2109852	Activation/Deactivation of QoS Flow to DRB Mapping for SMBR Enforcement	Futurewei	discussion	Rel-17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Nokia | unclear | If any bit rate enforcement enhancement is needed for SMBR, it should take place in the gNB and impact LCP in the UE. |
| CATT | NSupport | Dynamic "hold" on delivering SDAP PDUs to associated DRB when the aggregated bitrate across all GBR and Non-GBR QoS flows belonging to the PDU sessions associated with a network slice exceeds the UE-Slice-MBR. One first issue is what to do with these PDUs if they are not delivered? Discard? Buffered? Here again SDAP enhancements are likely to be discussed in the context of the R18 XR WI. We prefer to address those at that time. |
| OPPO | NSupport | There was a discussion on whether UE related enhancement is needed for SMBR in the slicing session, and finally, it is ruled out, since most companies thought it is sufficient to rely on gNB implementation. To us, it is not an essential issue and no need for further enhancement. |
| Lenovo, Motorola Mobility | unclear | We tend to agree that there might be limitations of enforcing UL SMBR but think a more detailed study is required. |
| Apple | NSupport | This area was discussed in Rel-17 RAN slicing, it should not come back in TEI17. During the earlier RAN2 R17 discussion, the understanding was that the RAN node is responsible for Slice MBR enforcement. This is also reflected in the running stage-2 CR for the RAN slicing WI. It is our understanding that allowedServingCells can support SMBR enforcement for higher number (more than two) of slices. All the QoS flows in a DRB belong to the same PDU session which can belong to only one slice, we don’t see the difficulty with LCP. |
| MediaTek | NAccept | The justification for such a change is weak. To start with, is the UE expected to be connected to multiple NW slices simultaneously? If so, such a mechanism could end up requiring the UE to now report slice-specific capabilities, which is highly undesirable. |
| ZTE | Nsupport | gNB can handle this. |
| Futurewei | Support (Proponent) | @Nokia, Oppo, Apple, ZTE - RAN2's view is that SMBR enforcement can be provided by configuring different resources per slice. A solution for support of the UL SMBR without different resources will require further study in RAN2. Limitations are identified when UE-Slice-MBR is enforced by resource separation using logical channel mapping restriction - logical channel mapping restriction can’t support resource separation for more than two network slices, and would result in poor radio efficiency due to the loss of multiplexing gain from sharing an UL grant among multiple logical channels. RAN2 conclusion is that RAN can target at enforcing average SMBR in a relatively longer-term, not rigorously in short-term as intended by SA2 specification, if different radio resources can’t be configured/reserved per slice. @MediaTek – UE being simultaneously connected to multiple network slices is not a proposal of this contribution. Instead, it is required/assumed in the network slicing feature from Rel-15. And it is not an AS capability issue. This contribution just discusses the enforcement in RAN the requirement (specified by SA2) that “The RAN shall ensure that the aggregated bitrate across all GBR and Non-GBR QoS Flows belonging to those PDU Sessions is not exceeding the UE-Slice-MBR, while always guaranteeing the GFBR of every GBR QoS Flow of those PDU Sessions as described in clause 5.7.2.5.” (TS 23.501). |
| vivo | unclear | There seems previous discussions on this issue in RAN slicing session, and finally no enhancment was identified as needed at that time. |
| NEC | NSupport | This might be started from some study at some point in time (in future), if really necessary. |
| Samsung | NSupport | UL UE-Slice-MBR enforcement was already discussed under Rel-17 RAN slicing WI and there were views that UL SMBR enforcement can be supported based on NW implementation, with or without separate resource configurations. |

Stopping CGT for ignored or skipped UL grant

R2-2111170	Stopping CGT for ignored or skipped UL grant	LG Electronics Inc.	discussion	TEI17

R2-2111172	CR to 38321 on stopping CGT for ignored or skipped UL grant	LG Electronics Inc.	CR	Rel-17	38.321	16.6.0	1177	-	F	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| LG | Support (Proponent) | Last meeting, companies understanding was that CGT is started at the gNB side when dynamic UL grant is skipped or ignored. However, for CG, neither CGT nore the CGRT starts. Therefore, we believe that even for dynamic UL grant, if it is ignored or skipped, the netowkr would not start CGT and CGRT. With this understanding, starting it only the UE side causes unsynchronized state of CGT and CGRT, which was the concern from the companies. So, rather than jumping into P3 for the suggested change, we would like to hear more on P1 and P2. |
| Nokia | NSupport | This is NBC. When the CG timer is running, it prevents the UE from using the process for CG, but network can still schedule dynamic grant for that process. Stopping the timer would cause problem at NW side since it creates misalignment. |
| CATT | Support | P1: OK P2: Not sure. This assumes NW correctly detects DTX. Anyways, if UE is assumed to keep the timer running, NW should also do so, to keep in sync, irrespective of DTX detection. Nor for P3, the difference between CG and DG is that, for the CG, the CGT is not started upfront by the DCI. And the problem is if the NW mis-detects the DTX, it will schedule a dynamic ReTx, and if, per Proposal 3, the CGT was stopped at the UE, UE may use that HARQ process to send new data (e.g. on a CG). That being said: 1) no data was transmitted in first place, so it is unlikely that now UE suddenly has new data to send, immediately after that 2) even if the UE ignores the dynamic ReTx, it is no big deal as the initial transmission didn't happen hence no data is lost. So we think we can indeed align the CGT behavior of DGs to that of CGs. |
| Huawei, HiSilicon | Support the intention | We acknowledge it is a bug in the spec but the NBC issue raised by Nokia may need to be investigated further. |
| OPPO | Nsupport | This is a legacy issue, we prefer to keep the spec as it is. |
| Apple | NSupport | This issue was discussed at the last RAN2 meeting and companies did not agree. We have a similar view as Nokia. |
| MediaTek | NSupport | This was discussed for Rel-16 and had no support as this was seen as an optimization. The issue being resolved is that a HARQ process is blocked from CG use for a period of time when the UE has no more data to send. But if the UE doesn’t have data to send, it doesn’t really matter if a HARQ process is blocked for a short while. |
| ZTE | Nsupport | Have been discussed before, no consensus is reached |
| vivo | Nsupport | Frankly, the proposed optimization can incur de-synchronization between the operation of the CGT at the NW and UE sides (i.e. the NW cannot preciously know whether UL skipping is performed at the UE side), which generally is not expected. Anyway, the NW can schedule a dynamic UL grant (which can be fully overlapped with a CG PUSCH) for potential data transmission without wasting the already CG PUSCH resource. In this sense, we fail to see the benefit of this optimization. |
| Samsung | NAccept | This Release 17 CR has an backward-compatiblity issue, since Rel-16 gNB does not understand the proposed behaivor. Even if it is agreed, the CR will be more complicated for sure. Technically, we do not agree with the motivation why we have to stop the timer. - If a dynamic grant assignment starts the timer, the running CGT does not harm. Since the HPI is reserved by the dynamic grant, gNB can continue to use this for dynamic grant allocation. - If a retransmission by CS-RNTI starts the timer, it is very clear that gNB understands there was a transmission in the original CG occasion. In this case, UE shall not stop the timer, which will give more confusion at the gNB side. In short, considering gain and pain, this CR is not nessary. |

Added after kick-off

Secondary DRX

Added 2021-11-04 1430 UTC in v04

R2-2111460 	Secondary DRX enhancements	Verizon, Ericsson, Qualcomm Inc, T-Mobile USA Inc	discussion	Rel-17	TEI17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| Nokia | Support BUT… | If there is already an activated cell in the group, there is no need to start the timer (since the gNB can already schedule that cell and by doing so, restart the timer – as for pDRX group). Besides, not sure how fast-activation would impact the proposal since without fast-activation, the inactivity timer could be expired already before the SCell is actually activated thus wouldn’t help. |
| CATT | Partly | P1 would be OK to us. But P2 has been discussed already and we don’t see the need for it. NW can derive/estimate a suitable DRX-InactivityTimer of each DRX group from the single preferredDRX-InactivityTimer. |
| Huawei, HiSilicon | NSupport | This first proposal significantly change the principle that both DRX groups are ensured to operate separately and hence it cannot be pursed in TEI. Actually that is the root why we agreed not to combine cross-carrier scheduling with sec DRX group. So if it is the case, we believe the sensible implementation would be to enable cross-carrier scheduling rather than this optimization with considerable UE complexity. For the second proposal, it has been discussed over several times and we still see no benefit to introduce another UE capability. |
| Apple | Support | We support both proposals, since they can provide the benefit on the service latency reduction and UE power reduction. |
| MediaTek | NSupport | For proposal 1, does it imply that the NW will activate a SCell while it is already in activated state? We think the motivation for proposal 1 is not sufficient. The intention is to reduce data latency. In that case, why not put delay sensitive data in first DRX cell group? If we want to have fast activation/deactivation for a SCell, instead of putting the SCell in long DRX, why not put it in dormant state? For proposal 2, we don’t see too much benefit on this. |
| ZTE | Nsupport | 1：Since the drx-inactivityTimer for FR1 serving cell is still running, we do not see any urgency to start the drx-inactiveTimer for FR2 serving cells. In addition, the intention for introducing the second DRX group is for additional power saving, is there any reason for us to add power non-saving mechanism here？If the delay issue is really concerned , NW can handle this issue for the coming data burst (i.e more DCI scheduling) 2: In our understanding, NW can estimate the ideal configuration of drx-inacitiveTimer for FR2 DRX group based on the one of main DRX group, no more assistance information is needed. |

Early identification of Emergency Call and MPS

Added 2021-11-07 2230 UTC in v23

R2-2111193	Discussion on early identification of Emergency Call	RadiSys, Reliance JIO	discussion	Rel-17

=> revised in R2-2111269

R2-2111269	Discussion on early identification of Emergency Call and MPS	Radisys, Reliance JIO, Verizon, Peraton Labs	discussion	Rel-17

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| R2 Chair |  | The proposals need to be corrected (e.g. to only include RACH indication partitioning), but even if this is done, such proposal may require significant discussion. |

UPIP

Added 2021-11-07 2230 UTC in v23

In the below document, only the first proposal on IP

R2-2109951	User Plane Improvements	Nokia, Nokia Shanghai Bell	discussion	Rel-17	TEI17

Proposal: allow a mode of operation where only a subset of PDCP SDUs is IPed.

| Company | Support / NSupport / NAccept / unclear | Comments |
| --- | --- | --- |
| NEC | unclear | this is interesting proposal and we are open for discussion but firstly this will need SA3 guidance? |

Conclusion

TBD