3GPP TSG RAN WG2 Meeting #116-e	R2-211xxxx

Electronic, 1st - 12th Nov, 2021

Title:	[draft]Reply LS on discovery and relay (re)selection

Response to:	R2-2111236 (S2-2107972)

Release:	Release 17

Work Item:     	NR_SL_Relay-Core

Source:         	CATT [To be RAN2]

To:             	SA2, RAN3

CC:

Contact Person:

Name:	Hao Xu

E-mail Address:	xuhao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for the LS on discovery and relay (re)selection. RAN2 had discussed the LS in RAN2#116-e meeting, and provided RAN2 understanding on these issues as below:

Q1) SA2 has assumed 5G MOCN architecture is supported for 5G ProSe Layer-2 UE-to-Network Relay as described in clause 4.2.7.2 of TS 23.304, and would like to ask RAN2 to confirm this assumption. SA2 has also realized PLMN IDs are required (before Layer-2 link has been established) for the Layer-2 Remote UE to perform PLMN selection as well as Relay selection under 5G MOCN architecture, and would like to know whether PLMN IDs are forwarded by Layer-2 UE-to-Network Relay to Layer-2 Remote UE via the AS layer message.

[Answer]:

For Rel-17 U2N sidelink relay, RAN2 discussed whether RAN sharing can be supported for the NG-RAN node for Rel-17 Layer-2 UE-to-Network Relay. Unfortunately, no consensus was reached. Therefore, RAN2 can’t confirm that 5G MOCN architecture described in clause 4.2.7.2 of TS 23.304 is supported for 5G ProSe Layer-2 UE-to-Network Relay. Whether and how PLMN IDs are forwarded by Layer-2 UE-to-Network Relay to Layer-2 Remote UE depend on whether RAN sharing can be supported. RAN2 will keep on discussing the above issues and inform SA2 once we have reached agreement.

Q2) SA2 has realized TAI is needed for 5G ProSe Layer-2 Remote UE to determine the type of initial access message (Mobility Registration Update or Service Request), and would like to ask whether TAI is forwarded by 5G ProSe Layer-2 UE-to-Network Relay to the 5G ProSe Layer-2 Remote UE via the AS layer message.

[Answer]:

RAN2 confirms that TAI can be forwarded by 5G ProSe Layer-2 UE-to-Network Relay to the 5G ProSe Layer-2 Remote UE via PC5-RRC message after PC5 connection establishment.

Q3) SA2 has discussed the Editor’s Note in TS 23.304 clause 6.4.3.6:

Editor’s Note 	Whether the Layer-2 link modification procedure is also applicable to ProSe Communication via 5G ProSe Layer-2 UE-to-Network Relay requires cooperation with RAN2.

SA2 understands that during the Layer-2 link establishment procedure the 5G ProSe Layer-2 UE-to-Network Relay and 5G ProSe Layer-2 Remote UE do not interact with the QoS Info (the information about PC5 QoS Flows), meaning there is no PC5 QoS Flow established in the PC5 unicast link between Layer-2 UE-to-Network Relay and Layer-2 Remote UE and the QoS handing is therefore setup by RAN. SA2 would like to ask as the Layer-2 link modification procedure is used to add/modify/remove PC5 QoS Flow(s) in the PC5 unicast link, whether this procedure is applicable or not to the Layer-2 UE-to-Network Relay?

[Answer]:

RAN2 confirm during the Layer-2 link establishment procedure the Relay UE and Remote UE do not interact with the PC5 QoS Flows Info. Whether the Layer-2 link modification procedure is used should be decided by SA2.

Q4) Per TS 23.304 clause 6.6.2, NG-RAN is provided with 5G ProSe authorised information indicating whether a UE is authorized to use 5G ProSe Direct Discovery, 5G ProSe Direct Communication, to act as a 5G ProSe Layer-2 UE-to-Network Relay, a 5G ProSe Layer-3 UE-to-Network Relay and a 5G ProSe Layer-2 Remote UE. NG-RAN is not provided with authorisation information for whether a UE is authorised to act as a 5G ProSe Layer-3 Remote UE.

Is the authorisation information for whether a UE can act as a 5G ProSe Layer-3 Remote UE needed by NG-RAN to enable configuring the UE with correct discovery configuration information via dedicated signalling?

[Answer]:

Whether authorization information for L3 remote UE is needed for NG-RAN can be decided by RAN3.

.

2. Actions:

To SA2: RAN2 kindly asks SA2 to take the above information into account for the future work.

To RAN3: RAN2 kindly asks RAN3 to take the above information into account for the future work and feedback on Q4.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2 #116bis-e 		Jan 17 – 25 2022	        Electronic Meeting

TSG-RAN WG2 #117-e			Feb 21 – Mar 3 2022	   Electronic Meeting