3GPP TSG-RAN WG2 Meeting #116 electronic											R2-2111425

Online, 1st – 12th Nov, 2021

Agenda item:       6.2.2

Source: 	Huawei, HiSilicon

Title: 	Summary [AT116-e][707][V2X/SL] Miscellaneous CR on 38.331

Document for:	Discussion and Decision

# Introduction

This document summarizes the offline discussion as:

[AT116-e][707][V2X/SL] Miscellaneous CR on 38.331 (Huawei)

Scope: Discuss CRs in R2-2109596, R2-2109630/R2-2109629, R2-2109806/R2-2109804, R2-2110269, R2-2110611, R2-2110795, and R2-2110831, and merge the agreeable changes.

Intended outcome: 38.331 CR in R2-2111424 and discussion summary in R2-2111425 (if need)

Deadline: 11/9, 10:00am UTC

| Company | Name | E-mail |
| --- | --- | --- |
| Huawei, HiSilicon | Tao Cai | tao.cai@huawei.com |

# Correction CRs based on LS in R1-2108393

Table 1: Correction CRs based on LS in R1-2108393

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2109596 | Miscelleneous CR on 38.331 | Huawei, HiSilicon | - “In the LS R1-2108393 from RAN1, RAN2 is requested to capture following RAN1 agreement on sidelink resource allocation mode 2 that A UE is expected to be (pre-)configured with a set sl-ResourceReservePeriod containing value of 0 ms Accordingly, in field description of sl-ResourceReservePeriodList in IE SL-ResourcePool, it should be clarified that in case Mode 2 is configured, at least value 0ms should be included in the list. “ sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. In case Mode 2 is configured, at least value 0ms should be included in the list. - Fix the editorial errors. | For the first change of Rapp’s Misc CR and the other four CRs in this group, all are based on RAN1 LS R1-2108393. The discussion would be to find the optimal wording for a straightforward clarification. |
| R2-2109630/ R2-2109629 | CR to 38.331 on ResourceReservationPeriodList | Qualcomm Finland RFFE Oy | sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. The value ms0 is always configured. |  |
| R2-2110611 | Corrections on RRC parameter sl-ResourceReservePeriodList | CATT | sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool, value ms0 shall be included. |  |
| R2-2110795 | Inclusion of 0 ms resource reservation period in sl-ResourceReservePeriodList | MediaTek Inc. | sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. Network includes a list entry with the value ms0 for each resource pool. |  |
| R2-2110831 | Correction on TS 38.331 from the latest RAN1 decision | ZTE Corporation, Sanechips | sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. A UE is expected to be (pre-)configured with a set sl-ResourceReservePeriod containing value of 0 ms. |  |

As the first change of R2-2109596 and changes in R2-2109630, R2-2110611, R2-2110795 and R2-2110831 in Table 1 are on the same issue by RAN 1 LS in R1-2108393, Rapp suggest we can discuss on the need of change and further discuss on optimal working for the change:

Q1: Would your company agree to have the change based on LS in R1-2108393 and which wording would your company prefer for the change?

Option 1: Up to 16 values can be configured per resource pool, and value ms0 shall be always configured.

Option 2: Up to 16 values can be configured per resource pool. The value ms0 is always configured.

Option 3: Up to 16 values can be configured per resource pool, value ms0 shall be included.

Option 4: Up to 16 values can be configured per resource pool. Network includes a list entry with the value ms0 for each resource pool.

Option 5: Up to 16 values can be configured per resource pool. A UE is expected to be (pre-)configured with a set sl-ResourceReservePeriod containing value of 0 ms.

| Company | Agree/disagree on having the change | Wording option | Further comments |
| --- | --- | --- | --- |

The other changes in R2-2109596, miscellaneous CR on 38.331, are all changes on font style or correcting typos.

Q2: Would your company disagree on any editorial changes in R2-2109596, miscellaneous CR on 38.331

| Company | Which editorial change | Further comments |
| --- | --- | --- |

# Other correction CRs

Table 2: Other correction CRs

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2109806/ R2-2109804 | Correction of IE sl-HARQ-FeedbackEnabled | Nokia, Nokia Shanghai Bell | sl-HARQ-FeedbackEnabled This field is mandatory present. It indicates the HARQ feedback enabled/disabled restriction in LCP for this sidelink logical channel. If set to enabled, the sidelink logical channel will be multiplexed only with a logical channel which enabling the HARQ feedback. If set to disabled, the sidelink logical channel cannot be multiplexed with a logical channel which enabling the HARQ feedback. Corresponds to 'sl-HARQ-FeedbackEnabled' in TS 38.321 [3]. If this field of at least one sidelink logical channel for the UE is set to enabled, sl-PSFCH-Config should be mandatory present in at least one of the SL-ResourcePool. | The change itself as one NBC approach is sensible/agreeable, once RAN2 acknowledges that there is indeed discrepancy between RRC and MAC spec regarding the configuration of “sl-HARQ-FeedbackEnabled”. |
| R2-2110269 | Correction on SL RLC parameter configuration | vivo | SL-RLC-Config field descriptions. sl-MaxRetxThreshold Parameter value of maxRetxThreshold for RLC AM for NR sidelink communications, see TS 38.322 [4]. Value t1 corresponds to 1 retransmission, value t2 corresponds to 2 retransmissions and so on.. sl-PollByte Parameter value of pollByte for RLC AM for NR sidelink communications, see TS 38.322 [4]. Value kB25 corresponds to 25 kBytes, value kB50 corresponds to 50 kBytes and so on. infinity corresponds to an infinite amount of kBytes.. sl-PollPDU Parameter value of pollPDU for RLC AM for NR sidelink communications, seeTS 38.322 [4]. Value p4 corresponds to 4 PDUs, value p8 corresponds to 8 PDUs and so on. infinity corresponds to an infinite number of PDUs.. sl-SN-FieldLength This field indicates the RLC SN field size for NR sidelink communication, see TS 38.322 [4]. For groupcast and broadcast, only value size6 (6 bits) is configured for the field sl-SN-FieldLengthUM.. sl-T-PollRetransmit Timer value of t-PollRetransmit for RLC AM for NR sidelink communications, see TS 38.322 [4], in milliseconds. Value ms5 means 5 ms, value ms10 means 10 ms and so on.. #### 9.1.1.4 SCCH configuration Parameters that are specified for unicast of NR sidelink communication, which is used for the sidelink signalling radio bearer of PC5-RRC message. The SL-SRB using this SCCH configuration is named as SL-SRB3. Name; Value; Semantics description; Ver. PDCP configuration. >t-Reordering; Undefined; Selected by the receiving UE, up to UE implementation. >pdcp-SN-Size; 12. RLC configuration; AM RLC. >sn-FieldLength; 12. >t-Reassembly; Undefined; Selected by the receiving UE, up to UE implementation. >t-PollRetransmit; Undefined; Selected by the transmitting UE, up to UE implementation. >pollPDU; Undefined; Selected by the transmitting UE, up to UE implementation. >pollByte; Undefined; Selected by the transmitting UE, up to UE implementation. >maxRetxThreshold; Undefined; Selected by the transmitting UE, up to UE implementation. >t-StatusProhibit; Undefined; Selected by the receiving UE, up to UE implementation. >logicalChannelIdentity; 3. MAC configuration. >priority; 1. >prioritisedBitRate; infinity. >logicalChannelGroup; 0. >schedulingRequestId; 0; The scheduling request configuration with this value is applicable for this SCCH if configured by the network.. | The change on the description of “SL-RLC-Config”, following the approach for the similar issue with "PDCP spec", is reasonable, also it is straightforward to add the missing parameters, once RAN2 confirms the changes are needed. |

Q3: On above CR in R2-2109806 in Table 2, would your company agree that there is discrepancy between RRC and MAC spec regarding the configuration of “sl-HARQ-FeedbackEnabled”, and the change is needed as proposed?

| Company | Agree/disagree on having the change | Further comments (e.g. on the change wording) |
| --- | --- | --- |

Q4: On above CR in R2-2110269 in Table 2, would your company agree that the first group of changes on the description of “SL-RLC-Config” is needed and/or the missing parameters shall be added (second group of changes)?

| Company | Agree/disagree on the first group of changes | Agree/disagree on the second group of changes | Further comments(e.g. on the change wording) |
| --- | --- | --- | --- |

# Conclusions