3GPP TSG-RAN WG2 #116-e	DocNumber

Electronic meeting, 1th – 12th November 2021

Agenda Item:	8.13.2.1

Source:	Ericsson

Title:	[AT116-e][850][SONMDT] Handover related SON aspects again (Ericsson)

Document for:	Discussion, Decision

# Introduction

This contribution addresses the following offline discussion:

CB on Thursday:

- [AT115e][850][SON/MDT] Handover related SON aspects again (Ericsson)

Scope: focus on proposals 5-14 in R2-2110889.

Intended outcome: Report

Deadline: 05:00 UTC, Wednesday November 10th

To aid better communication between the respective delegates handling this topic from different companies, it is requested to fill-in the contact information.

Contact Information

| Company | Name | Email |
| --- | --- | --- |

# Discussion

## CHO related

Proposal 5	RAN2 to further discuss the need to include in the RLF-Report an indicator indicating whether the last executed HO before the RLF in the target cell was a CHO HO.

There is one proposal on discussion related to including the HO type of the last executed HO. To help the discussions, the rapporteur has depicted the scenarios in the Figure 1. Scenario depicted in (a) shows a normal HO scenario and the scenario depicted in (b) shows a CHO scenario. In both scenarios, the content of the previousPCell and timeConnFailure is the same as shown in the figure

Cell-B

Cell-B

Cell-A

Cell-A

- Normal HO scenario

(b) CHO scenario

RLF

RLF

CHO from cell-A to cell-B

Normal HO from cell-A to cell-B

previousPCell  CellA

timeConnFailure  2 seconds

previousPCell  CellA

timeConnFailure  2 seconds

Figure 1: Scenarios involving RLF followed by (a) normal HO (b) conditional HO

Based on the above, the proponents (in the #899 email discussion) of including the HO type indicator mentions that there is a need to differentiate whether the HO from cell-A to cell-B was a normal HO or a CHO so that the corresponding HO parameters can be tuned accordingly. The proponents mentioned that the CHO parameters are different from the normal HO parameters.

Thus, rapporteur would like to ask the following question.

Question-1: Do you agree to include an indicator in the RLF report indicating whether the last executed HO before the RLF in the target cell was a CHO HO?

| Company | Agree/Disagree | Comments |  |
| --- | --- | --- | --- |

Rapporteur summary:

To be added later

## DAPS related

Proposal 6	RAN2 to further discuss the need to include in the RLF-Report an indicator indicating that the last executed HO before the RLF in the target cell was a DAPS HO.

The scenario is the same as depicted in Figure 1 but instead of CHO, the DAPS HO is used.

Thus, rapporteur would like to ask the following question.

Question-2: Do you agree to include an indicator in the RLF report indicating whether the last executed HO before the RLF in the target cell was a DAPS HO?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

## SHR related

There are several open issues related to SHR.

One such open issue is related to whether the source cell or the target cell configures the T304 related threshold value.

Proposal 7	The value of the T304 threshold to be provided in the SHR configuration is configured by the target cell.

The following technical reasonings are mentioned in the respones of the #899 email discussion:

- Agreeing companies

  - Source cell doesn’t know the value of T304. It is arbitrary for the source cell to configure the threshold without knowing the value of T304.

  - If the source cell is to decide on the T304 percentage value, it may choose a fixed value because it has no idea about T304 value.

  - Target node can also use the SHR to optimize its HO related parameters like T304 value that it configures in the future.

- Disagreeing companies

  - T304 threshold for the SHR configuration is not necessarily related to the T304 absolute value set by the target cell.

  - SHR is used by the source node to optimize the relevant parameters, such as the threshold to trigger HO.

Based on this, rapporteur would like to ask the following question.

Question-3: Do you agree that the value of the T304 threshold to be provided in the SHR configuration is configured by the target cell?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

Another topic discussed in #899 email discussion is related to if and when the UE includes the RA-InformationCommon in the SHR.

Proposal 8	RAN2 to discuss when the RA-InformationCommon should be included in the SHR:

a.  Only in case the SHR is generated due to T304 above the threshold (8/16)

b.  It should never be included (8/16)

The following technical reasonings are mentioned in the respones of the #899 email discussion:

- Companies supporting option-A

  - RA-InformationCommon corresponding to SHR could be replaced or deleted from RA report.

  - There is currently no way to link a specific RA report in the RA report list with the SHR

  - There is no indicator or timestamp to associate the SHR to a specific entry in the RA-Report

- Companies supporting option-B

  - Already part of RA-report. No need to duplicate it..

Based on this, rapporteur would like to ask the following question.

Question-4: Which is the following option is preferred for the inclusion of RA-InformationCommon in the SHR?

Option-1: RA-InformationCommon is included in SHR when T304 is above the threshold

Option-2: RA-InformationCommon is not included in SHR

| Company | Option-1 / Option-2 | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

Another topic discussed in #899 email discussion is related to the impact of SHR and RLF report being generated for the same HO event.

Proposal 9	RAN2 believes that it is not a problem if both the SHR and RLF-Report are generated after the same HO, and it is not a problem if the network fetches them separately.

The following technical reasonings are mentioned in the respones of the #899 email discussion:

- Companies who believes there is an issue

  - Since the two reports were caused by a single event, it may be beneficial to correlate them for further parameters analysis

  - The UE will report to the network both the SHR and the RLF-Report for the same HO event.

  - It is not clear how the network implementation can fix this issue, given that there will not be any indicator or timestamp linking the  RLF-Report to the SHR (and viceversa).

  - In DAPS HO case, where UE experience RLF is source during HO failure and then UE successfully HO to target the UE generates both SHR and RLF report.

- Companies who believe there is no issue

  - The two reports have different optimization objectives

  - This is related to network implementation issue

  - The network needs to collect enough SON reports and then can do a full anaysis on the issues.

Based on this, rapporteur would like to ask the following question.

Question-5: Do you agree that it is not a problem if both the SHR and RLF-Report are generated after the same HO, and it is not a problem if the network fetches them separately?

| Company | Yes/No | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

Another topic discussed in #899 email discussion is related to the scenario of including information related to the short-stay/ping-pong in the SHR.

Proposal 10	The SHR does not include information on whether the UE is handed-over to another cell early after the successful HO.

Almost all companies agreed with this proposal in the previous email discussion.

Based on this, rapporteur would like to ask the following question.

Question-6: Do you agree that the SHR does not include information on whether the UE is handed-over to another cell early after the successful HO?

| Company | Yes/No | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

Another topic discussed in #899 email discussion is related to UP measurements in the SHR.

Proposal 11	Related to the UP measurements to be included in the SHR, the UE should include at least the following:

a.	User plane interruption at handover, as evaluated at PDPC layer without considering duplicates

Proposal 12	The user plane interruption at handover, as evaluated at PDPC layer without considering duplicates is defined as follows: “Time from the last packet received from the source and the first non-duplicate packet received from the target, measured at the time of reception of the first non-duplicate packet from the target cell.”

Almost all companies agreed with this proposal-11 and proposal-12 in the previous email discussion.

Based on this, rapporteur would like to ask the following questions.

Question-7: Do you agree that the UE should include UP interruption time at HO as evaluated at PDCP layer without considering duplicates in the SHR?

| Company | Yes/No | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

Question-8: Do you agree with the following definition for UP interruption time at HO as evaluated at PDCP layer without considering duplicates?

“Time from the last packet received from the source and the first non-duplicate packet received from the target, measured at the time of reception of the first non-duplicate packet from the target cell.”

| Company | Yes/No | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

Another topic discussed in #899 email discussion is related to the additional scenarios for reporting SHR.

Proposal 13	RAN2 to discuss the following issues related to the SHR:

a.	Support of inter-RAT SHR reporting

b.	Including the time between the source RLF and DAPS HO completion

c.	How to discard the stored SHR at T304 expiry

d.	How to indicate SHR availability in case of RRCReconfigurationComplete message has already been generated

Rapporteur requests companies to be pragmatic here regarding what is essential to be included in Rel-17 considering the remaining time.

Question-9: Which of the following topics needs to be addressed in Rel-17?

a.	Support of inter-RAT SHR reporting

b.	Including the time between the source RLF and DAPS HO completion

c.	How to discard the stored SHR at T304 expiry

d.	How to indicate SHR availability in case of RRCReconfigurationComplete message has already been generated

e.	None

| Company | a, b, c, d, e | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

In the #899 email discussion, some companies indicated additional SHR triggering conditions.

Proposal 14	RAN2 to discuss the need for the following SHR triggering conditions:

a.	T310/T312 in target cell is started after a short time of successful HO

b.	The number of preamble attempt in target cell is greater than one threshold

c.	If the UP interruption time is above a certain threshold

d.	Configured CFRA RACH resource not used and the UE is forced to use the CBRA for HO.

Rapporteur requests companies to be pragmatic here regarding what is essential to be included in Rel-17 considering the remaining time.

Question-10: Which of the following triggering condition for SHR do you support?

a.	T310/T312 in target cell is started after a short time of successful HO

b.	The number of preamble attempt in target cell is greater than one threshold

c.	If the UP interruption time is above a certain threshold

d.	Configured CFRA RACH resource not used and the UE is forced to use the CBRA for HO.

e.	None

| Company | a, b, c, d, e | Comments |
| --- | --- | --- |

Rapporteur summary:

To be added later

# Conclusion

To be added later.