3GPP TSG RAN WG2#116-e	R2-2109314

e-Meeting, 1st - 12th November, 2021

3GPP TSG RAN WG1#106-e		R1-2108436

e-Meeting, 16th – 27th August, 2021

Title:	LS to RAN2 on default value for rb-Offset

Release:	Rel-16

Work Item:	NR_unlic

Source:	TSG RAN WG1

To:	TSG RAN WG2

Contact person:	Stephen Grant

stephen.grant@ericsson.com

# 1	Overall description

In RAN2 specification 38.331 within the ControlResourceSet IE, the parameter rb-Offset introduced in Rel-16 is defined. The parameter has the following field description which provides a default value in case the parameter is not configured:

rb-Offset Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1). When the field is absent, the UE applies the value 0.

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap

-	if a CORESET is not associated with any search space set configured with freqMonitorLocations, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset.

RAN1 has identified an incompatibility between the default value definition in 38.331 and RAN1 specification 38.213. The issue can be seen from the following text extract from Section 10.1:

In this text extract, the UE behavior is specified both for the case when the parameter rb-Offset is not provided (cyan highlighted text) and when the parameter rb-Offset is provided (yellow highlighted text). The issue is that for a Rel-16 UE not configured with a search space set configured with the Rel-16 parameter freqMonitorLocations, the cyan text will not be executed since the physical layer is always provided a value for rb-Offset due to the default value definition in 38.331.

RAN1 has discussed both RAN1 and RAN2-centric solutions, and has agreed that it would be more straightforward for RAN2 to make a change to 38.331 to resolve the incompatibility. The RAN2-centric solution that RAN1 discussed is the following; however, RAN1 acknowledges that the final decision is up to RAN2.

rb-Offset Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1). When the field is absent, the UE applies the value 0.

# 2	Actions

To TSG RAN WG2

ACTION: 	RAN1 respectfully requests RAN2 to implement a suitable correction to 38.331 to remove incompatibility between RAN1 and RAN2 specs with respect to the Rel-16 parameter rb-Offset.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #106bis-e	11 – 19 October 2021 		Online

TSG RAN WG1 Meeting #107-e	11 – 19 November 2021	Online