3GPP TSG RAN WG2#116bis-e	R2-2200093

e-Meeting, 17th - 25th January, 2022

3GPP TSG RAN WG1 #107-e	R1-2112971

e-Meeting, November 11th – 19th, 2021

Title:	LS on channel quality reporting for NB-IoT

Response to:

Release:	Rel-17

Work Item:	NB_IOTenh4_LTE_eMTC6-Core

Source:	RAN1

To:	RAN2, RAN4

CC:

# Contact Person:

# Name: 	Yubo Yang

E-mail Address: 	yangyubo1@huawei.com

# 1. Overall Description:

RAN1 has discussed the channel quality reporting for 16-QAM and has achieved the following agreements:

Agreement If 16-QAM is configured for NPDSCH, the channel quality report for 16-QAM is based on NPDSCH transport block that achieves an error probability not exceeding 10% BLER. Agreement - When configured with downlink 16-QAM, the channel quality can be reported in MAC CE. - FFS on support in Msg3 in connected mode Conclusion The channel quality report is not supported in Msg3 in connected mode in Rel-17. Conclusion There’s no consensus on the introduction of CSI reference resource in NB-IoT in Rel-17, whether/how to introduce CSI reference resource is up to RAN4.

RAN1 has also endorsed the following table for channel quality reporting in DL for 16-QAM NB-IoT UEs:

| Reported value | NPDCCH repetition level | NPDSCH transport block error probability not exceeding 0.1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Modulation | Code rate x 1024 | Repetition | Efficiency |
| noMeasurement | No measurement reporting | Out of range |  |  |  |
| candidateRep-A | 1 | QPSK | 221 | 1 | 0.4316 |
| candidateRep-B | 2 | QPSK | 280 | 1 | 0.2737 |
| candidateRep-C | 4 | QPSK | 81 | 1 | 0.1579 |
| candidateRep-D | 8 | QPSK | 81 | 2 | 0.0789 |
| candidateRep-E | 16 | QPSK | 81 | 4 | 0.0395 |
| candidateRep-F | 32 | QPSK | 81 | 8 | 0.0198 |
| candidateRep-G | 1 | QPSK | 336.8 | 1 | 0.6579 |
| candidateRep-H | 1 | QPSK | 453.6 | 1 | 0.8860 |
| candidateRep-I | 1 | QPSK | 579.4 | 1 | 1.1316 |
| candidateRep-J | 1 | QPSK | 759 | 1 | 1.4825 |
| candidateRep-K | 1 | 16QAM | 487.3 | 1 | 1.9035 |
| candidateRep-L | 1 | 16QAM | 541.2 | 1 | 2.1140 |
| candidateRep-M | 1 | 16QAM | 658 | 1 | 2.5702 |
| candidateRep-N | 1 | 16QAM | 783.7 | 1 | 3.0614 |
| candidateRep-O | 1 | 16QAM | 837.6 | 1 | 3.2719 |

It is RAN1’s assumption that the channel quality reporting table will be captured in spec by RAN4, and the corresponding signalling will be captured in spec by RAN2.

# 2. Actions:

To RAN2, RAN4

ACTION:

RAN1 would like to respectfully request RAN2 and RAN4 to take the above information into account in the future work.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107b-e	17th Jan. – 25th Jan. 2022	e-Meeting

TSG-RAN WG1 Meeting #108-e	21st Feb. – 3rd Mar. 2022	e-Meeting