3GPP TSG RAN WG2#116bis-e	R2-2200134

e-Meeting, 17th - 25th January, 2022

3GPP TSG-RAN WG4 Meeting # 101-e	R4-2120652

# Electronic Meeting, November 01-12, 2021

Title:	LS UE capability for supporting single DCI transmission schemes for multi-TRP

Release:	Rel-16

Work Item:	NR_eMIMO-Perf

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:	Manasa Raghavan

E-mail Address:	manasa.raghavan@apple.com

# 1. Overall Description

In Rel-16 RAN4 has defined performance requirements for multi-DCI and single DCI transmission schemes for multi-TRxP transmission in FR1. These requirements are applicable to UEs supporting the following UE capability.

| Test Case/ Transmission scheme | UE Capability |
| --- | --- |
| Multi-DCI non-overlapping | multiDCI-MultiTRP-r16, maxSimultaneousResourceSetsPerCC ≥ 2 |
| Single DCI SDM | singleDCI-SDM-scheme-r16, maxSimultaneousResourceSetsPerCC ≥ 2 |
| Single DCI Inter-Slot TDM | supportInter-slotTDM-r16, maxNumberTCI-states-r16=2, maxSimultaneousResourceSetsPerCC ≥ 2 |
| Single DCI FDM Scheme A | supportFDM-SchemeA-r16, maxSimultaneousResourceSetsPerCC ≥ 2 |

The requirements are defined with 2 active TCI states. The multi-DCI transmission scheme requirements are based on UE capability multiDCI-MultiTRP-r16 also includes maxNumberCORESETPerPoolIndex-r16, which is sufficient to indicate support of 2 active TCI states. For single DCI Inter-Slot TDM the capability of maxNumberTCI-states-r16=2 in supportInter-slotTDM-r16 covers support for 2 active TCI states. For single DCI FDM and SDM transmission schemes, the UE capability doesn’t include any additional UE capability for support of more than 1 active TCI state or any pre-requisite to support maxNumberActiveTCI-PerBWP other than n1.

RAN4 requests RAN1 to clarify if explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes or the UE capability of supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 is sufficient.

# 2. Actions:

To: RAN1

ACTION: RAN4 respectfully asks RAN1 to provide clarification to the above question.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG4   Meeting #101-bis-e	Jan. 17 – 25, 2022	Online

TSG RAN WG1   Meeting #102-e	Feb. 21– Mar. 03, 2022	Online