3GPP TSG-RAN WG2 Meeting #116bis-e                                                           R2-2200324

Online, January 17th – 25th, 2022

Source:	CATT

Title:	Leftovers of CP-UP Separation

Agenda Item:	8.4.2.2

Document for:	Discussion and Decision

Introduction

The agreements on CP-UP separation below had been captured in the running RRC CR [1]. This contribution will address the two FFSs.

The configuration of F1-C traffic on the indication of the the leg(s) used for transferring the F1-C traffic is configured to IAB-MT by a new field, e.g., f1c-TransferPath-r17 ENUMERATED {MCG, SCG, both}. As long as the BH RLC CH for F1-C on the indicated Cell Group is configured (the CG is indicated by the field f1c-TransferPath-r17), IAB node can be aware of whether to use F1-C transferring over BH or F1-C transferring over RRC, i.e. F1-C-over-BAP is selected as long as BH RLC CH for F1-C on the indicated CG is configured. It is not necessary for IAB-node to be aware whether the gNB allows “F1 over BAP” or only allows “F1-C over RRC” during cell (re)selection, in case the gNB broadcasts iab-Support. ONLY SRB2 is used for F1-C transport in CP/UP-separation scenario 1. ONLY split SRB2 is used for F1-C transport in CP/UP-separation scenario 2 FFS if For IAB-MT’s RRC message that carries F1-C/F1-C related traffic, the IAB-MT use split SRB2 via SCG in scenario 2 if f1c-TransferPath-r17 indicates ‘SCG’ or ‘both’ regardless of the primaryPath configuration. FFS on how to capture this in specs. FFS if In case the split SRB2 RRC message contains both F1-C traffic and other information unrelated to IAB, the IAB-MT follows the configuration of F1-C transfer path (if configured) to transmit this RRC message.

Discussion

The two scenarios of CP-UP separation are listed for better understanding.

- Scenario 1: F1-C uses NR access link via M-NG-RAN node (non-donor node) + F1-U uses backhaul link via S-NG-RAN node (donor node) - Scenario 2: F1-U uses backhaul link via M-NG-RAN node (donor node) + F1-C uses NR access link via S-NG-RAN node (non-donor node)

For the first FFS:

FFS if For IAB-MT’s RRC message that carries F1-C/F1-C related traffic, the IAB-MT use split SRB2 via SCG in scenario 2 if f1c-TransferPath-r17 indicates ‘SCG’ or ‘both’ regardless of the primaryPath configuration. FFS on how to capture this in specs.

We agreed “f1c-TransferPath-r17  ENUMERATED {MCG, SCG, both}” is used to indicate the path for transferring F1-C packets from NR-DC IAB-MT to the IAB-donor-CU. For split SRB, primaryPath must be the cell group ID corresponding to MCG. Referring to section 5.7.3b.4	“Actions related to transmission of MCGFailureInformation message” in RRC specification [2], it states that:

5.7.3b.4 Actions related to transmission of MCGFailureInformation message The UE shall set the contents of the MCGFailureInformation message as follows: … 1> if SRB1 is configured as split SRB and pdcp-Duplication is not configured: 2> if the primaryPath for the PDCP entity of SRB1 refers to the MCG: 3> set the primaryPath to refer to the SCG.

Similarly, if f1c-TransferPath-r17 indicates ‘SCG’ or ‘both’ and BH RLC CH for F1-C on SCG is not configured, IAB-MT can set the primaryPath to refer to the SCG, and transfer F1-C packets in RRC message via ULInformationTransfer message.

Proposal 1: When the F1-C transfer path is SCG and BH RLC CH for F1-C on SCG is not configured, IAB-MT should set the primaryPath to refer to the SCG for F1-C packets transmission.

For another detail in the running RRC CR [1]:

“In EN-DC if the f1c-TransferPath is not configured, the IAB node uses the NR leg as the default one. FFS the need of specifying a default transfer path if the f1c-TransferPathNRDC is not configured.”

If f1c-TransferPath-r17 is not configured and IAB-MT needs to transfer F1-C packets in RRC message, the default transfer path should be MCG.

Proposal 2: If f1c-TransferPath-r17 is not configured, the default transfer path should be MCG.

For the 2nd FFS:

FFS if In case the split SRB2 RRC message contains both F1-C traffic and other information unrelated to IAB, the IAB-MT follows the configuration of F1-C transfer path (if configured) to transmit this RRC message.

Basically, if both F1-C packet and other information unrelated to IAB need to be transmitted, IAB-MT should transmit them in two RRC PDUs. Even if the two types of information are contained in one split SRB RRC message, IAB-MT should follow the configuration of F1-C transfer path for F1-C packet transmission. There is no specification impact.

Proposal 3: Confirm that as long as the split SRB2 RRC message contains F1-C packet, IAB-MT should follow the configuration of F1-C transfer path.

Conclusion

According to the analysis in section 2, we reached below proposals.

Proposal 1: When the F1-C transfer path is SCG and BH RLC CH for F1-C on SCG is not configured, IAB-MT should set the primaryPath to refer to the SCG for F1-C packets transmission.

Proposal 2: If f1c-TransferPath-r17 is not configured, the default transfer path should be MCG.

Proposal 3: Confirm that as long as the split SRB2 RRC message contains F1-C packet, IAB-MT should follow the configuration of F1-C transfer path.

Reference

- R2-2111604, Running CR to 38.331 on NR IAB enhancements

- 3GPP TS38.331, Radio Resource Control (RRC) protocol specification, V16.7.0