3GPP TSG RAN WG2#116bis-e                                                               R2-2200396

Electronic meeting, Jan 17 – 25, 2021

Source:	CATT

Title:	The Corrections on Immediate MDT Enhancements

Agenda Item:	8.13.3.1

Document for:	Discussion and Decision

Introduction

In this contribution, we’d like to discuss the correction immediate MDT in running CR.

Discussion

The correction on immediate MDT of TS 37.320

In Rel-16, the immediate MDT is supported in EN-DC scenario, therefore, the related configuration is added in TS37.320. In RAN2 #113bis-e meeting, the following agreement is made for the immediate MDT:

All the immediate MDT configurations and reporting in EN-DC scenario (i.e. section 5.4.1.3 Immediate MDT for MR-DC in TS 37.320) are also applicable for (NG)EN-DC, NE-DC and NR-DC.

According to the above agreement, the current running CR [1] of “Immediate MDT is supported for EN-DC scenario” should be corrected to “Immediate MDT is supported for all MR-DC scenarios”.

Proposal 1: Change“Immediate MDT is supported for EN-DC scenario” to “Immediate MDT is supported for all MR-DC scenarios” in section 5.4.1.3 Immediate MDT for MR-DC in TS 37.320.

The correction on immediate MDT of TS 38.331

In running CR discussion [2], a FFS is added for the immediate MDT:

FFS: whether to change the existing definition (as below) - to configure at most one measurement identity per CG using a reporting configuration with the ul-DelayValueConfig;

If UE is configured with MN terminated SCG bearer and SN terminated SCG bearer, MN and SN can both configure the D1 measurement toward SCG. Meanwhile, for D1 measurement in the case of split bearer, the following agreements are achieved:

1 For non-duplication and duplication case, a single D1 is calculated. 2 The following method is used for configuring D1 in case of split bearer: only one node can configures D1 to UE, and UE reports D1 to corresponding node where configuration is received;

According to the above agreements, a single D1 measurement is sufficient for the split bearer, the configured D1 is not only related to MCG but also related to SCG. Therefore, the restriction of at most one measurement identity “per CG” for configuring D1 measurement is not correct, the corresponding restriction should be removed in current running CR.

Proposal 2: Remove the existing restriction for D1 configuration “to configure at most one measurement identity per CG using a reporting configuration with the ul-DelayValueConfig”.

Conclusion

According to the discussion in section 2, we propose:

Proposal 1: Change“Immediate MDT is supported for EN-DC scenario” to “Immediate MDT is supported for all MR-DC scenarios” in section 5.4.1.3 Immediate MDT for MR-DC in TS 37.320.

Proposal 2: Remove the existing restriction for D1 configuration “to configure at most one measurement identity per CG using a reporting configuration with the ul-DelayValueConfig”.

Reference

[1] 37.320 Running CR for R17 MDT in NR and E-UTRAN

[2] Running 38.331 for introducing R17 MDT