3GPP TSG-RAN2 Meeting #116 bis electronic	R2-2200734

Online, 17th Jan 2021 – 25th Jan 2021

Agenda item:	8.6.5

Source:	Qualcomm Incorporated

Title:	Remaining issues on CG based SDT

WID/SID:	NR_SmallData_INACTIVE-Core – Release 17

Document for:	Discussion and Decision

# Introduction

RAN2 #115e meeting has made several agreements related to CG based SDT.

Agreements 1. If none of the SSBs’ RSRP is above the RSRP threshold of CG-SDT criteria in the type selection phase, UE should select RA-SDT if RA-SDT criteria is met 2. MAC PDU rebuilding is not required (unless we find a case that is needed) 3. During subsequent CG transmission phase (i.e. after the UE has received response from NW) UE can initiate at least legacy RACH procedure (e.g. trigger due to no UL resources). No MAC PDU rebuilding is required. FFS if the RA-SDT RA resources can be used for subsequent data. a. At least the following conditions are agreed: (1) no qualified SSB when the evaluation is performed; (2) when TA is invalid; (3) when SR is triggered due to lack of UL resource 4. UE should release CG-SDT resource (if stored) when UE initiates RRC resume procedure from another cell which is different from the cell in which the RRCRelease is received. 5. The C-RNTI previously configured in RRC_CONNECTED state is used for UE to monitor PDCCH in CG-SDT. 6. CS-RNTI based dynamic retransmission mechanism can be reused for CG-SDT. FFS whether CS-RNTI is the same one as the one previously configured in RRC_CONNECTED or a new CS-RNTI one is provided to the UE 7. During the subsequent new CG transmission phase, for the purpose of CG resource selection, UE re-evaluates the SSB for subsequent CG transmission. FFS what happens if no SSBs are valid or if no sample is available 8. From RAN2 perspective, at least the following parameters should be included in the CG-SDT configuration. FFS whether these parameters are common for multiple CG-SDT configurations or per CG-SDT configuration. - The new TA timer in RRC_INACTIVE; - The RSRP change threshold for TA validation mechanism in SDT (details dependent on RAN1); - The SSB RSRP threshold for beam selection (i.e. UE selects the beam and associated CG resource for data transmission).

RAN2 #116e further made the following agreements.

Agreements - The Rel-16 CG configuration mechanism in licensed band is reused the baseline for CG-SDT. - At least for initial transmission we will have a mechanism to allow the UE to transmit the message again. FFS for retransmission for subsequent. - The UE uses/selects the same HARQ process for retransmission - The “CG-SDT timer” starts at the first “valid” PDCCH occasion from the end of the CG-SDT PUSCH transmission. The first “valid” PDCCH occasion is defined in RAN1 - The “CG-SDT timer” can be started/restarted during for initial and subsequent transmissions - The UE restarts the “CG-SDT timer” at least: - upon the PUSCH retransmission indicated by the CS-RNTI PDCCH - after each CG-SDT transmission 7. The “CG-SDT timer” stops at least: - When the UE receives RRC feedback messages (e.g. RRCResume, RRCSetup, RRCRelease and RRCReject) 8. The Rel-16 calculation on the HARQ process ID of the CG type-1 for licensed band is reused as the baseline for CG-SDT 9. The UE is allowed to initiate subsequent UL data transmission only after the reception of confirmation of initial transmission from the gNB 10. The UE can use multiple CG resources for the HARQ initial transmission as Rel-16 in the subsequent CG transmission phase 11. The following CG-SDT configurations are per UE: - The new TA timer in RRC_INACTIVE - The RSRP change threshold for TA validation mechanism in SDT - The SSB RSRP threshold for beam selection

In this paper, we would like to discuss and provide our views on more details about the CG-SDT.

# Discussion

Monitoring window for CG-SDT

The RAN2 #113bis-e meeting made the following agreement, i.e., UE should start a window after CG/DG transmission for CG-SDT. Whether introduce a new timer or reuse an existing timer is FFS.

UE start a window after CG/DG transmission for CG-SDT. FFS whether to design a new timer or to reuse an existing timer.

In the email discussion [1], whether a new timer or reusing the existing timer was discussed. The email discussion [2] also triggered discussions on the detailed behavior of CG-SDT timer. The RAN2 #116e meeting made several agreements related to the ‘CG-SDT timer’. The email discussion [3] had further discussions regarding more details of CG-SDT timer.

In our view, the new timer served for PDCCH monitoring after CG/DG transmission for CG-SDT should be a MAC layer timer. Within the timer period, UE should monitor PDCCH to check any feedback from network on the status of the latest transmission. According to the feedback, UE can decide to perform a new transmission or retransmission. The purpose of this timer is mainly for the UE power saving.

While for the SDT failure detection timer, i.e., new T319 timer, it should be a RRC layer timer serving for failure detection of CG-SDT, and the RAN2 #113bis-e meeting has agreed that the stop conditions of legacy T319 should apply to SDT failure detection timer. If UE does not receive any RRC DL response message (i.e., the stop conditions of legacy T319) before the SDT failure detection timer expires, UE should indicate the SDT failure to the upper layer. Thus, the intention of the SDT failure detection timer is obviously different from that of the monitoring window for PDCCH in PHY/MAC layer.

Observation 1: The new timer for PDCCH monitoring in the CG-SDT should be a MAC layer timer.

In the email discussion [2][3], the UE behavior of the MAC PDU autonomous retransmission upon the expiry of the CG-SDT timer was discussed. We think such autonomous retransmission mechanism may not be critical for SDT. RAN2 has already agreed to use DG for retransmission. The autonomous retransmission is a kind of optimization.

In our understanding, the Rel-15 basic CG mechanism can be mostly reused for SDT w/o further optimization, the behavior of the new timer could be designed to be similar to the HARQ retransmission timer. In addition, the configuredGrantTimer can be reused for CG-SDT to allow enough time performing retransmission in the HARQ process. UE should monitor PDCCH addressed to C-RNTI or CS-RNTI for any feedback scheduled by network during the new timer running. UE stops the timer if UE receives any PDCCH grant scheduling from network. When the timer expires, UE stops the PDCCH monitoring. If no dynamic grant for retransmission is received, UE assumes the transmission is successful.

Proposal 1: The behavior of the new timer could be similar to the HARQ retransmission timer.

Proposal 2: UE should monitor PDCCH addressed to C-RNTI or CS-RNTI when CG-SDT timer is running. The CG-SDT timer should be stopped when UE receives any PDCCH scheduling from network.

The email discussion [3] raised the question regarding whether supports subsequent new transmission on CG-SDT. We do believe subsequent transmission on CG resource should be supported which is also one of the WID objectives and has been agreed in the past RAN2 meetings. During the email discussion, there are two options for down selection. In our view, we think that implicit ACK by DG of uplink new transmission can be one of the acknowledgement methods in subsequent CG-SDT. For the other options such as MAC CE based acknowledgement is not desired which introduces too many specification impacts.

Proposal 3: UE assumes the implicit acknowledgement of CG transmission if UE receives the PDCCH addressed to C-RNTI of uplink new transmission.

Re-evaluate the SSB for every CG transmission

In RAN2 #115e meeting, it has been agreed that UE has to re-evaluate the SSB for every CG transmission during the subsequent data phase. Such SSB re-evaluation for every CG transmission is for the purpose of CG resource selection.

In some cases, it should be noted that UE may not finish evaluating the beam before the next CG occasion. If UE is configured with periodical CG resources, a necessary condition for UE to perform SSB evaluation before each CG transmission would be: SSB is transmitted within each period of CG resource. Otherwise, UE has nothing to measure between two adjacent CG occasions. Furthermore, UE usually needs to take certain time to perform the beam evaluation and calculate the qualified one which should be defined in RAN4. If UE does not finish SSB evaluation when the next CG occasion is coming, it is worse for UE reporting no qualified SSB for CG resource. In RAN2 #115e meeting, there is FFS on what happens if no SSBs are valid or if no sample is available.

Observation 2: In some cases, it is possible there is no valid SSB available to be evaluated before each CG transmission and/or UE may not finish evaluating the beam before the next CG occasion.

It should be noted that a Note is already specified in the RA resource selection, i.e., 5.1.2 section of MAC spec [3], NOTE 1: When the UE determines if there is an SSB with SS-RSRP above rsrp-ThresholdSSB or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS, the UE uses the latest unfiltered L1-RSRP measurement.

Considering the Note above and to resolve the issue above, we believe the similar approach can be applied for the SSB evaluation in CG case. That is the latest SSB should be allowed to use for performing the next CG transmission if no valid SSB available and/or UE has not finish the SSB evaluation. Thus, we have the following proposals.

Proposal 4: If no valid SSB available to be evaluated and/or UE hasn’t finished the SSB evaluation before the next new CG transmission, UE should be allowed to use the latest SSB to perform the next CG transmission.

CG resource request message

In the RAN2 #112e and #113e post email discussion, whether UE should send CG resource request message for CG-SDT resource while being in RRC_CONNECTED has been extensively discussed. In the LTE PUR, the PUR resource configured for a UE could be triggered either by UE sending a PUR Configuration Request message or by network side information such as, subscription information and/or traffic statistics by network itself. Based on UE request, network may configure PUR to the UE.

For NR small data transmission, the similar mechanism could be adopted which will be quite efficient for network to configure the appropriate CG resource for small data transfer. Even the Subscription UE Differentiation Information is known by network, UE may have different traffic requirements in various scenarios. The QoS information cannot always accurately reflect the user requirement. Even for the same type of IoT device, the uplink traffic pattern could not be always the same.

Observation 3: The UE uplink traffic has different traffic profile even for the same device type.

As the analysis in our companion paper [4], the BSR could only reflect the current amount of data UE may already have but could not represent the future traffic profile.

Observation 4: The buffer status report information only reflects the amount of data that UE may already have. It is difficult for network to know the future traffic profile.

If UE is allowed to send CG resource request message, it can be observed that it is beneficial at least in the following scenarios.

- UE requests CG resource configuration to the network by sending CG resource request message when the UE is in RRC_CONNECTED, and network provides the appropriate CG resource configuration in RRC Release message.

- UE sends CG resource request message in MSGA/Msg3 to indicate UE buffer and traffic status so that network can decide whether to move UE to RRC_CONNECTED (if potential large amount of data is reported), or keep UE in RRC_INACTIVE and providing the CG resource configuration in the RRC Release message. Noted that in the last meeting, RAN2 has confirmed the RRC Release message sent at the end of the SDT may contain the CG resource.

- The UE reported CG resource request message could be under the UE assistance information framework and the serving gNB may forward this message via Xn to anchor gNB, which is helpful for the anchor gNB to make decision on the UE RRC state transition or additional UL resource allocation for small data.

Observation 5: It is helpful and beneficial for network to make decision on UE RRC state transition or providing appropriate CG resource for UE if UE sends CG resource request message.

The following contents could be considered in the CG resource request message.

- Buffer status information, to indicate the amount of data UE may already have in the buffer.

- Small data traffic pattern, to indicate the type of small data, e.g., one-shot or multi-shot. The traffic pattern can also optionally include the periodicity of traffic and the amount of data for each traffic occasion.

The CG request message could be a new RRC message or under the UE assistance framework, and the details on prevention redundant request from UEs and whether needs prohibit mechanisms can be for further study. Thus, we have the following proposal.

Proposal 5: UE is allowed to send the CG resource request message to request for appropriate CG configuration from network.

Proposal 6: The CG resource request message contains at least the buffer status, traffic pattern indication with one-shot or multi-shot, periodicity of traffic and estimated amount data of each shot.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Monitoring window for CG-SDT

Observation 1: The new timer for PDCCH monitoring in the CG-SDT should be a MAC layer timer.

Proposal 1: The behavior of the new timer could be similar to the HARQ retransmission timer.

Proposal 2: UE should monitor PDCCH addressed to C-RNTI or CS-RNTI when CG-SDT timer is running. The CG-SDT timer should be stopped when UE receives any PDCCH scheduling from network.

Proposal 3: UE assumes the implicit acknowledgement of CG transmission if UE receives the PDCCH addressed to C-RNTI of uplink new transmission.

Re-evaluate the SSB for every CG transmission

Observation 2: In some cases, it is possible there is no valid SSB available to be evaluated before each CG transmission and/or UE may not finish evaluating the beam before the next CG occasion.

Proposal 4: If no valid SSB available to be evaluated and/or UE hasn’t finished the SSB evaluation before the next new CG transmission, UE should be allowed to use the latest SSB to perform the next CG transmission.

CG resource request message

Observation 3: The UE uplink traffic has different traffic profile even for the same device type.

Observation 4: The buffer status report information only reflects the amount of data that UE may already have. It is difficult for network to know the future traffic profile.

Observation 5: It is helpful and beneficial for network to make decision on UE RRC state transition or providing appropriate CG resource for UE if UE sends CG resource request message.

Proposal 5: UE is allowed to send the CG resource request message to request for appropriate CG configuration from network.

Proposal 6: The CG resource request message contains at least the buffer status, traffic pattern indication with one-shot or multi-shot, periodicity of traffic and estimated amount data of each shot.

# References

- R2-2107930, Report of [Post114-e][508][SData] Open issues for CG-SDT, Qualcomm.

- R2-2110670, [Post115-e][509][SDT] CG open issues, Xiaomi.

- R2-2200033, [Post116-e][509][SDT]CG open issues (Huawei), Huawei.

- R2-2200727, Remaining issues on CP aspects of SDT, Qualcomm.

# Annex