3GPP TSG RAN WG2 Meeting #116-bis-e	R2-22000886

Electronic meeting, January 17-25th, 2022

Title:	Updated NR_NTN_solutions work plan

Source:	Thales

Type:	Work Plan

For:	Endorsement

Agenda Item:	8.10.1 - Organizational [NR_NTN_solutions]

Release:	Rel-17

# Introduction

At the RAN#92-e meeting, the work item “Solutions for NR to support non-terrestrial networks (NTN)” was revised, with the core part completion by March 2022 (RAN#95), and the RAN4 performance part by September 2022 (RAN#97) [1].

In this contribution, an updated work plan for the Rel-17 NR-NTN work item is proposed for RAN 2 & 3.

# WID Objective

The work item aims to specify the enhancements identified for NR NTN (non-terrestrial networks) especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air-to-ground) scenarios according to the following principles:

- FDD is assumed for core specification work for NR-NTN.

  - NOTE: This does not imply that TDD cannot be used for relevant scenarios, e.g. HAPS, ATG.

- Earth fixed Tracking area is assumed with Earth fixed and moving cells.

- UEs with GNSS capabilities are assumed.

- Transparent payload is assumed

- Handheld devices in FR1 are supported (e.g. Power class 3)

- “VSAT” devices with external antenna (including fixed and moving platform mounted devices) at least in FR2 are supported for the RAN1-3 specifications. “VSAT” characteristics in TR 38.821 can be assumed for the RAN1-3 specifications.

The detailed objectives are to specify enhancing features to Rel-15 & Rel-16’s NR radio interface & NG-RAN as follows:

## RAN1

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821):

- Timing relationship enhancements[RAN1,RAN2]

- Enhancements on UL time and frequency synchronization [RAN1,RAN2]

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

In addition, the following topics should be specified if beneficial and needed:

- Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2].

- Feeder link switch [RAN2,RAN1]

- Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2]

  - Including signalling of polarization mode

## RAN2

NOTE: offset based solutions for timer adaptations are assumed.

The following user plane procedures enhancements should be specified (see TR 38.821)

- MAC

  - Random access:

    - Definition of an offset for the start of the ra-ResponseWindow for NTN.

    - Introduction of an offset for the start of the ra-ContentionResolutionTimer to resolve Random access contention

    - Solutions for resolving preamble ambiguity and extension of RAR window.

    - Adaptation for Msg-3 scheduling

      - Only for the case with pre-compensation of timing and frequency offset at UE side)

  - Enhancement on UL scheduling to reduce scheduling latency.

  - DRX:

    - If HARQ feedback is enabled, introduction of offset for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL.

    - If HARQ is turned off per HARQ process, adaptions in HARQ procedure

  - Scheduling Request: Extension of the value range of sr-ProhibitTimer

- RLC

  - Status reporting: Extension of the value range of t-Reassembly

  - Sequence Numbers: extension of the SN space only for GEO scenarios

- PDCP

  - SDU discard: Extension of the value range of discardTimer.

  - Sequence Numbers: extension of the SN space for GEO scenarios.

The following control plane procedures enhancements should be specified (see TR 38.821)

- Idle mode:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

  - Definition of NTN (satellite/HAPS) cell specific information in SIB

- Connected mode

  - Enhancement necessary to take into account location information (UE & Satellite/HAPS) and/or ephemeris in determining when to perform hand-over, in order to have a high degree of hand-over control for hand-over robustness and coverage management.

  - Enhancement to existing measurement configurations to address absolute propagation delay difference between satellites (e.g. SMTC measurement gap adaptation to the SSB/CSI-RS measurement window) [RAN2/4].

- Service continuity for mobility from TN to NTN and from NTN to TN systems (to be addressed when connected mode mobility has sufficiently progressed)

- Identify potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]

Furthermore the following can be considered with 2nd priority

- Verify the applicability of existing Rel-16 ANR techniques to solve PCI confusion in order to support co-channel operation between HAPS & terrestrial networks and develop enhancements if needed [RAN2/3]

## RAN3

The following NG-RAN architecture enhancements should be specified (see TR 38.821)

- to support feeder link switch over in Transparent payload architecture based LEO scenarios

- network identities handling

- registration update and paging handling

- cell relation handling and related features e.g. neighbours, ANR, RAN paging.

# Work Plan

## For RAN1 WG

In this section, the RAN1 work plan for NR support non-terrestrial network WI is provided:

February 2022, RAN1#108, e-meeting

- Rel-17 maintenance

## For RAN2 WG

In this section, the RAN2 work plan for NR support non-terrestrial network WI is provided:

January 2022, RAN2#116-bis, e-meeting

- Agree detailed solutions for user plane aspects (MAC, RLC/PDCP)

  - TA information reporting in idle and connected mode

  - UL synch issue that holds two parts, the validity timer for ephemeris and common TA that is broadcasted, but also the GNSS fix issue

  - HARQ

- Agree detailed solution for control plane aspects about

  - system information signaling

  - cell (re) selection in idle mode (including NTN-TN mobility)

  - mobility procedures in connected mode (including NTN-TN mobility)

  - measurements

- Further discuss UE location in NTN

- Discuss solutions for HAPS enhancement

February 2022, RAN2#117, e-meeting

- Agree remaining details for user plane aspects (MAC, RLC/PDCP)

  - TA information reporting in idle and connected mode

  - UL synch issue that holds two parts, the validity timer for ephemeris and common TA that is broadcasted, but also the GNSS fix issue

  - HARQ

- Agree remaining details for control plane aspects about

  - system information signaling

  - cell (re) selection in idle mode (including NTN-TN mobility)

  - mobility procedures in connected mode (including NTN-TN mobility)

  - measurements

- Agree potential adaptations for UE location

- Agree solutions for HAPS enhancement

## For RAN3 WG

In this section, the RAN3 work plan for NR support non-terrestrial network WI is provided:

January 2022, RAN3#114-bis, e-meeting

- Handling of incoming LSs from SA2, SA3 and RAN2 with possible impact related to Multiple TAC and UE location aspects.

- Discuss the introduction of a new cause value for UE Context Release Request in case of wrong PLMN selection (country mismatch).

# References

RP-213691, “Solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#94-e, December 2021.

END