3GPP TSG-RAN WG2 #116bis-e                                                                                  R2-2201296

eMeeting, 17th – 25th January, 2022

Agenda Item:	    8.2.2.1

Source: 	    MediaTek Inc.

Title:		    CSI-RS reporting for deactivated SCG

Document for:	    Discussion and decision

# 1 Introduction

The paper discusses on CSI-RS reporting on or for deactivated PSCell.

# 2 Discussion

Whether to send CSI-RS reporting on the (or for the) deactivated PSCell is discussed before but seems no clear conclusion. We think CSI-RS reporting is not a must for deactivated PSCell since no data transmission is ongoing. The basic PSCell quality could be maintained by RLM/BFR mechanism. Sending CSI-RS reporting on the deactivated PSCell will increase power consumption and is not preferable from our side. Sending CSI reporting for the deactivated PSCell via MCG requests additional inter-node message design and may not worth it considering the inter-node delay. Therefore, we suggest keeping the design simple and propose NO CSI reporting for or on the deactivated PSCell. Corresponding TP is provided in Annex A.

Proposal 1: CSI-RS reporting in the deactivated PSCell or for the deactivated PSCell is NOT supported. RAN2 to adopt the TP in Annex for running CR discussion.

# 3 Conclusions

Base on the discussion in section 2, we propose the following:

Proposal 1: CSI-RS reporting in the deactivated PSCell or for the deactivated PSCell is NOT supported. RAN2 to adopt the TP in Annex for running CR discussion.

# Annex: TP for 38.321 (Delta against running MAC CR R2-2111643)

## 5.X	Activation/Deactivation of SCG

Editor note: for terminology” activation/deactivation of SCG”, further discuss if a better wording is needed.

The network may activate and deactivate the configured SCG. Upon configuration of an SCG, the SCG is activated unless the parameter scg-State is set to deactivated for the SCG by upper layers.

The configured SCG is deactivated by:

-  receiving scg-State per SCG;

Editor note: FFS if MAC CE is used for SCG activation/deactivation.

The MAC entity shall for the configured SCG:

1>	if an SCG is configured with scg-State set to activated upon SCG configuration:

2>	activate the SCG according to the timing defined in TS 38.xxx [xx] for direct SCG activation; i.e. apply normal SCG operation including:

3>	SRS transmissions on the PSCell;

3>	CSI reporting for the PSCell;

3>	PDCCH monitoring on the PSCell;

3>	PUCCH transmissions on the PSCell.

- else if upper layers indicate that the SCG is deactivated:

2>	deactivate all the SCells of the configured SCG according to clause 5.9;

2>	deactivate PSCell according to the timing defined in TS 38.xxx [xx], including:

3>	not transmit SRS on the PSCell:

3> not transmit CSI reporting on the PSCell or for the PSCell;

3>	not transmit on UL-SCH on the PSCell:

3>	not monitor the PDCCH on the PSCell.