3GPP TSG-RAN WG2 Meeting #116bis-e	R2- 2201370

Online, Jan 17 – Jan 25, 2022

Agenda Item		:	8.1.3.1

Source		:	LG Electronics Inc.

Title		:	Remaining issues for MII

Document for		:	Discussion and Decision

1. Introduction

In RAN2#115e-meeting, the following agreements were made:

Agreements @RAN2#115-e: - For CONNECTED: - The UE reports the following MBS interest information (as LTE SC-PTM): - MBS frequency list - priority between the reception of all listed MBMS frequencies and the reception of any unicast bearer - TMGI list - FFS whether the MII is reported via UEAssistanceInformation or a new RRC message. Agreements @RAN2#116-e: - Confirm that the UE may initiate MII procedure upon successful connection establishment, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast reception, upon change to a PCell broadcasting SIBx1. FFS other triggers. FFS network control.

In this contribution, we discuss remaining issues on the MBS Interest Idication.

2. Discussion

In NR, we have a unified procedure, i.e. UE assistance information, to report a various UE information to assist network in configuring proper parameters that fits the condition of individual UE. The UE assistance information procedure is initiated by different condition depending on the UE information to be reported. For instance, if the overheating assistance information is reported via UE assistance information, the procedure is initiated upon detecting internal overheating. Therefore, though the triggering condition RAN2 defined for MBS interest reporting is different from that of existing UE assistance information, it can be easily extended to support the MBS interest. There is no clear reason to introduce a separate procedure to indicate the MBS related UE information, and it is better to use the unified procedure to keep the consistency in RRC spec. Therefore, we propose to use UE assistance information procedure to report the UE’s broadcast interest.

Proposal 1. UE reports the broadcast interest via UEAssistanceInformation.

Unlike broadcast session, for multicast session ‘subscription’ and ‘joining’ procedures is required prior to beginning the multicast reception. Therefore, the core network, i.e. AMF, knows which multicast session UE wants to receive without a separate interest indication. However, the concern is whether or not the UE’s multicast interest is forwarded to gNB in real time. If AMF provides the up-to date information on multicast interest to gNB upon completion of ‘subscription’ and ‘joining’ procedures, gNB would be able to make a mobility decision for the multicast service continuity based on the information. However, if not, UE should report the MBS interest for multicast session also to gNB. Therefore, RAN2 needs to confirm whether gNB is aware of multicast session interest of UE or not. If not, we suggest that AMF should provide the multicast interest of UE to gNB.

Proposal 2. RAN2 needs to check whether gNB can acquire the multicast interest of UE from core network. If it is impossible, UE should report the MBS interest for multicast session also.

Regarding the triggering condition of MBS interest indication, one concern raised during the last meeting is whether BWP switching should trigger the MBS interest indication or not. However, in the last meeting, RAN2 agreed that when evaluating which frequencies to report in MBS interest indication, the UE does not take into account the serving frequencies that are currently configured. Since the BWP switching happens within a current serving cell on the serving frequency, the frequency of interest is not changed by BWP switching. Therefore, UE doesn’t need to report the MBS interest indication upon BWP switching.

Proposal 3. BWP switching is not considered as a triggering condition of MBS interest indication.

3. Conclusion

Proposal 1. UE reports the broadcast interest via UEAssistanceInformation.

Proposal 2. RAN2 needs to check whether gNB can acquire the multicast interest of UE from core network. If it is impossible, UE should report the MBS interest for multicast session also.

Proposal 3. BWP switching is not considered as a triggering condition of MBS interest indication.

4. References

[1] TS 38.331 v16.7.0, NR; Radio Resource Control (RRC) protocol specification

[2] TS 36.331 v16.7.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

[3] TS 38.321 v16.6.0, NR; Medium Access Control (MAC) protocol specification