3GPP TSG-RAN WG2 Meeting #116bis electronic   						    	R2-2201386

E-Meeting, 17th Jan – 25th Jan, 2022   							    revision of R2-2110678

Source: 	Xiaomi Communications

Title:  	Clarification on the serving cell measurement for mTRP

Agenda Item:	8.17.2

Document for:	Discussion and Decision

# Introduction

According to the previous RAN2 discussion on the L1/L2-centric inter-cell beam management for feMIMO, RAN2 made the following agreements for inter-cell beam management:

- RAN2#114e meeting agreements [1]: - RAN2 confirm the simplified procedures on the inter-cell multi-TRP-like model as a baseline RAN2 understanding: - Scenario 1: Inter-cell multi-TRP-like model - 1. UE receives from serving cell, configuration of SSBs of the TRP with different PCI for beam measurement, and configurations needed to use radio resources for data transmission/reception incl resources for different PCI. - 2. UE performs beam measurement for the TRP with different PCI and report it to serving cell. - 3. Based on the above reports, TCI state(s) associated to the TRP with different PCI is activated from the serving cell (by L1/L2 signaling). - 4. UE receives and transmits using UE-dedicated channel on TRP with different PCI. - 5. UE should be in coverage of a serving cell always, also for multi-TRP case, e.g. UE should use common channels BCCH PCH etc. from the serving cell (as in legacy).

- RAN2#116e meeting agreement [6]: - Add SSB/PCI information for ICBM as cell-level information and link unified TCI state information to that. FFS on exact Stage-3 details.

According to the RAN#92-e meeting discussion, RAN decided to only consider the “inter-cell-mTRP-like model” in Rel-17.

Proposed way forward (WF) (outcome of the intermediate round): On the scope of Rel-17 NR_FeMIMO: - RAN confirms that inter-cell mTRP in RAN1 work only considers multi-DCI and multi-PDSCH reception (per WI objective). Any scheme tailored for reception of a single PDCCH and/or a single PDSCH is not supported in Rel-17 mTRP. - Regarding scope and workflow of L1/L2-centric inter-cell beam management for multi-beam enhancement, for Rel-17: - Only scenario for inter-cell-mTRP-like model (with no change in serving cell) will be considered in Rel-17. - Scenarios where change in serving cell via a L1/L2-triggered handover scheme are not considered in Rel-17 and may be considered in Rel-18 - Further discuss how to clarify the Rel-17 objectives associated with scenario 1 for L1/L2-centric inter-cell beam management (during later round(s)) - Only intra-DU and intra-frequency scenarios will be considered in Rel-17 (excluding inter-DU or inter-frequency scenarios) - In RAN1#106-e, conclude on the synchronization and the timing advance assumptions between the cells

In this contribution, we discuss the RAN2 serving cell measurement impacts brought by the “inter-cell-mTRP-like model”.

# Discussion

## Serving cell measurement

According to the current RRC specification [3] as quoted above, the UE would only measure the frequency of the serving cell, as provided by the servingCellMO in the ServingCellConfig IE. The servingCellMO contains the measurement object providing the measurement frequency (i.e. ssbFrequency), the reference signal configuration (i.e. referenceSignalConfig), and the cell-specific offsets (i.e. cellsToAddModList). According to the current servingCellMO configuration, the UE can obtain the measurement results of multiple PCI(s) including the serving cell PCIs, when multiple PCIs are associated to the same serving cell.

Observation 1: According to the current servingCellMO configuration, the UE obtains the measurement results of multiple PCI(s) (including multiple serving cell PCIs), when multiple PCIs are associated to the same serving cell.

According to the measurement report configuration as quoted above, the event-triggered or periodical measurement report configuration does not indicate which PCI is used for the measurement event evaluation or which PCI needs to be reported (e.g. when a measurement event is triggered), when multiple PCIs are associated to the same serving cell.

Observation 2: In the measurement event configuration, the UE needs to know which measurement result of the serving cell PCI is used for the measurement event evaluation, when multiple PCIs are associated to the same serving cell.

Observation 3: In the measurement report, the UE needs to know which measurement result of the serving cell PCI is reported, when multiple PCIs are associated to the same serving cell.

To resolve the issue given in Observation 2, we think that the specification should be clarified on which PCI measurement result is used for the measurement event evaluation.

Proposal 1: When multiple PCIs are associated to the same serving cell, RAN2 is kindly requested to clarify which PCI is used for the measurement event evaluation (e.g. A3/A5).

As quoted above, when the UE needs to report the serving cell measurement results (e.g. piggybacked with other measurement results or periodic report), the serving cell measurement result is included in the measResultServingCell within measResultServingMOList. For each serving cell, the UE is only allowed to indicate one measurement result (i.e. measResultServingCell including only one PCI) of the serving cell. Then the specification should be clarified on which PCI measurement result is included in the serving cell measurement report.

Proposal 2: When multiple PCIs are associated to the same serving cell, RAN2 is kindly requested to clarify which PCI measurement result is included in the serving cell measurement report (i.e. measResultServingCell).

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: According to the current servingCellMO configuration, the UE obtains the measurement results of multiple PCI(s) (including multiple serving cell PCIs), when multiple PCIs are associated to the same serving cell.

Observation 2: In the measurement event configuration, the UE needs to know which measurement result of the serving cell PCI is used for the measurement event evaluation, when multiple PCIs are associated to the same serving cell.

Observation 3: In the measurement report, the UE needs to know which measurement result of the serving cell PCI is reported, when multiple PCIs are associated to the same serving cell.

Proposal 1: When multiple PCIs are associated to the same serving cell, RAN2 is kindly requested to clarify which PCI is used for the measurement event evaluation (e.g. A3/A5).

Proposal 2: When multiple PCIs are associated to the same serving cell, RAN2 is kindly requested to clarify which PCI measurement result is included in the serving cell measurement report (i.e. measResultServingCell).

# Reference

[1] RAN2#114-e meeting minutes.

[2] RAN#92-e meeting minutes.

[3] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”.

[4] RAN2#116-e meeting minutes.