3GPP TSG-RAN2 Meeting #116-bis-e	R2-2201493

Online, 17-25 January 2022

Agenda item:	8.12.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM relaxations for REDCAP

WID/SID:	NR_redcap-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

RAN2 objectives for reduced capability (REDCAP) devices were revised in RP-211574:

In this contribution we discuss RRM relaxations as per the following revised work item objectives:

Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation. - Specify UE requirements for RRM measurement relaxation [RAN4] - No RRM measurement relaxations are specified for the serving cell.

The following was agreed for RRM relaxations in RAN2#113ebis meeting:

Agreements:

- Assuming there will be a stationary property based on subscription (which is FFS), we will not restrict to this and will continue to assume that a UE can use some RSRP/RSRQ based criteria (FFS whether reuse R16 thresholds or new ones. FFS also on the use of a beam based criteria)

Agreements:

1.	At least for RRC idle/inactive, a measurement-based R17 stationarity criterion can be configured separately from R16 low-mobility criterion for R17 UEs supporting the feature. FFS how the configuration is provided. FFS whether this stationarity criterion is based on:

-	the same algorithm used in R16 low-mobility criterion but with its own specific set of thresholds; and/or

-	a combination of R16 low-mobility criterion and/or beam-change based criterion. Exact details of beam change criterion are FFS.

Agreements - via email (from offline [102]):

- Network can configure R17 stationarity criterion/criteria together with a not-at-cell-edge criterion, to trigger RRM relaxations in RRC Idle/Inactive for R17 UEs supporting the feature. FFS whether the R16 not-at-cell-edge thresholds can be reused or separate R17 not-at-cell-edge thresholds are needed.

The following was agreed for RRM relaxations in RAN2#114 meeting:

Agreements:

- An RSRP/RSRQ based stationarity criterion (Working Assumption: the same as in idle/inactive) can be configured for UEs in RRC Connected. If the criterion is met, this is reported to the network (FFS how/when). It is FFS whether, based on this, besides possibly reconfiguring RRM measurements (up to network implementation), the network can enable RRM measurement relaxation (FFS whether same method as in Idle/Inactive)

Agreements:

- Subscription based relaxation criteria will not be considered in Rel-17 RRM relaxation

Agreements via email (from offline 111):

- Reuse R16 low mobility criterion, as part or whole of Rel-17 stationary criterion in RRC_IDLE/INACTIVE. When NW configures both Rel-17 stationary criterion and Rel-16 low mobility criterion, NW configures different Rel-17 thresholds (i.e., SSearchDeltaP_stationary/TSearchDeltaP_stationary) from Rel-16 (SSearchDeltaP / TSearchDeltaP). How to configure the criterion (e.g. more stringent) is left to NW implementation (i.e. no specification impact to RAN2).

- Postpone the following discussion until RAN4 defines RRM relaxation method for Rel-17:

When NW configures both R16/R17 relaxation criteria and the UE fulfills both, UE performs:

- Option 1) UE performs Rel-17 RRM relaxation method

- Option 2) It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation

Agreements online:

- Working Assumption: If beam-level criterion is adopted for Rel-17 stationary criterion in RRC_IDLE/INACTIVE, it is configured separately with Rel-16 low mobility criterion reused

- When NW configures Rel-17 RRM relaxation for RRC_IDLE/INACTIVE, Rel-17 stationary criterion is mandatory, and Rel-17 not-at-cell-edge criterion is optional configuration. FFS whether the same applies to RRC Connected

- Continue discussion on Rel-17 not-at-cell-edge criterion in RRC_IDLE/INACTIVE within two options:

- Option 1) Reuse Rel-16 not-at-cell-edge criterion with the same thresholds (i.e., SSearchThresholdP / SSearchThresholdQ)

- Option 2) Reuse Rel-16 not-at-cell-edge criterion with the different thresholds

The following was agreed for RRM relaxations in RAN2#115 meeting:

Agreements:

- Do not introduce nor reuse not-at-cell-edge threshold for R17 RRC_CONNECTED UEs.

from offline 110:

- Do not introduce beam change based criterion in Rel-17.

- The network provides the configuration of stationarity criterion to the UE via dedicated signalling (e.g. RRCReconfiguration message) in RRC_CONNECTED.

- Send LS to RAN4 to inform RAN2 conclusions for RRM relaxation.

- The LS to RAN4 includes the agreed RAN2 conclusions and “For RRC_IDLE/INACTIVE, RAN4 is asked to study and define corresponding R17 RRM relaxation method” .

Agreements:

- Introduce separate Rel-17 not-at-cell-edge threshold, and the new threshold is only associated with Rel-17 stationary criterion (if configured). If configured with a not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17

The following was agreed for RRM relaxations in RAN2#116 meeting:

Agreements:

- RAN2 assume that the existing RRM measurement framework can be used as baseline for enabling and disabling RRM relaxations for UEs in RRC Connected. Other methods can be considered too based on relaxation methods agreed by RAN4.

- RAN2 understands that no prohibit timer is needed, if legacy measurement reporting framework is reused by UE to report its relaxation status

- The granularity of RRM measurement relaxations (i.e. whether it should be specified per beam, per cell or per frequency) should be handled by RAN4

Agreements via email - from offline 111:

- UE is not allowed to relax its RRM measurements if both stationarity criterion and R17 not-at-cell-edge criterion are configured but UE meets only the R17 not-at-cell-edge criterion.

- UE reports to network when it no longer meets relaxation criteria.

- No additional signaling is introduced for network to tell UE whether and which criteria for RRM relaxation is considered satisfied when leaving RRC_CONNECTED state.

- No need for UE to send UE Assistance Information to request network configuring it with relaxation criteria.

- UE does not report its history/state of RRM relaxation when transitioning from RRC Idle/Inactive to RRC Connected.

- Relaxation criteria for UEs in RRC Connected are configured by only dedicated signaling.

# 2	Discussion

RAN2 work item objective for RRM relaxations define that RRM relaxations shall be specified for non-serving cells only:

Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation. - Specify UE requirements for RRM measurement relaxation [RAN4] No RRM measurement relaxations are specified for the serving cell

Proposal 1: RRM relaxation is supported for the non-serving cells only

Work item defines also objective to have support for enabling/disabling of RRM relaxation by both broadcast and dedicated signalling.

Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation.

Broadcast configuration should be supported for RRM relaxation according to work item objective. Also network dedicated control for enabling RRM relaxations should be supported for all the RRC states as clearly indicated in the work item objectives. When releasing UE’s RRC connection, RAN can then enable RRM relaxation for the UE in the RRC Release message. The UE can apply the enabled RRM relaxation method once it is in RRC Idle/Inactive.

Proposal 2: RRM relaxation configuration can be provided in system information and in dedicated signaling for IDLE/INACTIVE UE

Proposal 3: Network can enable/disable RRM relaxation for IDLE/INACTIVE/CONNECTED UE with dedicated signaling

Proposal 4: Network can enable/disable RRM relaxation for IDLE/INACTIVE UE with RRC Release message

# 3	Conclusion

In this contribution RRM relaxations for REDCAP UE was discussed, and the following is proposed:

Proposal 1: RRM relaxation is supported for the non-serving cells only

Proposal 2: RRM relaxation configuration can be provided in system information and in dedicated signaling for IDLE/INACTIVE UE

Proposal 3: Network can enable/disable RRM relaxation for IDLE/INACTIVE/CONNECTED UE with dedicated signaling

Proposal 4: Network can enable/disable RRM relaxation for IDLE/INACTIVE UE with RRC Release message