3GPP TSG-RAN WG2 Meeting #116bis-e	R2-2201710

Online, 17 – 25 January 2022

Agenda item:	8.20.1

Source: 	Qualcomm Incorporated

- Title:  	[AT116bis-e][210][71 GHz] RRC aspects of CR for 71 GHz (Qualcomm)

Document for:	Discussion and decision

# Introduction

This document will capture the RRC open issues and suggested solutions as an outcome of this email discussion:

[AT116bis-e][210][71 GHz] RRC aspects of CR for 71 GHz (Qualcomm)

Scope: Update running RRC CR for 71 GHz based: 1) how to handle MIB with 71 GHz (e.g. use spare bit, define new MIB, modify existing fields)? 2) are new values needed for some fields (e.g. time offsets needed for various fields)? 3) is there some input from RAN1 that needs to be added to the RRC running CR?

Intended outcome: Discussion summary in R2-2201710.

Deadline 3 (discussions for 2nd week Mon/Tue online):

Comment deadline: Thursday W1, 1600 UTC (for collecting views)

Rapporteur proposals: Friday W1, 0900 UTC (proposed resolution of issues)

Document deadline: Monday W2, 1200 UTC (report or agreed CRs)

No extensions to this deadline for regular discussions. Discussions handling CRs may continue to 1-week email (based on chair decision).

Please provide your contact information in the table below.

| Company | Contact Name, Email |
| --- | --- |
| Qualcomm | Ozcan Ozturk, oozturk@qti.qualcomm.com |
| ZTE | Eswar Vutukuri, eswar.vutukur@zte.com.cn |
| Apple | Naveen Palle, naveen.palle@apple.com |
| LGE | Gyeong-Cheoll LEE, gyeongcheol.lee@lge.com |
| Intel Corporation | Seau Sian Lim, seau.s.lim@intel.com |
| Huawei, HiSilicon | Tao Cai, tao.cai@huawei.com |
| Ericsson | Min wang, min.w.wang@ericsson.com |
| OPPO | Shi Cong, shicong@oppo.com |
| Samsung | Taeseop Lee, taeseop.lee@samsung.com |
| NTT Docomo | Shoki Inoue, syouki.inoue.cr@nttdocomo.com |
| vivo | Yitao Mo (Stephen), yitao.mo@vivo.com |

# Discussion

The following papers submitted to RAN2#116bis-e discuss and propose solutions for Control Plane and RRC related aspects, except for UE capability, for 71 Ghz WI:

R2-2200718	List of issues for completion of FR2-2 Work (Rapporteur Input)	Qualcomm Incorporated	discussion

R2-2200940	Open issue list of RRC CR for 71 GHz	Ericsson (rapporteur)	discussion

R2-2201682	Discussion about RAN2 impacts of Ext 52-71GHz	Huawei, HiSilicon	discussion

R2-2200942	Remaining RRC aspects	Ericsson	discussion

R2-2200733	Discussion on UAI enhancement for operation in FR2-2	Samsung	discussion

R2-2200884	Initial access aspects	Nokia, Nokia Shanghai Bell	discussion

R2-2200461	UP and CP impact on NR operation for upto 71GHz	Intel Corporation	discussion

R2-2201033	Consideration on RRC and MAC running CR	ZTE corporation, Sanechips	discussion

R2-2200941	Remaining protocol aspects	Ericsson	discussion

R2-2201284	Remaining issues for Ext 71GHz	vivo Mobile Com. (Chongqing)	discussion

## Initial and Channel Access

An LS from RAN1 in [1] asks RAN2 feedback on using the spare bit in MIB1 for signaling of of NSSBQCL, a parameter used to derive the QCL assumptions for SSB. RAN1 had the following working assumption (quoted from the LS text):

2 bits will be repurposed in FR2-2 to convey up to 4 values of NSSBQCL, a parameter used to derive the QCL assumptions for SSB. The 2 bits identified by RAN1 for usage are ‘subCarrierSpacingCommon’ and ‘spare’ bit contained in the MIB IE. Note that the former can be repurposed since according to WID for FR2-2, the SCS for SS/PBCH block and CORESET0 are the same

This is a working assumption in RAN1 since MIB ASN.1 is RAN2 responsbility. If RAN2 does not agree to the usage of the spare bit, RAN1 will need to find another way to signal the above QCL parameter, possibly agree to one of the other options they discuss before. Please also see Questions A2 and A3 below for proposals in RAN2 contributions to solve this without using the spare bit.

In the above RAN2 contributions, companies have different opinions. Some did not see any problem with using the only spare bit in MIB while others wanted to keep it for future when it can be used in cases where there are no other alternatives and which can benefit all NR bands.

RAN2 needs to make a simple Yes-No decision.

Question A1: Do you support using the last spare bit in MIB for the the signaling of FR2-2 QCL assumptions for SSB per RAN1 working assumption?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | No | We disagree with the characterisation that this is a spare bit in MIB. In our view this bit is an indication of MIB version. In NR we only have possibility to have two versions of MIB now since this is the last bit in MIB. In case any need in future is identified for a new version of MIB (e.g. some critical system issue with existing MIB is detected), we need the possibility to broadcast a new MIB and this is what the bit is there for. It should be noted that this discussion already happened in Rel-15. We replied to RAN1 that this bit is not there for RAN1 to use for any other purpose (please see R2-1712056: RAN2 agreed that at least one spare bit is needed for RAN2 use in future). So, we don’t think this message has changed between then and now. |
| Apple | No | Same comments as ZTE |
| LGE | No | Prefer to keep the last spare bit in MIB. |
| Intel | Yes | Even though the spare bit is used for FR2-2, it can still be reused by other band types (i.e. FR1 and FR2-1) for other purpose. Some companies mentioned that this was discussed before in NR-u, but RAN1 provided 2 alternatives at that time and RAN2 selected the alternative that does not require the use of spare bit in MIB. However, there is no alternative given this time round as all bits in the MIB are still applicable to FR2-2. |
| Nokia | Yes | Using it at this point of NR lifetime is not anymore big issue. Alternatively if there is strong concerns we can ask whether RAN1 can live with reduced set (e.g. 2 values) of NSSBQCL We would also point out that the use of the spare bit usage only impact applies for FR2-2 - for other frequency ranges. For FR1 and FR2-1, the spare bit can still bnebe repurposed (if needed) later on. if necessary. |
| Huawei, HiSilicon | Comments | We think it is necessary that RAN1 to check other possibilities first, e.g. reduce the number of needed bits before to use the last spare bit. |
| Ericsson | No | It is preferable to reserve the last spare bit in the MIB for future enhancements that are useful for any band. The re-interpretation of MIB parameters and the combination of different parameters to derive another parameter depending on different cases reduces the readability of the ASN.1 code. In this meeting, it is necessary for RAN2 to indicate our concern in the LS reply to RAN1 so that RAN1 can further study if there is other option to be feasible to be adopted. After that, RAN2 can further discuss the issue based on RAN1 progress. |
| OPPO | No | We also prefer to keep the spare bit. Given RAN1 make it as working assumption, we think it’s ok to send LS back to RAN1 for asking them to reduce the Q to be 2 valaues. |
| Samsung | No | Same view with ZTE. It would be better to keep the last spare bit in MIB for the future critical problem. |
| NTT Docomo | No | Same comment as ZTE. We prefer to keep the last spare bit in any band in case of critical issue. |
| vivo | Yes | Although it is safer to reserve the last spare bit, we think now indicating NSSBQCLcan be regarded as critical usage as we already have no more efficient way to indicate NSSBQCL. |
| Qualcomm | No | This bit should be used only if there is no other alternative. RAN1 has not indicated that there is no other way for signaling of QCL. |

Summary: Out of 12 companies, only 3 support using the spare bit in MIB for signaling of QCL assumptions in FR2-2. ZTE also clarified that this bit is not really for “spare” and same discussion happened in Rel-15.

Given the majority, it seems we can respond to RAN1 that RAN2 will not agree to using the spare bit.

Proposal A1: RAN2 does not agree to using the spare bit in MIB for the signaling of FR2-2 QCL assumptions for SSB. Respond to RAN1 LS accordingly.

There is a proposal in R2-2200942 to introduce a new MIB including a new BCCH-BCH-Message with the justification that this “avoids pre-processing in the network and post-processing in the UE for ASN.1 encoding/decoding and improves readability of the ASN.1 code”. With this approach, there is also no need to change the interpretation of subCarrierSpacingCommon (as RAN1 intends to do with the above QCL signaling) and also no need to use the spare bit.

Question A2: Do you support introducing a new MIB as proposed in R2-2200942?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Okay | We are okay with such approach. We can go with majority view on the alternative options. |
| Apple | We are ok with introducing a new MIB |  |
| LGE | No | It may depend on how important to support up to 4 values of NSSBQCL. In our view, supporting up to 4 values of NSSBQCL may not be critical factor to make the system work with shared spectrum channel access for 480/960 kHz SCS in 60 GHz. |
| Intel | Not prefer but can accept if companies think this can solve the spare bit issue | Defining a new MIB does not help in saving further bits for FR2-2 since all the bits in existing MIB are still needed by FR2-2. It is no difference in using the spare bit in the MIB. It only makes it more readable. |
| Nokia | No | We do not see need for this. One should be able to just redefine existing bits without any issues. And this would be easier also for RAN1 as they have already developed their CRs based on assumption that we repurpose existing fields. Although there is nothing technically wrong in introducing new MIB. |
| Huawei, HiSilicon | No for now | We understand what Ericsson proposed is one way to use the spare bit in (a new) MIB for FR2-2. We suggest first let RAN1 to discuss how to handle the repurposing and re-interpretation. |
| Ericsson | Yes | Defining a new MIB including a new BCCH-BCH-Message avoids pre-processing in the network and post-processing in the UE for ASN.1 encoding/decoding and improves readability of the ASN.1 code. But, we think for this meeting, RAN2 can aim to at least reach an agreement that reusage of the spare bit should be avoided. Whether a new MIB or other options should be adopted can wait for next meeting. |
| OPPO | No | Don’t see the need for introducing a new MIB, may be good to ask RAN1 to update the working assumption so that the supported number of values of Q can be indicated by the existing MIB, i.e., by repurposing the subCarrierSpacingCommon. |
| Samsung | Fine, but | If there are no other solutions (e.g. do not introduce 4 values for QCL assumptions for SSB), introducing a new MIB approach can be acceptable to us. |
| NTT Docomo | No | A new MIB helps to keep the last spare bits for FR1 and FR2-1. However, this is not for FR2-2. So we do not prefer this. If supporting up to 4 values of NSSBQCL is important, a new MIB can be a reasonable option. But we think it may not be critical factor. |
| vivo | No | Using the existing spare bit seems simpler and more efficient, compared with introducing a new MIB. |

Summary: Out of 11 companies, 4 support a new MIB. Therefore, we should assume that legacy MIB will be used for FR2-2.

Proposal A2: As a baseline, the legacy MIB is also used for FR2-2.

On the MIB spare bit issue, another proposal (R2-2201424) is to suggest to RAN1 to support only 2 values of the QCL parameter, by which the need for the spare bit is eliminated.

In general, we can make a decision whether to suggest any alternative options to RAN1, assuming RAN2 does not agree to using the last spare bit.

Question A3: Assuming RAN2 does not agree to using the last spare bit, should RAN2 propose limiting the QCL assumptions to 2 or other alternative options to RAN1?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Up to RAN1 | If this is feasible from RAN1 perspective then it is okay. However, we should also investigate other RAN2 solutions. |
| Apple | We do not agree to proposing to RAN1. Just that the spare bit cannot be used. | We think they can decide the next steps. |
| LGE | See comment | RAN2 can indicate our preference on this issue, i.e., limiting the QCL up to 2, if using the last spare bit is not agreed. However, the preference should not be a specific RAN2 solution and other alternative should be discussed in RAN1 first. |
| Intel | Up to RAN1 | We do not think it is in the scope of RAN2 to limit the QCL assumptions. RAN 2 can just deliver RAN2’s conclusion on whether spare bit in the existing MIB can be used for QCL assumptions for FR2-2 or a new MIB for FR2-2. Other than that, RAN2 should leave it to RAN1. |
| Nokia | Yes | QCL assumptions should be then limited to 2. We do not see strong concerns with this approach as the extra QCL assumptions seem extreme corner cases that will never occur |
| Huawei, HiSilicon | Maybe not | This discussion is up to RAN 1. |
| Ericsson | uncertain | We think this should be left to RAN1 further study. In total, there may be a couple of options feasible - Limiting the QCL value to 2. - Using a new MIB However, we don’t think it is RAN2 responsibility to pick out the options. This needs to be further studied by RAN1 first. |
| OPPO | Yes | We think RAN2 can sugguest RAN1 to do so. |
| Samsung | Up to RAN1 | See our response in A2. |
| NTT Docomo | Up to RAN1 | We think RAN2 can just reply the conclusion, i.e., the last spare bit cannot be used, if using the last spare bit is not agreed. |
| vivo | No | We prefer not to bother RAN1 on this issue as they have already completed the WI work. From the RAN2 perspective, we can anyway make a way out (e.g. defining a new MIB for sake of progress). |

Summary: Only two companies support RAN2 suggest using 2 QCL assumptions while other companies think this should be up to RAN1.

Proposal A3: In RAN2 reply LS to RAN1, do not include any suggestions for changing the QCL assumptions for FR2-2.

The following issue was captured in the paper R2-2200940 by the RRC rapporteur:

- Whether to define the value of channelAccessMode2 as ENUMERATED {enabled, disabled} or ENUMERATED {enabled}

This parameter was called LBT-Mode in the RAN1 list. However, RRC rapporteur preferred to change the name to channelAccessMode2 since this is “aligned name with existing channelAccessMode for FR1 which also describes LBT procedures)”.

With the second option, the UE will assume that the gNB is operating in no-LBT mode when the IE is not signaled.

Question A4: Which option do you prefer for the signaling of channelAccessMode2?

Option 1: ENUMERATED {enabled, disabled}

Option 2: ENUMERATED {enabled}

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Option 2 |  |
| LGE | Option 2 |  |
| Intel | Option 2 |  |
| Nokia | Option2 – field optional | ASN.1 detail – option2 style is more aligned with existing ASN.1 (that is also what is used throughout NR RRC – if we were to use option 1, then it should also be BOOLEAN instead of ENUMERATED for better readability) |
| Huawei, HiSilicon | Option 2 | Agree with Nokia |
| Ericsson | Option 1 | ENUMERATED {enabled, disabled} can reflect 3 different states: - enabled -> operation with shared spectrum channel access; LBT is required. - disabled -> operation with shared spectrum channel access; no LBT is required. - channelAccessMode2 is absent -> operation in licensed spectrum With ENUMERATED {enabled}, the absence of the parameter would only indicate that no LBT is required, but a UE in idle mode behaves differently during cell search depending on whether the cell is operating in licensed spectrum (the UE searches only for the strongest cell on a frequency) or not (the UE may also read MIB/SIB1 from the non-strongest cell). Therefore, we think that it is beneficial to define channelAccessMode2 as ENUMERATED {enabled, disabled} Regarding Nokia’s comments, we think it is a matter of taste. For option 1, we are fine to the majority view to choose either ENUMERATED or BOOLEAN |
| OPPO | Option 2 |  |
| Samsung | Option 2 | We don’t think the strong need of three different states on this field. As RAN1 requested, two state (i.e. LBT-mode and no LTE mode) seems enough. |
| vivo | Option 2 | It is straightforward and aligns the other NR existing similar parameter. |

Summary: Only the proponent company supports Option 1 with the justification that the UE procedures could be different between licensed spectrum and shared spectrum with no LBT.

Proposal A4: channelAccessMode2 is signaled as ENUMERATED {enabled}. This implies that the UE can not distinguish between licensed spectrum and shared spectrum without LBT.

## SCS of 480/960kHz

Since FR2-2 uses the new SCS of 480kHz and 960kHz (in addition to 120kHz), these values need to be captured in the RRC parameters which refer to SCS.

R2-2201033 captures these IEs in the following Table:

| Parameter Position | Comment |
| --- | --- |
| RRCRelease->redirectedCarrierInfo->nr->ssbSubcarrierSpacing | Since SCS of 120, 480 and 960kHz have been supported for SSB, the corresponding field description for these SCS should be added |
| SIB4 ->InterFreqCarrierFreqInfo->ssbSubcarrierSpacing |  |
| MeasObjectNR->ssbSubcarrierSpacing |  |
| SRS-Config->srs-PosResourceToAddModList-r16->spatialRelationInfoPos-r16->ssb-Ncell-r16->ssb-Configuration-r16 ->ssbSubcarrierSpacing-r16 |  |
| MeasObjectCLI-r16->srs-ResourceConfig-r16->srs-SCS-r16 | There is no frequency usage restriction, so it may also be used for FR2-2 |
| MeasObjectCLI-r16->rssi-ResourceConfig-r16->rssi-SCS-r16 |  |
| RateMatchPattern->subcarrierSpacing |  |
| CSI-RS-ResourceConfigMobility->subcarrierSpacing | It is similar to SCS of BWP, so SCS for FR2-2 should also be added |

Question B1: Do you agree adding text with the new SCS values in the field description of parameters in the above Table?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Yes (proponent) |  |
| Apple | Yes, we see no other way. |  |
| LGE | Yes |  |
| Intel | Yes |  |
| Nokia | Looks reasonable | One could also consider could we have more generic definition somewhere e.g. have IE description include this information instead. |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes | These are missing except RateMatchPattern->subcarrierSpacing |
| OPPO | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |

Summary: All companies support the proposals and hence it can be agreed.

Proposal B1: Add text with the new SCS values in the field description of the parameters listed in Table 1 in R2-2201033.

## PDSCH/PUSCH/PUCCH Parameters

R2-2201033 notes that the parameter enableTimeDomainHARQ-BundlingType1-r17 is missing in the running RRC CR. This was listed in the RAN1 list [2] as follows:

The Rapporteur assumes that this should be placed in ServingCellConfig, since it is per serving cell, with the value “ENUMERATED {enabled}”

Question C1: Do you agree adding the parameter enableTimeDomainHARQ-BundlingType1-r17 in ServingCellConfig with the value “ENUMERATED {enabled}?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Apple | Yes |  |
| LGE | Yes |  |
| Intel | Yes |  |
| Nokia | Yes – Look reasonable (with OPTIONAL) | Seems necessary change |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes | we can shorten the field name timeDomainHARQ-BundlingType1-r17 ENUMERATED {enabled} |
| OPPO | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |

'

Summary: There is consensus to introduce this. RRC rapporteur suggested to shorten the IE name.

Proposal C1: The parameter enableTimeDomainHARQ-BundlingType1-r17 is introduced in ServingCellConfig with the value “ENUMERATED {enabled}”. FFS if the name can be shortened.

Another issue for multi-PUSCH is observed in R2-2201033 that the value of maxNrofMultiplePDSCHs-r17 in the running RRC CR as below is not defined.

R2-2201033 proposes to define maxNrofMultiplePDSCHs-r17 as 8, based on the following RAN1 agreements.

Agreement: - The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 120, 480 and 960 kHz. Agreement - The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is also 8 when 2 TB is enabled or when 2 TB is scheduled, for SCS of 120, 480 and 960 kHz. - Note: This is to handle FFS (the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled) in previous agreement in RAN1#106bis-e.

Question C2: Do you agree defining maxNrofMultiplePDSCHs-r17 in pdsch-TimeDomainAllocationListForMultiPDSCH-r17 as 8?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Apple | Ok |  |
| LGE | Yes |  |
| Intel | Yes |  |
| Nokia | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |

Summary: There is consensus to introduce this IE.

Proposal C2: maxNrofMultiplePDSCHs-r17 is defined in pdsch-TimeDomainAllocationListForMultiPDSCH-r17 with the value 8.

There are multiple RRC issues discussed in R2-2200942 by the RRC rapporteur with proposals for their resolution. There are FFS’es in the running CR for some of these. We can discuss them here and gather feedback from companies.

For pdsch-TimeDomainAllocationListForMultiPDSCH in PDSCH-Config, there is an Editor’s Note that “FFS whether any restriction applies to pdsch-TimeDomainAllocationListForMultiPDSCH.

Question C3: Do you think any restrictions should be captured in RRC for pdsch-TimeDomainAllocationListForMultiPDSCH?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | No |  |
| LGE | No | As already indicated in R2-2200942, it seems clear in TS 38.214. |
| Intel | No |  |
| Nokia | No |  |
| Huawei, HiSilicon | No |  |
| Ericsson | No | pdsch-TimeDomainAllocationListForMultiPDSCH only applies to DCI format 1_1, and pdsch-TimeDomainAllocationList (with or without suffix) applies to DCI format 1_0 and DCI format 1_1. If pdsch-TimeDomainAllocationListForMultiPDSCH is not allowed to be configured simultaneously with the pdsch-TimeDomainAllocationList (with or without suffix), it would not be possible to configure a TDRA table for DCI format 1_0, in this case, it would be only possible to apply the default TDRA table for DCI format 1_0. If pdsch-TimeDomainAllocationListForMultiPDSCH is configured, then pdsch-TimeDomainAllocationList should not apply to DCI format 1_1. This has been already clarified in TS 38.214, Table 5.1.2.1.1-1, which is sufficient. |
| OPPO | No |  |
| Samsung | No |  |
| vivo | No |  |

Summary: All companies support not introducing any restrictions.

Proposal C3: No restrictions are captured in RRC for pdsch-TimeDomainAllocationListForMultiPDSCH.

RAN1 has left some parts of the configuration of PDSCH TDRA for multi PDSCH to RAN2. R2-2200942 proposes the following:

- The new PDSCH-TimeDomainResourceAllocation-r17 IE can be configured with either PDSCH repetition or multiple PDSCH.

- Introduce the field pdsch-TimeDomainAllocationListDCI-1-2-r17 and the field pdsch-TimeDomainAllocationList-r17 so that PDSCH repetitions can be used with the new k0 value range.

Question C4: Do you agree with the above proposals for PDSCH TDRA configuration for FR2-2?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes | This is a clean approach. |
| Intel | Yes |  |
| Nokia | OK | Seems Ok approach – can be reveisited in ASN.1 review if issues found |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Samsung | Yes | (same as for r16-suffix IE) |
| vivo | Yes |  |

Summary: There is also consensus for this proposal.

Proposal C4: The following are agreed for signaling of PDSCH TDRA:

- The new PDSCH-TimeDomainResourceAllocation-r17 IE can be configured with either PDSCH repetition or multiple PDSCH.

- Introduce the field pdsch-TimeDomainAllocationListDCI-1-2-r17 and the field pdsch-TimeDomainAllocationList-r17 so that PDSCH repetitions can be used with the new k0 value range.

RAN1 has also left some PUSCH TDRA configuration aspects to RAN2 and R2-2200942 proposes similarly as follows:

- Introduce the field pusch-TimeDomainAllocationListDCI-1-2-r17 and the field pusch-TimeDomainAllocationList-r17 so that PUSCH repetition can be used with the new k2 value range

Question C5: Do you agree with the above proposal for PUSCH TDRA configuration for FR2-2?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes | This is a clean approach. |
| Intel | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Samsung | Yes | (same as for r16-suffix IE) |
| vivo | Yes |  |

Summary: There is also consensus for this proposal.

Proposal C5: Introduce the field pusch-TimeDomainAllocationListDCI-1-2-r17 and the field pusch-TimeDomainAllocationList-r17 so that PUSCH repetition can be used with the new k2 value range.

For PUSCH TDRA, RAN2 also needs to agree on the maximum number of rows in PUSCH-TimeDomainResourceAllocationList-r17. R2-2200942 proposes to limit this to 16 as in Rel-16.

Question C6: Do you agree that pusch-TimeDomainAllocationListForMultiPUSCH-r17 can be configured with up to 16 list elements?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes | Seems fine (no input to extend the max value) |
| Intel | Yes |  |
| Nokia | Yes |  |
| Huawei, HiSilicon | Yes with comments | Update shall be allowed if further input received from RAN1. |
| Ericsson | Yes | This is the similar as R16 |
| OPPO | Yes |  |
| Samsung | Yes | (unless RAN2 receives a specific input from RAN1) |
| vivo | Yes |  |

Summary: There is also consensus for this proposal. There is one comment that update can happen if RAN1 provides further input but this is business as usual.

Proposal C6: The IE pusch-TimeDomainAllocationListForMultiPUSCH-r17 is configured with up to 16 list elements.

Another proposal in R2-2200942 is regarding the k2 values for multiple PUSCH. It is proposes that the multiple PUSCH should be scheduled in subsequent slots and thus k2(n) corresponding to k2 of the n-th PUSCH, n>1, the value k2(n) is set to k2(n-1)+1. It is not clear to the Rapporteur whether this is an acceptable restriction for uplink scheduling and thus will be good to verify. Alternatively, explicit signaling of each k2 can be considered.

Question C7: If multiple PUSCHs are configured per PDCCH, do you agree that k2(n) corresponding to k2 of the n-th PUSCH, n>1, the value k2(n) should be set to k2(n-1)+1?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | No | We think explicit signaling of each K2 should be configured |
| Apple | No | Similar thoughts as ZTE, should be explicitly configured. |
| LGE | No | Explicit signalling is preferred. |
| Intel |  | Either approach is fine with us |
| Nokia | No | assuming that k2 is the scheduling delay for PUSCH: DCI reception à PUSCH transmission) RAN1 has agreed that both multi-PDSCH and multi-PUSCH should support allocation of multiple PxSCH to non-contiguous slots - explicit configuration is needed |
| Huawei, HiSilicon |  | The explicit signaling of each k2 should be considered. Need futher check according to Ericsson proposed revision of the proposal. |
| Ericsson | Yes | This applies to the UE in case K2(n) is not configured/absent. This is aligned with RAN1 assumption. The proposal was not accurate; it should state: “If k2(n) is absent, k2(n) corresponding to k2 of the n-th PUSCH, n>1, the value k2(n) should be set to k2(n-1)+1.” Otherwise, the configured value would be used. |
| OPPO | No | Each k2 can be signalled in an explicit way. |
| Samsung | - | If the question is about the "default value" for k2(n) as described in R2-2200942, we are fine with the proposal. Otherwise, each K2 should be configured explicltly. |
| vivo | No strong view | We can follow the majority view. |

Summary: 5 out of 11 companies think k2 should always be signaled explicitly. The proponent clarified that the proposal was only for the case when k2(n) is not signaled. Since the question may have been confusing, this can be discussed online.

Proposal C7: Discuss whether k2(n) should always be signaled vs the alternative proposal “If k2(n) is absent, k2(n) corresponding to k2 of the n-th PUSCH, n>1, the value k2(n) should be set to k2(n-1)+1”.

R2-2200942 also proposes to introduce a new IE UL-AccessConfigListDCI-1-1-r17 which contains only list elements that would actually be used for FR2-2. This seems to be in contrast to what RAN1 has captured in [2] that the existing IEs can be used. Similar discussion also applies to ul-AccessConfigListDCI-0-1.

Question C8: Do you support introducing new Rel-17 IEs for UL-AccessConfigListDCI-0-1 and UL-AccessConfigListDCI-1-1 instead of re-using Rel-16 versions?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Apple | Ok |  |
| LGE | Yes | This is a clean approach. |
| Intel | Yes |  |
| Nokia |  | It can be simpler to follow RAN1 understanding and just limit the possibilities for FR2-2 in field/IE description |
| Huawei, HiSilicon | No | Adding extra description for FR2-2 in existing IEs is preferred. RAN1 has already introduced new Table 7.3.1.2.2-6A (38.212) to describe allowed three entries for FR2-2, so seems the current RAN1 added description is quite clear. No sure why we need to do extra work with creating new Rel-17 IEs. |
| Ericsson | yes | It is a cleaner solution compared to reusing R16 version |
| OPPO | No strong view | We think we can follow what RAN1 suggesteed, but since RAN2 is the responsible WG for the ASN.1, we think RAN2 can do the clean way. |
| Samsung | - | No strong view but we are fine to introduce the new Rel-17 IE, which is clearner approach than reusing the Rel-16 one. |
| vivo | Yes | With the new Rel-17 IE, we can clearly know it is used for FR2-2. |

Summary: 2 companies prefer to follow the RAN1 suggestion and one pointed out that this was already captured in RAN1 specifications. 2 companies are neutral while 6 companies prefer to introduce a new Rel-17 IE. We can go with the majority view, assuming RAN1 will be fine with this.

Proposal C8: New Rel-17 IEs for UL-AccessConfigListDCI-0-1 and UL-AccessConfigListDCI-1-1 are introduced. This does not follow the RAN1 agreement to re-use Rel-16 versions and thus may need to be confirmed by RAN1.

## RACH Parameters

In RAN1 #107, the following agreement was made regarding extended RAR window for both 4-step and 2-step RACH.

Conclusion: For FR2-2, support the same mechanism as in Rel-16 for extended RAR window for both 4-step and 2-step RACH.

R2-2201033 proposes adding new values for ra-ResponseWindow since the ones in Rel-16 were based on a maximum of 60kHz SCS for NR-U 4-step RACH and a maximum of 120kHz SCS for 2-step RACH.

Based on the slot calculation in the paper, they propose to add the following for FR2-2 shared spectrum:

ra-ResponseWindow-r17            ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} OPTIONAL  -- Need R

Question D1: Do you agree adding the parameter ra-ResponseWindow-r17 with the above values for 4-step RACH for operation in FR2-2 shared spectrum?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | Yes (proponent) |  |
| LGE | Yes |  |
| Intel | Yes |  |
| Nokia | OK | No strong need but fine to add |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes | For the SCS of 480 and 960kHz, the new added values have the same absolute time length as 120kHz in R16. |
| OPPO | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |

Summary: All companies support the proposal.

Proposal D1: A new parameter ra-ResponseWindow-r17 with the value ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} is introduced for 4-step RACH for operation in FR2-2 shared spectrum.

Similarly, for 2-step RACH, the above paper proposes a new IE:

msgB-ResponseWindow-r17                 ENUMERATED {sl640, sl960, sl1280, sl1920, sl2560}    OPTIONAL -- Need R

Question D2: Do you agree adding the parameter msgB-ResponseWindow-r17 with the above values for 2-step RACH?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | Yes (proponent) |  |
| LGE | Yes |  |
| Intel | Yes |  |
| Nokia | OK | No strong need but fine to add |
| Huawei, HiSilicon | Yes | We can add further values once RAN1 agrees. |
| Ericsson | Yes | For the SCS of 480 and 960kHz, the new added values have the same absolute time length as 120kHz in R16. |
| OPPO | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |

Summary: All companies support the proposal.

Proposal D2: A new parameter msgB-ResponseWindow-r17 with the value ENUMERATED {sl640, sl960, sl1280, sl1920, sl2560} is introduced for 2-step RACH for operation in FR2-2.

## RAN2 Parameters

The new SCS and slot durations can also impact several parameters used in RAN2 procedures and in particular LCP and UAI.

R2-2201682 proposes to discuss whether new values shall be added to maxPUSCH-Duration, e.g. 0.0313ms, 0.0156ms, 0.01ms, etc. This parameter is used in LCP restriction. Note that one slot equals to 0.03125ms and 0.0156ms for 480 kHz and 960 kHz, respectively.

Question E1: Do you agree adding new values to maxPUSCH-Duration due to the new SCS values?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | Yes |  |
| LGE | Yes |  |
| Intel | No strong view |  |
| Nokia | No strong view | Does not seem to be critical change. |
| Huawei, HiSilicon | Yes (proponent) | They would be straightforward additions. |
| Ericsson | OK |  |
| OPPO | Yes |  |
| Samsung | No strong view, but | The maxPUSCH-Duration is introduced to limit the maximum PUSCH duration of the data transmission for some URLLC service. Thus, we can’t see the strong motivation to introduce new values for maxPUSCH-Duration without any new URLLC service requirement. |
| vivo | Yes |  |

Summary: All companies either support the proposal or do not have a strong view. Thus, it seems possible to agree.

Proposal E1: New values, e.g. 0.0313ms, 0.0156ms, 0.01ms, are added to maxPUSCH-Duration for FR2-2.

There are also several parameters in UAI with dependency to the SCS and bandwidth. R2-2200733 suggests that new fields/values are needed for the following power saving parameters:

MaxBW-Preference-r17

MaxMIMO-LayerPreference-r17

MinSchedulingOffsetPreference-v17xy to report higher preferred K0/K2 values also for 480/960 kHz SCS

R2-2200733 also proposes a similar exercise for overheating parameters.

As a first step, we can make an agreement whether to introduce new IEs or values in UAI parameters related to power savings and overheating. Then, the actual Ies and values can be agreed in the discussion of the running CR.

Question E2: Do you agree to adding new values for IEs in UAI power saving and overheating parameters to reflect the new SCS, K0/K2, and bandwidth sizes for FR2-2?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | Yes |  |
| LGE | See comment | UAI may not be essential to support RAN1 design for 71 GHz, but overheating may be essential to protact a heating problem on a mobile device. |
| Intel | Yes | We have agreed to differentiate UAI power saving for BW and MIMO. So it seems logical to extend this. For overheating parameters, we need to increase the BW to handle the larger BW for SCS480 and 960 |
| Nokia | Fine |  |
| Huawei, HiSilicon |  | Open to discuss |
| Ericsson | See comment | We are ok to the changes for differentiation UAI power saving for BW and MIMO, which is according to RAN2 agreements. But for schedulingoffset and overheadting parameters, RAN2 has not made any agreement, they are not critical for 71GHz. RAN2 needs further discussion. Or better to be handled by other WIs (e.g., power saving or overheadting Wis). |
| OPPO | Yes |  |
| Samsung | Yes (Proponent) | We agree that RAN2 can have further discussion for schedulingoffset and overheating parameters. However, if we do not make any agreement on this, there is no way for the UE to report its preference for FR2-2 operation, which can lead to severe problem especially for overheating case. |
| vivo | Yes |  |

Summary: No company is agains the proposal so it seems possible to agree.

Proposal E2: New values are added to IEs in UAI power saving and overheating parameters to reflect the new SCS, K0/K2, and bandwidth sizes for FR2-2.

RAN2 has previously agreed that CG autonomous retransmission will be configurable. Based on this and also the optionality of LBT in FR2-2, R2-2201032 proposes that cg-RetransmissionTimer should be optionally configured for operation in the shared spectrum in FR2-2.

Question E3: Do you agree that cg-RetransmissionTimer should be optionally configured for operation in FR2-2 shared spectrum?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | Yes |  |
| LGE | Yes |  |
| Intel | Yes | In Rel-17 IIoT discussion, for UCE, our understanding is that cg-RetransmissionTimer is already optional in shared spectrum while in UCE. This can be extended to the case for the no LBT mode |
| Nokia | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes | Could be conditional mandatory if channelAccessMode2 is enabled |
| OPPO | Yes |  |
| Saumsung | Yes | Agree with the motivation. With assumption that the contention would not be severe in FR2-2 with directional LBT, the autonomous retransmission might not need to be mandatory. |
| vivo | Yes |  |

Summary: There is consensus to make the IE optional.

Proposal E3: cg-RetransmissionTimer is optionally configured for operation in FR2-2 shared spectrum.

R2-2201284 proposes to introduce new periodicity and time offset values for CG and SR and new periodicity values for SPS in FR2-2. The existing parameters are integers and in number of symbols or slots. Therefore, the paper argues that new values are needed to have the same absolute periodicity and offset values.

Question E4: Do you agree that new periodicity and offset values are needed for Configured Grant for operation in FR2-2?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | Yes | Besides the parameters mentioned, some new values are needed for cg-minDFI-Delay-r16 in ConfiguredGrantConfig. |
| LGE | Yes |  |
| Intel | Yes |  |
| Nokia | Fine | but not critical |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes | but, we need to keep in mind that, changes may extend/introduce new shorter absolute values, which needs to confirmed by RAN1. So, RAN2 can send a LS to RAN1 of RAN2 agreements on these parameters, to check if the extended shorter absolute values are ok to RAN1. |
| OPPO | Yes |  |
| Samsung | Yes | Even with the reduced symbol length with higher SCS, the mixmum periodicity of CG should be able to be maintained as before. |
| vivo | Yes (Proponent) |  |

Summary: All companies are fine with the proposal.

Proposal E4: New periodicity and offset values corresponding to the existing absolute periodicity and offset are introduced for Configured Grant in FR2-2.

Question E5: Do you agree that new periodicity and offset values are needed for Scheduling Request for operation in FR2-2?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | Yes |  |
| LGE | Yes |  |
| Intel | Yes |  |
| Nokia | Fine | but not critical |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes | but, we need to keep in mind that, changes may extend/introduce new shorter absolute values, which needs to confirmed by RAN1. So, RAN2 can send a LS to RAN1 of RAN2 agreements on these parameters, to check if the extended shorter absolute values are ok to RAN1. |
| OPPO | Yes |  |
| Samsung | Yes |  |
| vivo | Yes (Proponent) |  |

Summary: All companies are fine with the proposal. One company pointed out the shorter absolute values should be confirmed by RAN1. However, the proposals in R2-2201284 are for keeping the same absolute values.

Proposal E5: New periodicity and offset values corresponding to the existing absolute periodicity and offset are introduced for Scheduling Request in FR2-2.

Question E6: Do you agree that new periodicity values are needed for SPS for operation in FR2-2?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | Yes |  |
| LGE | Yes |  |
| Intel | Yes |  |
| Nokia | Fine | not critical though |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes | but, we need to keep in mind that, changes may extend/introduce new shorter absolute values, which needs to confirmed by RAN1. So, RAN2 can send a LS to RAN1 of RAN2 agreements on these parameters, to check if the extended shorter absolute values are ok to RAN1. |
| OPPO | Yes |  |
| Samsung | Yes |  |
| vivo | Yes (Proponent) |  |

Summary: All companies are fine with the proposal. One company pointed out the shorter absolute values should be confirmed by RAN1. However, the proposals in R2-2201284 are for keeping the same absolute values

Proposal E6: New periodicity values corresponding to the existing absolute periodicities are introduced for SPS in FR2-2.

R2-2201284 also discusses secondary DRX, which was introduced in Rel-16 NR for FR1-FR2 CA, where the FR2 serving cells of can have different DRX parameters (i.e. drx-onDurationTimer and drx-InactivityTimer).

R2-2201284 argues that the same justification and benefit for this feature is also applicable to FR2-2 and thus it should be supported as such.

Question E7: Do you agree that Secondary DRX group can be supported for FR1/FR2-2 CA?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Ok |  |
| ZTE | - | No strong view |
| LGE | Yes |  |
| Intel | - | Not sure the need to explicitly define it since the support of secondary DRX group is not FRx differentiated. |
| Nokia | OK – see comment | Secondary DRX is not FRx differentiated - so no need to define anywhere separately |
| Huawei, HiSilicon |  | Agree with above “no FRx differntiated” comments. |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Samsung | - | Agree with the comment from Intel and Nokia. |
| vivo | Yes (Proponent) |  |

Summary: No company is against the proposal. It was pointed out that secondary DRX does not have FRx differentiated so it is not necessary to explicitly define this.

Proposal E7: Secondary DRX group is supported for FR1/FR2-2 CA. FFS if any new texts in the specifications are necessary.

For DRX timers, RAN2#116 agreed to “to keep the current DRX timer values for now, but it can be revisited for performance optimization after high priority issues are resolved.”

However, R2-2201682 argues that the legacy values are not optimal and suggests “for example, up to 224 can be defined for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL”

Question E8: Do you support introducing new values for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL for operation in FR2-2?

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | No strong view |  |
| Apple | No | We do not see the need as of now. |
| LGE | No | As addressed by the rapporteur, this is arelady discussed in the last RAN2 meeting. Considering that the situation is not changed, we think that this is a small optimization and the RAN2 agreement of RAN2#116 should be respected. |
| Intel | No strong view |  |
| Nokia | No | No need |
| Huawei, HiSilicon | Yes (proponent) | We are fine that this issue “can be revisited for performance optimization after high priority issues are resolved”. We want to remind that there is tangible power saving gain with perhaps minimum discussion/change. The RTT timer values are counted with number of symbol length. Without higher “maximum number” for FR2-2, UE in FR2-2 needs to wake up earlier unnecessarily. |
| Ericsson | Yes | The timer is set with the value in number of symbols, it would be beneficial to extende the value for the SCS of 480 and 960 kHz. |
| OPPO | Yes |  |
| Samsung | No strong view | If this can be easily agreed with all companies, we can also support it. Otherwise, we should not waste more time for this issue during online discussion according to the previous agreement (i.e., keep the current DRX timer values for now). |
| vivo | No | We prefer not to revert the agreement. |

Summary: 3 companies support the proposal while 3 companies do not have a strong view and 4 companies are against. Given this, no changes can be made to the RAN2#116 agreement.

RAN2#116 agreed to “to keep the current DRX timer values for now, but it can be revisited for performance optimization after high priority issues are resolved

Proposal E8: Do not change the RAN2#116 agreement “to keep the current DRX timer values for now, but it can be revisited for performance optimization after high priority issues are resolved”.

# Conclusion

Based on the discussion and the feedback from the companies above, the following are proposed for the progress in running FR2-2 RRC CR:

Proposal A1: RAN2 does not agree to using the spare bit in MIB for the signaling of FR2-2 QCL assumptions for SSB. Respond to RAN1 LS accordingly.

Proposal A2: As a baseline, the legacy MIB is also used for FR2-2.

Proposal A3: In RAN2 reply LS to RAN1, do not include any suggestions for changing the QCL assumptions for FR2-2.

Proposal A4: channelAccessMode2 is signaled as ENUMERATED {enabled}. This implies that the UE can not distinguish between licensed spectrum and shared spectrum without LBT.

Proposal B1: Add text with the new SCS values in the field description of the parameters listed in Table 1 in R2-2201033

Proposal C1: The parameter enableTimeDomainHARQ-BundlingType1-r17 is introduced in ServingCellConfig with the value “ENUMERATED {enabled}”. FFS if the name can be shortened.

Proposal C2: maxNrofMultiplePDSCHs-r17 is defined in pdsch-TimeDomainAllocationListForMultiPDSCH-r17 with the value 8.

Proposal C3: No restrictions are captured in RRC for pdsch-TimeDomainAllocationListForMultiPDSCH.

Proposal C4: The following are agreed for signaling of PDSCH TDRA:

- The new PDSCH-TimeDomainResourceAllocation-r17 IE can be configured with either PDSCH repetition or multiple PDSCH.

- Introduce the field pdsch-TimeDomainAllocationListDCI-1-2-r17 and the field pdsch-TimeDomainAllocationList-r17 so that PDSCH repetitions can be used with the new k0 value range.

Proposal C5: Introduce the field pusch-TimeDomainAllocationListDCI-1-2-r17 and the field pusch-TimeDomainAllocationList-r17 so that PUSCH repetition can be used with the new k2 value range.

Proposal C6: The IE pusch-TimeDomainAllocationListForMultiPUSCH-r17 is configured with up to 16 list elements.

Proposal C7: Discuss whether k2(n) should always be signaled vs the alternative proposal “If k2(n) is absent, k2(n) corresponding to k2 of the n-th PUSCH, n>1, the value k2(n) should be set to k2(n-1)+1”.
Proposal C8: New Rel-17 IEs for UL-AccessConfigListDCI-0-1 and UL-AccessConfigListDCI-1-1 are introduced. This does not follow the RAN1 agreement to re-use Rel-16 versions and thus may need to be confirmed by RAN1.

Proposal D1: A new parameter ra-ResponseWindow-r17 with the value ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} is introduced for 4-step RACH for operation in FR2-2 shared spectrum.

Proposal D2: A new parameter msgB-ResponseWindow-r17 with the value ENUMERATED {sl640, sl960, sl1280, sl1920, sl2560} is introduced for 2-step RACH for operation in FR2-2.

Proposal E1: New values, e.g. 0.0313ms, 0.0156ms, 0.01ms, are added to maxPUSCH-Duration for FR2-2.

Proposal E2: New values are added to IEs in UAI power saving and overheating parameters to reflect the new SCS, K0/K2, and bandwidth sizes for FR2-2.

Proposal E3: cg-RetransmissionTimer is optionally configured for operation in FR2-2 shared spectrum.

Proposal E4: New periodicity and offset values corresponding to the existing absolute periodicity and offset are introduced for Configured Grant in FR2-2.

Proposal E5: New periodicity and offset values corresponding to the existing absolute periodicity and offset are introduced for Scheduling Request in FR2-2.

Proposal E6: New periodicity values corresponding to the existing absolute periodicities are introduced for SPS in FR2-2.

Proposal E7: Secondary DRX group is supported for FR1/FR2-2 CA. FFS if any new texts in the specifications are necessary.

Proposal E8: Do not change the RAN2#116 agreement “to keep the current DRX timer values for now, but it can be revisited for performance optimization after high priority issues are resolved”.

# References

[1] R2-2200076, LS on initial access for 60 GHz, Intel

[2] R2-2200095, LS on updated  LTE and NR higher-layers parameter list, Ericsson