3GPP TSG-RAN WG2 #116bis-e	draftTdoc R2-2201738

Electronic meeting, Jan 17th – 25th January 2022

Agenda Item:	8.12.2.2

Source:	Ericsson (Rapporteur)

Title:	NCD-SSB and Initial BWP aspects

Document for:	Discussion, Decision

1	Introduction

RAN1 sent an LS to RAN2 and RAN4 on use of NCD-SSB instead of CD-SSB in R1-2112802 and asked for feedback from RAN2 and RAN4 on whether the working assumptions are acceptable from RAN2 and RAN4 perspectives, respectively.

In RAN2#116bis-e, a report is prepared to summarize the Tdocs listed below:

- R2-2200190  Discussions on RedCap-specific BWPs	Qualcomm Incorporated

- R2-2200287  Open issues on Early identification, camping restrictions and NCD-SSB	Intel Corporation

- R2-2200401 BWP configuration for RedCap UE	DENSO CORPORATION

- R2-2200554 Identification and access restriction of RedCap UE, and NCD-SSB related issuesHuawei, HiSilicon

- R2-2200597 Issues on NCD SSB, identification and access for RedCap	vivo, Guangdong Genius

- R2-2200608 Discussion on separate initial BWP and NCD-SSB for RedCap UE	ZTE Corporation, Sanechips

- R2-2200830 Using NCD-SSB or CSI-RS in DL BWPs for RedCap UEs		Ericsson

- R2-2200831 [DRAFT] Reply LS on the use of NCD-SSB or CSI-RS in DL BWPs for RedCap UEs Ericsson

- R2-2200862 Discussion on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE	CMCC

- R2-2201113 RedCap UE power-saving aspects at cell re-selection		Apple

- R2-2201461 Aspects related to use of NCD-SSB	MediaTek Inc.

The summary was provided in R2-2201732 and during the online discussion, the following agreements were made:

Agreements:

1.	A RedCap UE in idle/inactive mode monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB and performs cell (re-)selection and measurements on the CD-SSB

2.	If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

In this document, we continue the discussion based on the agreements above and the list of Tdocs provided above with the intention to formulate a list of proposals that are agreeable and a list of proposals that require further discussion during the related online session.

2	Discussion

2.1	RRC Idle/Inactive mode

Q 2.1.1 If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, do you think that measurements should be based on CD-SSB for RACH resource selection? Please elaborate your reply. If you agree comment on whether field description of rach-ConfigCommon should be updated accordingly.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | The field description update can be discussed in the running CR. |

Summary – Q 2.1.1

TBD

…

Q 2.1.2 If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, do you think that PDCCH-ConfigCommon of the separate initial DL BWP should include common search space configuration for RAR but not for paging, SIB1 and other SIBs? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

Summary – Q 2.1.2

TBD

Q 2.1.3 If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, do you think

- (Option 1) it should be up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before a Msg1/A retransmission, or

- (Option 2) UE should always perform new RSRP measurement in a DL BWP associated with CD-SSB?

Please elaborate your reply.

| Company | Option (1/2/Other) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option1 | If there is no clear RAN2 majority, maybe we can consult/wait for R4 on this issue. |

Summary – Q 2.1.3

TBD

Q 2.1.4 Do you think RedCap-specific two-step RACH (if configured) and four-step RACH should always configured in the same BWP? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | It seems quite clear from the agreement we made:” 2. If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.” |

Summary – Q 2.1.4

TBD

Q 2.1.5 Please provide your comments here if you think there are any other issues that should be discussed for RedCap UEs in idle/inactive mode.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | To save SIB1 size, the RedCap-specific initial BWP configurations (introduced by RAN1) should be only configured with the delta parameters compared to the legacy one.( i.e. use the same value as legacy if absent). |

Summary – Q 2.1.5

TBD

2.2	RRC Connected mode

Q 2.2.1 In RRC connected mode, do you think it should be possible to configure NCD-SSB for a RedCap UE in dedicated DL BWP? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | But, it can be further discussed whether the new IE is added in BWP-DownlinkDedicated or BWP-DownlinkCommon. |

Summary – Q 2.2.1

TBD

Q 2.2.2 For connected mode operation do you think NCD-SSB should have the same properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) as the corresponding CD-SSB? Please elaborate your reply. Note that the question is about “properties”, not “configuration” (please see the next question regarding the discussion on configuration).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes, but | But see our comments in below question. |

Summary – Q 2.2.2

TBD

Q 2.2.3 For connected mode operation do you think it should be possible for the network to provide absoluteFrequencySSB, ssb-PositionsInBurst, and ssb-periodicity explicitly for NCD-SSB, i.e., other properties such as PCI, ssb-PBCH-BlockPower are configured with the same values from serving cell's CD-SSB? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Not all | - ssb-PositionsInBurst should be same. - ssb-PBCH-BlockPower should be decided by R1/R4. - ssb-periodicity: In addition to the periodicty, the time offset of NCD-SSB should also be able to configure different values. If multile NCD-SSBs always have the same time offset with CD-SSB (even with different periodicty), it means the transmisison of NDC-SSBs may occur in the same time slot as CD-SSB. From gNB side, transmitting many SSBs at the same time will cause unaffordable power, which could be one blocking issue. We can use the similar IE like periodicityAndOffset in SSB-MTC |

Summary – Q 2.2.3

TBD

Q 2.2.4 For connected mode operation do you think periodicity of NCD-SSB shall be not less than the periodicity of serving cell’s CD-SSB? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

Summary – Q 2.2.4

TBD

Q 2.2.5 For connected mode operation if NCD-SSB is configured in a dedicated DL BWP, do you think RedCap UE should assume that the “SSB” in QCL-Info IE and “ssb-Index” in RadioLinkMonitoringRS IE refer to the beam with the same index in the NCD-SSB configured in that BWP? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

Summary – Q 2.2.5

TBD

Q 2.2.6 For connected mode operation if NCD-SSB is configured in a dedicated DL BWP whose paired UL BWP is configured with RACH-ConfigDedicated, RACH-ConfigCommon or BeamFailureRecovery Config, do you think the SSB in that RACH configuration (e.g., in CFRA-SSB-Resource IE or in PRACH-ResourceDedicatedBFR IE) should refer to the NCD-SSB configured in that DL BWP? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | NCD-SSB can be used for RO selection. |

Summary – Q 2.2.6

TBD

Q 2.2.7 For connected mode operation do you think neighbour cell measurements based on NCD-SSB should be supported for RedCap UEs? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | 1st, there is no guarantee that all the neighbor cells have NCD-SSB and all the NCD-SSB of neighbor cells will be in the same absoluteFrequencySSB. So, in normal cases, UE has to switch to CD-SSB for some measurement of neighbor cells. In that case, there is no benefit of power saving to support this NCD-SSB based measurement, if CD-SSB based measurement is anyway required. 2nd, it is not clear how the UE maintain the intra-frequency cells list, if the NCD-SSB is on the same frequency but the CD-SSB is on different frequency of one neigbor cell, especially when UE performs BWP switch. |

Summary – Q 2.2.7

TBD

Q 2.2.8 For serving cell measurement based on NCD-SSB in connected mode; do you think

- (Option 1) MeasObjectId should be configured for each NCD-SSB, or

- (Option 2) MeasObjectNR is extended to include ssbFrequency for each NCD-SSB?

Please elaborate your reply.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1, but no strong view | The intention is to indicate the ssbFrequency for each NCD-SSB of BWP. In case of BWP switch, for serving cell measurement based on NCD-SSB, UE’s serving cell measurement object is the ssbFrequency associated with the NCD-SSB of its active BWP. Therefore, the RRC measurement configuration should provide the ssbFrequency of all possible NCD-SSB. |

Summary – Q 2.2.8

TBD

Q 2.2.9 Do you think RAN1 working assumption regarding the use of CSI-RS in connected mode is acceptable from RAN2 standpoint? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes, but up to R4 | From RAN2 perspective, we can say it is possible from signaling perspective. The final decision should be up to R4 reply. |

Summary – Q 2.2.9

TBD

Q 2.2.10 Do you think a RedCap UE, which does not support CSI-RS, should be able to report “Not need NCD-SSB” as an optional UE capability? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | As agreed in RAN1. |

Summary – Q 2.2.10

TBD

Q 2.2.11 Do you think it should be possible to use NCD-SSB to trigger the handover procedure, i.e., whether SSB indicated in absoluteFrequencySSB of frequencyInfo-DL IE in handover command must be CD-SSB? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | We can postpone this and focus on other essential issues. Before making the decision, we need to analyses the whole spec impacts, e.g. whether SIB1 is included in HO command, whether DL syncnization is on NCD-SSB. Also, we have not decided on the neighbor cell measurement based on NCD-SSB, which is the pre-condition of this proposal. |

Summary – Q 2.2.11

TBD

Q 2.2.12 Do you think a non-RedCap UE should be able to use NCD-SSB instead of CD-SSB with an optional capability? Please elaborate your reply.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | The motivation of supporting NCD-SSB is only for the UE with narrow bandwidth, e.g. for load balancing purpose among BWPs. |

Summary – Q 2.2.12

TBD

Q 2.2.13 Please provide your comments here if you think there are any other issues that should be discussed for RedCap UEs in connected mode.

| Company | Comments |
| --- | --- |

Summary – Q 2.2.13

TBD

3	Conclusion

Based on the discussion above rapporteur suggests a discussion on the following proposals:

Proposal 1	…

References

R2-2200190 Discussions on RedCap-specific BWPs	Qualcomm Incorporated

R2-2200287 Open issues on Early identification, camping restrictions and NCD-SSB	Intel Corporation

R2-2200401 BWP configuration for RedCap UE	DENSO CORPORATION

R2-2200554 Identification and access restriction of RedCap UE, and NCD-SSB related issues Huawei, HiSilicon

R2-2200597 Remaining issues on NCD SSB, identification and access for RedCap	vivo, Guangdong Genius

R2-2200608 Discussion on separate initial BWP and NCD-SSB for RedCap UE	ZTE Corporation, Sanechips

R2-2200830 Using NCD-SSB or CSI-RS in DL BWPs for RedCap UEs		Ericsson

R2-2200831 [DRAFT] Reply LS on the use of NCD-SSB or CSI-RS in DL BWPs for RedCap UEs Ericsson

R2-2200862 Discussion on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE	CMCC

R2-2201113 RedCap UE power-saving aspects at cell re-selection		Apple

R2-2201461 Aspects related to use of NCD-SSB	MediaTek Inc.