3GPP TSG-RAN WG2 #116bis-e	R2-2201701

Electronic Meeting, 17 – 25 January, 2022

Agenda Item:	8.2.2.1

Source:	Samsung

Title:	[AT116bis-e][221][DCCA] MAC aspects (Samsung)

WID/SID:	LTE_NR_DC_enh2-Core

Release:	Rel-17

Document for:	Discussion and Decision

1	Introduction

This document is to handle the following email discussion:

[AT116bis-e][221][DCCA] MAC aspects (Samsung)

Scope: Discuss the following topics: 1) How to define the "partial MAC reset" for SCG deactivation? 2) What are the MAC actions SCG activation (e.g. is PHR triggered, are some variables reset, etc.)? 3) Other MAC aspects related to SCG deactivated state (e.g. CSI-RS reporting)

Intended outcome: Discussion summary in R2-2201701.

Deadline: Deadline 3

Deadline 3 (discussions for 2nd week Mon/Tue online):

- Comment deadline: Thursday W1, 1600 UTC (for collecting views)

- Rapporteur proposals: Friday W1, 0900 UTC (proposed resolution of issues)

- Document deadline: Monday W2, 1200 UTC (report or agreed CRs)

- No extensions to this deadline for regular discussions. Discussions handling CRs may continue to 1-week email (based on chair decision).

The following documents are to be treated in this email discussion:

8.2.2.1	Deactivation of SCG and UE behaviour in deactivated SCG

By Email [221] (4+1)

Partial MAC reset for SCG deactivation:

R2-2200601	Partial MAC reset upon SCG deactivation	Samsung Electronics Polska	discussion	LTE_NR_DC_enh2-Core

R2-2201416	Partial MAC reset upon SCG deactivation	DENSO CORPORATION	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201075	UE behavior in deactivated SCG and SCG deactivation	Qualcomm Incorporated	discussion	Rel-17

(only P10-P14 relevant for MAC)

R2-2201319	Remaining issues for MAC procedure in deactivated SCG	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

Does MAC allow CSI-RS reporting when SCG is deactivated?:

R2-2201296	CSI-RS reporting for deactivated SCG	MediaTek Inc.	discussion

8.2.2.2	Activation of deactivated SCG

R2-2201562	Efficient SCG activation	Ericsson	discussion	LTE_NR_DC_enh2-Core

Only P11 is discussed online (P4-8 can be discussed under by [222] and P1,2,12,13 can be discussed under [221])

By Email [221] (1)

PHR reporting for deactivated SCG and triggering upon SCG activation:

R2-2200584	PHR issues for SCG activation	Samsung Electronics Polska	discussion	LTE_NR_DC_enh2-Core

By Email ([221] and [222], depending on proposals)

UE-initiated SCG activation:

R2-2200542	Futher discussion on UE initiated SCG fast activation	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2110909

R2-2200605	Activation of deactivated SCG	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2200637	Discussion on activation of deactivated SCG	Spreadtrum Communications	discussion	Rel-17

R2-2200649	UP details of deactivated SCG activation	Transsion Holdings	discussion	Rel-17

R2-2200772	Discussion on SCG activation	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2200882	Open issues in activation of SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2200895	Remaining issues on SCG (de)activation	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201060	Activation of deactivated SCG	Qualcomm Incorporated	discussion	Rel-17

R2-2201249	Considerations on Activation of Deactivated SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201362	Discussion on SCG activation and deacitvation	LG Electronics Inc.	discussion	LTE_NR_DC_enh2-Core

R2-2201393	Activation of deactivated SCG	vivo	discussion	LTE_NR_DC_enh2-Core

R2-2201431	SCG/split bearer handling upon SCG deactivation and SCell state upon SCG activation	Sharp	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201538	Conditional reconfiguration execution while SCG is deactivated	Sharp	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201641	Activation of SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

8.2.2.3	Other aspects of SCG activation/deactivation

By Email ([221], [222] or [223], depending on proposals) (4)

Other aspects of SCG (de)activation:

R2-2201073	Other aspects of SCG activation/deactivation	Qualcomm Incorporated	discussion	Rel-17

R2-2201317	Deactivation of SCG	LG Electronics Finland	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201333	Discussion on SCG (de)activation	NTT DOCOMO, INC.	discussion	Rel-17

R2-2201575	Rest issues of SCG Activation	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2111018

# 2 Contact Information

The rapporteur encourages the delegates who provide input to fill their contact information in the below table:

| Company | Contact: Name (E-mail) |
| --- | --- |
| Samsung (Donggun Kim) | s_dg.kim@samsung.com |

3	Discussion

3.1 MAC aspects

Firstly, we can discuss the details of partial MAC reset.

The first action to be discussed is

1>	initialize Bj for each logical channel to zero;

This action can be done upon SCG activation for fairness among logical channels. On the other hand, given that the network can release and add logical channel configuration (e.g. RLC bearer configuration) to initialize Bj values according to LCP procedure, it can be up to network implementation upon SCG activation, i.e. the network can initialize them by using RRCReconfiguration including SCG activation indication, if needed. In this regard, the action can be done as a part of partial MAC reset upon SCG deactivation since no critical problem would be foreseen.

- Option 1. Initialize Bj for each logical channel to zero upon SCG activation as a separate procedure.

- Option 2. Initialize Bj for each logical channel to zero upon SCG deactivation as a part of partial MAC reset.

Q1. Which option do you prefer if you agree that UE should do this action related to MAC reset for SCG activation/deactivation? or do you have any other suggestion?

| Company | Preferred option | Comments |
| --- | --- | --- |

The second actions to be discussed is

1>	stop (if running) all timers;

1>	consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2;

As RAN2 agreed, the only timer to be kept would be timeAlignmentTimers. So it seems straightforward to have the following action as a part of partial MAC reset upon SCG deactivation.

1>	stop (if running) all timers except timeAlignmentTimers;

Q2. Do you agree that UE should do this action as a part of partial MAC reset upon SCG deactivation? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |

The third action to be discussed is

1>	discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

This action is related to whether to allow dedicated RACH resources indicated before SCG activation indication (when going to the SCG deactivated state or while the SCG is deactivated). If it is allowed, then UE should keep CFRA resources. Otherwise, this action can be performed as a part of partial MAC reset upon SCG deactivation.

- Option 1. discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

- Option 2. Do not discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

Q3. Which option do you prefer if you agree that UE should do this action as a part of partial MAC reset upon SCG deactivation? or do you have any other suggestion?

| Company | Preferred option | Comments |
| --- | --- | --- |

The fourth action to be discussed is

1>	reset all BFI_COUNTERs;

RAN2 agreed to support BFD and RLM for deactivated PSCell. Therefore, the UE continues to perform BFD and RLM even after SCG deactivation, if configured. In this case, UE should not reset BFI_COUNTER for deactivated PSCell since UE still performs BFD and RLM for the same TCI state on the same BWP. However, UE should reset all BFI_COUNTERs if BFD and RLM are not configured upon SCG deactivation. In addition to this, the UE should reset BFI_COUNTER if the TCI state and BWP are changed and if BFD and RLM are configured.

Note that the legacy SCell deactivation does not reset BFI_COUNTER as well as beamFailureDetectionTimers, and BFD cannot be performed if SCell is deactivated. So, we can follow the legacy procedure for SCells because RAN2 already agreed to deactivate SCells of SCG for deactivated SCG.

Moreover, given that 38.321 already specified the initialization procedure for beam failure detection as shown below, UE will reset BFI_COUNTER if the TCI state and BWP are changed and if BFD and RLM are configured.

1>	if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell:

2>	set BFI_COUNTER to 0.

In this regard, the following action would be enough.

- reset all BFI_COUNTERs if BFD and RLM are not configured for deactivated SCG;

Q4. Do you agree that UE should do this action as a part of partial MAC reset upon SCG deactivation? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |

The next actions to be discussed are

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	stop, if any, ongoing Random Access procedure;

1>	flush Msg3 buffer;

1>	flush MSGA buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	cancel, if any, triggered Configured uplink grant confirmation;

1>	flush the soft buffers for all DL HARQ processes;

1>	for each DL HARQ process, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI.

The above actions can be performed as a part of partial MAC reset upon SCG deactivation.

Q5. Do you agree that UE should do these actions as a part of partial MAC reset upon SCG deactivation? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |

In legacy, the common understanding is that the corresponding BWP is deactivated when a SCell is deactivated. Based on this principle, we can apply the same principle to the BWP associated with PSCell.

Proposal. The BWP associated with PSCell is deactivated upon SCG deactivation.

Q6. Do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |

Regarding CSI-RS reporting of deactivated PSCell, sending CSI-RS reporting on the deactivated PSCell would just increase power consumption given that no data transmission is ongoing. Moreover, sending CSI reporting for the deactivated PSCell via MCG would request additional inter-node message design and may not worth it considering the inter-node delay. The basic PSCell quality could be maintained by RLM/BFD mechanism.

Proposal. CSI-RS reporting in the deactivated PSCell or for the deactivated PSCell is NOT supported.

Q7. Do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |

In RAN2#116e it was agreed that UE indicates via the MCG that it has UL data to send for SCG DRBs, FFS indication contents and format.

The data volume calculation procedure in the PDCP specs and the RLC specs are used for the buffer status reporting triggering in the MAC spec. The same procedures can be re-used for reporting UE buffer status but rather deliver to the RRC layer due to that the message is encoded at the RRC layer. For SCG DRBs, if the total amount of PDCP data volume and RLC data volume pending for initial transmission is larger than zero, then the UE can indicate it in an RRC message.

On the other hand, one indication could be enough given that UE can send BSR MAC CE after SCG activation as in legacy, i.e. no further optimization would be needed.

- Option 1. The UE indication of uplink data and the total data volume (e.g. long BSR MAC CE) can be included in UE Assistance Information message.

- Option 2. Only the UE indication of uplink data can be included in UE Assistance Information message. BSR can follow the legacy procedure.

Q8. Which option do you prefer? or do you have any other suggestion?

| Company | Preferred option | Comments |
| --- | --- | --- |

PSCell can be activated and deactivated based on SCG activation and deactivation indication from network. For deactivated PSCell, the UL BWP behavior would not include any uplink transmission, i.e. the uplink power control would not be needed and thus it seems reasonable not to include the PHR report for deactivated PSCell.

Proposal. For deactivated PSCell, PHR is not reported.

Note that this proposal would not have any impact on the MAC specification.

Q9. Do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |

In the legacy MAC specification, PHR is triggered upon addition of the PSCell, i.e. PSCell is newly added or changed. However, given that the feature of SCG activation/deactivation is introduced, we need to note that the uplink transmission would start after SCG activation from deactivated SCG. Therefore, a new PHR trigger would be beneficial to help the network manage fast uplink power control, i.e. PHR can be triggered in case that PSCell is activated. One can argue that it would be beneficial to trigger a PHR upon SCG deactivation as well but it would be the next step.

Proposal. PHR is triggered upon activation of the PSCell.

Q10. Do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |

If the proposal related to Q10 is agreeable, then the text proposal could be controversial. We need to note that RAN2 had the same discussion about whether to trigger a PHR for re-activation of SCell, several times before, i.e. whether to limit the PHR triggering to the case that SCell is activated from deactivated state. It has ended up with no such limitation. We can follow the legacy principle of SCell. On the other hand, such limitation would have no harm and thus could be acceptable based on the majority view.

- Option 1. Activation of the PSCell (the same as activation of SCell).

- Option 2. Activation of the PSCell from deactivated state.

Q11. If you say yes for Q10, which option do you prefer? or do you have any other suggestion?

| Company | Preferred option | Comments |
| --- | --- | --- |

If the proposal related to Q10 is agreeable, then it may have an impact on the legacy procedure because RAN2 agreed that SCG can be added/changed with activated state or deactivated state. If PSCell is added/changed with deactivated state, then the PHR doesn’t have to be triggered upon addition of PSCell. Hence, the following proposal could be beneficial.

Proposal. PHR is triggered upon addition of the PSCell (i.e. PSCell is newly added or changed with activated state).

Q12. If you say yes for Q10, do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |

Conclusion

TBD