3GPP TSG-RAN WG2 #116bis-e	R2-2201701

Electronic Meeting, 17 – 25 January, 2022

Agenda Item:	8.2.2.1

Source:	Samsung

Title:	[AT116bis-e][221][DCCA] MAC aspects (Samsung)

WID/SID:	LTE_NR_DC_enh2-Core

Release:	Rel-17

Document for:	Discussion and Decision

1	Introduction

This document is to handle the following email discussion:

[AT116bis-e][221][DCCA] MAC aspects (Samsung)

Scope: Discuss the following topics: 1) How to define the "partial MAC reset" for SCG deactivation? 2) What are the MAC actions SCG activation (e.g. is PHR triggered, are some variables reset, etc.)? 3) Other MAC aspects related to SCG deactivated state (e.g. CSI-RS reporting)

Intended outcome: Discussion summary in R2-2201701.

Deadline: Deadline 3

Deadline 3 (discussions for 2nd week Mon/Tue online):

- Comment deadline: Thursday W1, 1600 UTC (for collecting views)

- Rapporteur proposals: Friday W1, 0900 UTC (proposed resolution of issues)

- Document deadline: Monday W2, 1200 UTC (report or agreed CRs)

- No extensions to this deadline for regular discussions. Discussions handling CRs may continue to 1-week email (based on chair decision).

The following documents are to be treated in this email discussion:

8.2.2.1	Deactivation of SCG and UE behaviour in deactivated SCG

By Email [221] (4+1)

Partial MAC reset for SCG deactivation:

R2-2200601	Partial MAC reset upon SCG deactivation	Samsung Electronics Polska	discussion	LTE_NR_DC_enh2-Core

R2-2201416	Partial MAC reset upon SCG deactivation	DENSO CORPORATION	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201075	UE behavior in deactivated SCG and SCG deactivation	Qualcomm Incorporated	discussion	Rel-17

(only P10-P14 relevant for MAC)

R2-2201319	Remaining issues for MAC procedure in deactivated SCG	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

Does MAC allow CSI-RS reporting when SCG is deactivated?:

R2-2201296	CSI-RS reporting for deactivated SCG	MediaTek Inc.	discussion

8.2.2.2	Activation of deactivated SCG

R2-2201562	Efficient SCG activation	Ericsson	discussion	LTE_NR_DC_enh2-Core

Only P11 is discussed online (P4-8 can be discussed under by [222] and P1,2,12,13 can be discussed under [221])

By Email [221] (1)

PHR reporting for deactivated SCG and triggering upon SCG activation:

R2-2200584	PHR issues for SCG activation	Samsung Electronics Polska	discussion	LTE_NR_DC_enh2-Core

By Email ([221] and [222], depending on proposals)

UE-initiated SCG activation:

R2-2200542	Futher discussion on UE initiated SCG fast activation	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2110909

R2-2200605	Activation of deactivated SCG	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2200637	Discussion on activation of deactivated SCG	Spreadtrum Communications	discussion	Rel-17

R2-2200649	UP details of deactivated SCG activation	Transsion Holdings	discussion	Rel-17

R2-2200772	Discussion on SCG activation	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2200882	Open issues in activation of SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2200895	Remaining issues on SCG (de)activation	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201060	Activation of deactivated SCG	Qualcomm Incorporated	discussion	Rel-17

R2-2201249	Considerations on Activation of Deactivated SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201362	Discussion on SCG activation and deacitvation	LG Electronics Inc.	discussion	LTE_NR_DC_enh2-Core

R2-2201393	Activation of deactivated SCG	vivo	discussion	LTE_NR_DC_enh2-Core

R2-2201431	SCG/split bearer handling upon SCG deactivation and SCell state upon SCG activation	Sharp	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201538	Conditional reconfiguration execution while SCG is deactivated	Sharp	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201641	Activation of SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

8.2.2.3	Other aspects of SCG activation/deactivation

By Email ([221], [222] or [223], depending on proposals) (4)

Other aspects of SCG (de)activation:

R2-2201073	Other aspects of SCG activation/deactivation	Qualcomm Incorporated	discussion	Rel-17

R2-2201317	Deactivation of SCG	LG Electronics Finland	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2201333	Discussion on SCG (de)activation	NTT DOCOMO, INC.	discussion	Rel-17

R2-2201575	Rest issues of SCG Activation	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2111018

# 2 Contact Information

The rapporteur encourages the delegates who provide input to fill their contact information in the below table:

| Company | Contact: Name (E-mail) |
| --- | --- |
| Samsung (Donggun Kim) | s_dg.kim@samsung.com |
| Wenjuan Pu | wenjuan.pu@vivo.com |
| Apple | naveen.palle@apple.com |
| OPPO | wangshukun@oppo.com |
| Nokia | Jarkko.t.koskela@nokia.com |
| Futurewei | Jialinzou88@yahoo.com |
| Ericsson (Zhenhua Zou) | zhenhua.zou@ericsson.com |
| LG | San (geumsan.jo@lge.com) |

3	Discussion

3.1 MAC aspects

Firstly, we can discuss the details of partial MAC reset.

The first action to be discussed is

1>	initialize Bj for each logical channel to zero;

This action can be done upon SCG activation for fairness among logical channels. On the other hand, given that the network can release and add logical channel configuration (e.g. RLC bearer configuration) to initialize Bj values according to LCP procedure, it can be up to network implementation upon SCG activation, i.e. the network can initialize them by using RRCReconfiguration including SCG activation indication, if needed. In this regard, the action can be done as a part of partial MAC reset upon SCG deactivation since no critical problem would be foreseen.

- Option 1. Initialize Bj for each logical channel to zero upon SCG activation as a separate procedure.

- Option 2. Initialize Bj for each logical channel to zero upon SCG deactivation as a part of partial MAC reset.

Q1. Which option do you prefer if you agree that UE should do this action related to MAC reset for SCG activation/deactivation? or do you have any other suggestion?

| Company | Preferred option | Comments |
| --- | --- | --- |
| vivo | Option 2 | In the last RAN2 meeting, RAN2 has agreed that upon SCG deactivation, instruct the SCG MAC entity to perform partial MAC reset. It is reasonable to initialize Bj as a part of MAC reset. And since there is no SCG transmission during SCG deactivation, no need to initialize Bj again upon SCG activation. So, we prefer option 2. |
| Apple | Ok with Op2 |  |
| OPPO | Option 2 | In my understanding, the MAC reset behavior is same as legacy MAC reset behavior in section 5.12 except TAT timer and dedicated RACH resource. |
| Nokia | Option 1 | No reason to differentiate from normal MAC reset. It seems companies think option 2 is existing mac reset but that is not the case in our understanding |
| Futurewei | Option 1 | Option 1 is fine. |
| Ericsson | Option 1 | As Rapportuer explained, B_j should be re-set to the correct value (i.e, zero) upon SCG activation. Release and add logical channel may work, but it does not seem to capture the possiblity that the network can also re-establish the RLC entity. It is not our preference to force the network to perform release/add just to ensure B_j is set to the correct value. If RAN2 does not agree on option 1, it does not make sense either to initialize B_j at the SCG de-activation. |
| LG | Option 2 |  |
| Samsung | Option 2 | It would be better to avoid a separate procedure only for this since there is a way to reset Bj value by network configuration, if needed. |

The second actions to be discussed is

1>	stop (if running) all timers;

1>	consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2;

As RAN2 agreed, the only timer to be kept would be timeAlignmentTimers. So it seems straightforward to have the following action as a part of partial MAC reset upon SCG deactivation.

1>	stop (if running) all timers except timeAlignmentTimers;

Q2. Do you agree that UE should do this action as a part of partial MAC reset upon SCG deactivation? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | Yes | Actually, stopping some specific timers does not bring clear benefit, but there is no harm to do this. For simplicity, the UE can just stop all timers except TAT. |
| Apple | Yes |  |
| OPPO | Yes |  |
| Nokia | Yes |  |
| Futurewei | Yes | Per RAN2 agreement. |
| Ericsson | No, the exception shall also include BeamFailureDetectionTimer | It is agreed to continue BFD in SCG de-activation state and not reasonable to stop the beamFailureDetectionTimer, if UE is configured to continue BFD. |
| LG | Yes |  |
| Samsung | Yes, but | Ericsson’s comment sounds reasonable. |

The third action to be discussed is

1>	discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

This action is related to whether to allow dedicated RACH resources indicated before SCG activation indication (when going to the SCG deactivated state or while the SCG is deactivated). If it is allowed, then UE should keep CFRA resources. Otherwise, this action can be performed as a part of partial MAC reset upon SCG deactivation.

- Option 1. discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

- Option 2. Do not discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

Q3. Which option do you prefer if you agree that UE should do this action as a part of partial MAC reset upon SCG deactivation? or do you have any other suggestion?

| Company | Preferred option | Comments |
| --- | --- | --- |
| vivo | Option 2 | First, we think there is no benefit to restrict the network flexibility on this aspect. If the NW has concern to reserve CFRA resources for SCG activation, the NW can decide not configure the dedicated RACH resource to the UE. While if the NW wants to speed up SCG activation, for example, to support MAC CE based SCG activation or allow RACH-based UE initiated SCG activation, the NW is enabled to provide CFRA resources before SCG activation. So, we prefer to allow the NW to pre-config the CFRA resources before SCG activation. And if provided, the UE should not discard it. |
| Apple | Op 2 | We also mentioned this in discussion #223. Op1 will not allow NW to configure dedicated resources to the UE if the UE discards them at deactivation of SCG. |
| OPPO | Option 2 |  |
| Nokia | Option 1 | NW can reconfigure upon activation if it intends to. |
| Futurewei | Option 1 | It is not worth to let the UE to lock the dedicated resource from the beginning of the deactivation. Deactivated SCG could stay for long time and if the UE holds the dedicated resource for deactivated SCG, the on-going normal mobility UEs will not be able to use it. Otherwise, the resource would be used by many UEs for their HOs. It will compromise the mobility performance in high UE density areas. It has been agreed in RAN2, the dedicated resources can be configured at the activation if the network decided to do so. This allows CFRA and will be more efficient. |
| Ericsson | Option 1 | Any CFRA resource can be provided in the RRC message that contains SCG activation command. In Option 2, not to discard CFRA resource might be useful, if the UE can initiate SCG activation but this has not been agreed yet. The option 2 also raises the follow-up questions on the validity duration of the CFRA resources and so a further complication. |
| LG | Option 2 | In #223, it is discussed whether the dedicated RACH resources should be configured for the purpose of faster MCG failure recovery. If dedicated RACH resources for MAG failure recovery is introduced, RAN2 should go Option 2. |
| Samsung | Option 1 | This issue is related to another discussion, #223. It would be better to revisit this, if needed. |

The fourth action to be discussed is

1>	reset all BFI_COUNTERs;

RAN2 agreed to support BFD and RLM for deactivated PSCell. Therefore, the UE continues to perform BFD and RLM even after SCG deactivation, if configured. In this case, UE should not reset BFI_COUNTER for deactivated PSCell since UE still performs BFD and RLM for the same TCI state on the same BWP. However, UE should reset all BFI_COUNTERs if BFD and RLM are not configured upon SCG deactivation. In addition to this, the UE should reset BFI_COUNTER if the TCI state and BWP are changed and if BFD and RLM are configured.

Note that the legacy SCell deactivation does not reset BFI_COUNTER as well as beamFailureDetectionTimers, and BFD cannot be performed if SCell is deactivated. So, we can follow the legacy procedure for SCells because RAN2 already agreed to deactivate SCells of SCG for deactivated SCG.

Moreover, given that 38.321 already specified the initialization procedure for beam failure detection as shown below, UE will reset BFI_COUNTER if the TCI state and BWP are changed and if BFD and RLM are configured.

1>	if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell:

2>	set BFI_COUNTER to 0.

In this regard, the following action would be enough.

- reset all BFI_COUNTERs if BFD and RLM are not configured for deactivated SCG;

Q4. Do you agree that UE should do this action as a part of partial MAC reset upon SCG deactivation? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | Partial agree | We prefer: - reset all BFI_COUNTERs if BFD and RLM are not configured for deactivated SCG; The reasons are as follows: - If BFD is not configured for deactivated SCG, the values of BFI_COUNTERs and beamFailureDetectionTimer will be outdated when activating the SCG, so it is reasonable to reset BFI_COUNTERs and beamFailureDetectionTimer when deactivating the SCG. - If BFD is configured for deactivated SCG, there is no quick beam failure detection requirement since there will be no data transmission during SCG deactivation, so it seems also fine to reset BFI_COUNTERs and beamFailureDetectionTimer upon SCG deactivation. So, the condition “if BFD and RLM are not configured for deactivated SCG” seems not needed. |
| Apple | Agree, but needs some critical clarifications | Our intention is that if the RRC message that deactivates SCG does not ‘change’ anything on the RLM/BFD ‘AND’ if the UE supports BFD/RLM in deactivated SCG (a UE capability that we have to discuss), then the UE does not reset. Otherwise the UE resets. |
| OPPO | Yes | In my understanding, RAN2 agreed that BFD and RLF supporting for deactivated SCG is up to network configuration. |
| Nokia | No | Firstly, RLM does not affect MAC and hence should not be listed. Secondly, if BFD is performed only for PSCell, then the BFI_COUNTERs of SCells should be reset. Hence, could reformulate to: „1> if PSCell is configured for beam failure detection for deactivated SCG: 2> reset all BFI_COUNTERs except BFI_COUNTER associated with PSCell. 1> else: 2> reset all BFI_COUNTERs.” On the other hand, if BFD can be performed also for SCells, then should only reset BFI_COUNTERS for serving cells for which BFD not configured. The above TP could hence be: „1> if at least one Serving Cell is configured for beam failure detection for deactivated SCG: 2> reset all BFI_COUNTERs except BFI_COUNTERs associated with Serving Cells configured for beam failure detection. 1> else: 2> reset all BFI_COUNTERs.” |
| Futurewei |  | The text proposed by Nokia sound reasonable and the details could be further discussed. |
| Ericsson | Yes, but | As replied in Q2, the UE shall not stop the beamFailureDetectionTimer if BFD is configured. On the other hand, the UE shall stop the beamFailureDetectionTimer if BFD is not configured. Not sure the need to mention RLM here at the MAC spec.. can be removed? |
| LG | No | We also prefer the following text as in legacy MAC reset. - reset all BFI_COUNTERs if BFD and RLM are not configured for deactivated SCG; Even if all BFI_COUNTERs reset regardless of whether the BFD is configured or not, we do not see the problem. As motioned by vivo, there is no quick beam failure detection requirement while de-activating the SCG. |
| Samsung | Yes, but | Based on comments from others, it would be better to remove “RLM” even if BFD and RLM are configured by one parameter in RRC message. Technically, there is no reason to re-count BFI_COUNTER from 0 if BFD continues. SCG deactivation will deactivate SCell of SCG according to RAN2 agreement. Note that legacy SCell deactivation does not reset BFI_COUNTER. So, simply we can do nothing if BFD is configured for deactivated SCG while we can reset all BFI_COUNTERs like legacy MAC reset if BFD is not configured for deactivated SCG. |

The next actions to be discussed are

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	stop, if any, ongoing Random Access procedure;

1>	flush Msg3 buffer;

1>	flush MSGA buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	cancel, if any, triggered Configured uplink grant confirmation;

1>	flush the soft buffers for all DL HARQ processes;

1>	for each DL HARQ process, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI.

The above actions can be performed as a part of partial MAC reset upon SCG deactivation.

Q5. Do you agree that UE should do these actions as a part of partial MAC reset upon SCG deactivation? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | Yes | Since there will be no SCG transmission during SCG deactivation, all the above actions to be performed upon SCG deactivation seem reasonable. |
| Apple | Yes, but | We also need to keep in mind the outcome of #223 (MCG failure recovery using deactivated SCG, and if SR can be used for this). We are ok with stopping the SR (and RACH) at deactivation, the UE can start again if needed (in case RAN2 progress on #223). |
| OPPO | Yes |  |
| Nokia | Yes |  |
| Futurewei | Yes |  |
| Ericsson | Yes, but | What about the LBT_COUNTERs? It might be okay to preclude the actions related with V2X, IAB, since we don’t see any use-case for those yet. But for NR-U, maybe it is clearer that this can be useful for SCG deactivation, or ? |
| LG | Yes |  |
| Samsung | Yes |  |

In legacy, the common understanding is that the corresponding BWP is deactivated when a SCell is deactivated. Based on this principle, we can apply the same principle to the BWP associated with PSCell.

Proposal. The BWP associated with PSCell is deactivated upon SCG deactivation.

Q6. Do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | Maybe | Case 1: the PSCell BWP is activated during SCG deactivation. If we support this case, then according to the MAC spec, the UE behavior defined for SCG deactivation in SCG activation/deactivation section will be not aligned with the UE behavior defined for active BWP in the BWP operation section. This maybe solved by adding a clarification in the SCG activation/deactivation section. Case 2: the PSCell BWP is deactivated during SCG deactivation. If we support this case, the following issues should be discussed: - How does the UE perform BFD/RLM while the SCG is deactivated, considering the BFD/RLM RS is configured per BWP? E.g., the network indicates which BWP is used for this purpose or network can configure per UE level RLM/BFD configuration. - What is the active BWP of PSCell upon SCG activation? In our view, it can be first active BWP of PSCell or the BWP on which the UE performed BFD/RLM during SCG deactivation. - According to the current 331 spec, upon reconfiguring first active BWP ID for PSCell, the BWP indicated by this ID should be activated immediately. So, we need to discuss how to avoid this behavior. In our understanding, we can add some restriction on UE behavior or network configuration. |
| Apple | No | We think bringing deactivation/activation brings more issues, and it’s better to leave BWP as is. BWP ‘switch’ if any, can be dealt with a SCG re-activation (using RRC message). |
| OPPO | No | RAN2 agreed that all SCG SCell is deactivated state when SCG is deactivated. But for PSCell, it is not clear. In my understanding, PSCEll is not deactivated state, because CSI reporting, BFD….are supported on PSCEll. The further discussion about PSCell state should be discussed in RAN2. |
| Nokia | Yes |  |  | Does not seem necessary to send CSI reports while there is no data transmission |
| Futurewei |  | No action is required for the BWP. |
| Ericsson | No | The UE perform RLM and BFD in PSCell. It is not correct to say that the BWP is de-activated. SCell and PSCell deactivation are quite different things, so there seems no motivation to say that BWP is deactivated for PSCell. |
| LGE | Yes |  |
| Samsung | Yes | Our understanding is that PSCell is deactivated upon SCG deactivation according to current running MAC CR. Then, it seems reasonable to deactivate BWP. The exceptional behaviour, e.g. BFD, etc. should be defined separately in RAN1 or RAN2 specification. |

Regarding CSI-RS reporting of deactivated PSCell, sending CSI-RS reporting on the deactivated PSCell would just increase power consumption given that no data transmission is ongoing. Moreover, sending CSI reporting for the deactivated PSCell via MCG would request additional inter-node message design and may not worth it considering the inter-node delay. The basic PSCell quality could be maintained by RLM/BFD mechanism.

Proposal. CSI-RS reporting in the deactivated PSCell or for the deactivated PSCell is NOT supported.

Q7. Do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | No strong view | It is beneficial for fast SCG activation that CSI-RS measurement and reporting can be performed when TAT is still running during SCG deactivation. However, if the majority do not want to support, we are also fine for progress. |
| Apple | Yes | We agree this is not essential for Rel-17. |
| OPPO | No | RLM/BFD is detected in UE side and CSI reporting to network aims to aid the network to do scheduling. So I do not think the BFD/RLM can also used for scheduling. |
| Nokia | Yes | Does not seem necessary to send CSI reports while there is no data transmission |
| Futurewei | Yes | It has been agreed in RAN2. |
| Ericsson | Yes | BFD should be good enough as compromise between readiness for activation and power consumption and we therefore not need CSI measurement and reporting. |
| LG | Yes | The UE should not perform CSI-RS report in SCG deactivated state because CSI-RS report increases the power consumption. However, we support to perform CSI-RS measurement for the fast SCG activation. |
| Samsung | Yes |  |

In RAN2#116e it was agreed that UE indicates via the MCG that it has UL data to send for SCG DRBs, FFS indication contents and format.

The data volume calculation procedure in the PDCP specs and the RLC specs are used for the buffer status reporting triggering in the MAC spec. The same procedures can be re-used for reporting UE buffer status but rather deliver to the RRC layer due to that the message is encoded at the RRC layer. For SCG DRBs, if the total amount of PDCP data volume and RLC data volume pending for initial transmission is larger than zero, then the UE can indicate it in an RRC message.

On the other hand, one indication could be enough given that UE can send BSR MAC CE after SCG activation as in legacy, i.e. no further optimization would be needed.

- Option 1. The UE indication of uplink data and the total data volume (e.g. long BSR MAC CE) can be included in UE Assistance Information message.

- Option 2. Only the UE indication of uplink data can be included in UE Assistance Information message. BSR can follow the legacy procedure.

Q8. Which option do you prefer? or do you have any other suggestion?

| Company | Preferred option | Comments |
| --- | --- | --- |
| vivo | Option 2 | This question has been covered by the offline [222]. Better to discuss this issue in [222] since this issue is related to UL aspect not MAC aspect. |
| Apple | Op 2 | Same view as Vivo |
| OPPO | None | In my understanding, the MCG MAC cannot report SCG BSR right currently, right? So option 1 does not work. One indication for the UL data arrival is simple. The indication is included in RRC message or MAC CE, it can be discussed further. |
| Nokia | Option 2 | One bit indication is enough to indicate UL data arrival for SCG. |
| Futurewei | Option 1 | Option 1 takes the advantage of UE initiated RRC SCG activation request to carry information which should be reported by the UE. Option 2 should also work to get the information after the UE access to the SN with additional delay and signaling overhead. |
| Ericsson | Option 1 | A timely report of the Buffer Status from the UE to the network is always beneficial for the network to schedule the correct amount of radio resources upon SCG activation, to reduce the latency of the packet delivery. To add a detailed total data volume in the RRC is very simple and beneficial. NOTE that this question is also handled in offline [222]. We should avoid double discussion, and suggest to treat this topic as part of [222]. |
| LG |  | This issue has been already discussed in offline 222. |
| Samsung | Option 2 | OK to leave it to offline 222. |

PSCell can be activated and deactivated based on SCG activation and deactivation indication from network. For deactivated PSCell, the UL BWP behavior would not include any uplink transmission, i.e. the uplink power control would not be needed and thus it seems reasonable not to include the PHR report for deactivated PSCell.

Proposal. For deactivated PSCell, PHR is not reported.

Note that this proposal would not have any impact on the MAC specification.

Q9. Do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | Yes | It is obvious. |
| Apple | Yes | In the deactivated SCG state, no PHR is needed. |
| OPPO | Not sure | RAN2 agree to support to keep TAT running when SCG is deactivated. The purpose is to skip RACH procedure and can receive scheduling directly when SCG is activated. So the PHR and CSI reporting are necessary. Please note that RAN2 did not agreed that PSCell is deactivated state during SCG deactivation. |
| Nokia | Yes | The PHR would anyway be reported to MCG MAC entity only and by the PSCell Serving Cell ID in the bitmap set to 0 the MCG can deduce no report is present (and knows the SCG is deactive). |
| Futurewei | Not completely | In most cases, it is true. But in case of UE initiated activation, the UE could report PHR via RRC activation request before the SCG is activated. It would be beneficial for RACH less activation to minimize the delay. |
| Ericsson | Yes | Perhaps what is interesting to discuss is whether the PHR can be triggered or not when SCG is de-activated. At least, even if a PHR is triggered, PHR is not reported. In the MAC spec, the report is written under the condition that: If the MAC entity has UL resources allocated for a new transmission the MAC entity shall: |
| LG | Yes |  |
| Samsung | Yes |  |

In the legacy MAC specification, PHR is triggered upon addition of the PSCell, i.e. PSCell is newly added or changed. However, given that the feature of SCG activation/deactivation is introduced, we need to note that the uplink transmission would start after SCG activation from deactivated SCG. Therefore, a new PHR trigger would be beneficial to help the network manage fast uplink power control, i.e. PHR can be triggered in case that PSCell is activated. One can argue that it would be beneficial to trigger a PHR upon SCG deactivation as well but it would be the next step.

Proposal. PHR is triggered upon activation of the PSCell.

Q10. Do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | Yes |  |
| Apple | Yes |  |
| OPPO | Not sure | We need confirm the PSCell state during SCG deactivation first. |
| Nokia | Yes | Should also be triggered for deactivation to update the power situation even if the PH for the deactivated PSCell itself is not reported in the PHR. |
| Futurewei | Yes in general, but | It can be different under different scenarios. - When RACH is required, PHR should be reported after random access is completed per legacy procedure. For RACH-less case, if the activation is network initiated, PHR report will be scheduled after the UE received the activation command. If the activation is UE initiated, PHR report can be carried by the UE issued activation request. |
| Ericsson | Yes |  |
| LG | Yes |  |
| Samsung | Yes |  |

If the proposal related to Q10 is agreeable, then the text proposal could be controversial. We need to note that RAN2 had the same discussion about whether to trigger a PHR for re-activation of SCell, several times before, i.e. whether to limit the PHR triggering to the case that SCell is activated from deactivated state. It has ended up with no such limitation. We can follow the legacy principle of SCell. On the other hand, such limitation would have no harm and thus could be acceptable based on the majority view.

- Option 1. Activation of the PSCell (the same as activation of SCell).

- Option 2. Activation of the PSCell from deactivated state.

Q11. If you say yes for Q10, which option do you prefer? or do you have any other suggestion?

| Company | Preferred option | Comments |
| --- | --- | --- |
| vivo | Option 1 | Both are ok, and fine to follow the legacy principle of SCell. |
| Apple | We do not understand Op 1, as to what does it mean: activation of PSCell other than from deactivated SCG state | PHR to be triggered when UE moves out of SCG deactivated state to activated state. |
| OPPO |  | We need confirm the PSCell state during SCG deactivation first. |
| Nokia | Option 1? | Activation of PSCell always happens from deactivated state – no similar issue as with SCell due to no MAC CE based activation. Hence, either option works. |
| Futurewei |  | See our response to Q10. Don’t see much difference between option 1 and 2. |
| Ericsson |  | Not sure i have understood the comments by the rapportuer.. they are the same or is it more stage-3 detail, or ?? |
| LG | Option 1 | We prefer to follow the legacy principle. |
| Samsung | Option 1 | The intention was to discuss stage-3 details, i.e. how to specify the triggering condition even if re-activation issue like SCell may not exist. |

If the proposal related to Q10 is agreeable, then it may have an impact on the legacy procedure because RAN2 agreed that SCG can be added/changed with activated state or deactivated state. If PSCell is added/changed with deactivated state, then the PHR doesn’t have to be triggered upon addition of PSCell. Hence, the following proposal could be beneficial.

Proposal. PHR is triggered upon addition of the PSCell (i.e. PSCell is newly added or changed with activated state).

Q12. If you say yes for Q10, do you agree to this proposal? or do you have any other suggestion?

| Company | Yes or No | Comments |
| --- | --- | --- |
| vivo | Yes |  |
| Apple | PHR is triggered when the PSCell is activated (either as a new PSCell using RRC, or just using RRC for re-activation of existing deactivated PSCell). |  |
| OPPO |  | It is current behavior, right? === - addition of the PSCell (i.e. PSCell is newly added or changed); |
| Nokia | No | It is reasonable to trigger the PHR and report to MCG even for addition of deactivated PSCell. |
| Futurewei |  | Have similar view as Apple at high level. Details could be different under different scenarios as our response to Q10. |
| Ericsson | No | It should be: PHR is triggered upon activation of the PSCell (i.e. either when PSCell is added in activated state or when activity state is changed from deactivated to activated). |
| LG | Yes |  |
| Samsung | Yes | The intention was the change to the legacy condition as shown below: - addition of the PSCell (i.e. PSCell with activated state is newly added or changed); |

Conclusion

TBD