3GPP TSG-RAN WG2 Meeting #117e	R2-2202339

Online, 21th Feb - 3rd Mar, 2022

Agenda:	8.7.2.4

Source:	OPPO

Title:	 	Left issue on QoS for layer 2 relay.

Document for:	Discussion, Decision

- Introduction

After RAN2 #116bis-e meeting, the open issue list discussion of NR sidelink Relay work item had been triggered. Whereas just few remaining issues related to QoS have been identified. In this contribution, we will just to conclude on those remaining issues.

- Discussion

As it is mentioned by companies, in current Uu, there is a parameter related and (implicitly) decided by PDB, i.e., discardTimer in PDCP-Config, which is used for UEs to discard the buffer if the buffering time of a user plane packet lead to expiry of this timer. While in L2 UE-to-Network Relay, PDCP discard timer will be maintained in the PDCP entities of both Remote UE side and serving gNB side.

- On the one hand, for the PDCP entity in the remote UE side, it can be left to gNB implementation to configure a suitable discard timer value.

- On the other hand, the relay UE will be configured with two PDB values per PC5 RLC bearer granularity. For the relay UE it is an issue whether a PDCP PDU whose buffer time extends the configured PDB will be discarded or not. Therefore, they proposed that it should be up to relay UE implementation whether to discard a PDCP PDU whose buffer time beyond the configured PDB.

However, from our view, firstly, for L2 UE-to-Network Relay, there is no PDCP layer in Relay UE, i.e., it acts as a connection below the PDCP layer. Also, the buffer management for each layer (if needed) is always implemented by UE implementation, which does not equal to timer-based discarding operation at PDCP layer. Therefore, there is no need to discuss this issue.

RAN2 not pursue spec effort for PDCP PDU (of remote UE) discarding behaviour by Relay UE .

- Conclusion

During this meeting, the remaining issues of relay discovery have been pointed out and discussed.  the following proposals are given out:

Proposal 1:	RAN2 not pursue spec effort for PDCP PDU (of remote UE) discarding behaviour by Relay UE .