3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-2203281

Elbonia, 21 February – 03 March 2022

Agenda item:	8.12.3.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Early identification capability

WID/SID:	NR_redcap-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

The following WA was made in the previous meeting:

Working assumption: 1. Msg3 early identification is mandatorily supported by RedCap UE

This paper discusses this issue and proposes to confirm the WA.

# 2	Discussion

In the previous RAN2 meeting, also the followings were agreed:

1. In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, when the Msg3 includes the CCCH data (no other precondition) 2. Also when msg1 early identification is configured, new dedicated LCID is used for CCCH identification

As the Msg3 identification is encoded by the RedCap UE regardless of any precondition to the Msg3 (and MsgA), there cannot be capability for such support. The Msg3/MsgA early identification support has to be mandatorily supported by any RedCap UE.

Proposal 1: Msg3/MsgA identification is mandatorily supported by a RedCap UE.

Although Msg1 based identification would not be required given the Msg3 identification is always encoded by a RedCap UE, the network – by configuring the PRACH for RedCap – expects RedCap UEs to access via such PRACH (e.g., by applying more conservative scheduling for Msg3). Hence, Msg1 identification shall be similarly supported by a RedCap UE.

Proposal 2: Msg1 identification is mandatorily supported by a RedCap UE.

# 3	Conclusion

Proposal 1: Msg3/MsgA identification is mandatorily supported by a RedCap UE.

Proposal 2: Msg1 identification is mandatorily supported by a RedCap UE.