3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-2203301

Online, February 21- March 03, 2022

Agenda item:	8.10.3.2.1

Source:	Samsung

Title:	Open issues on RRC aspects

WID/SID:	NR_NTN_solutions-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This document discusses some open issues for CHO in running RRC CR [1] [2].

# 2	Discussion

RAN2 has agreed to support time-based CHO for both moving cell and fixed cell scenario. The time-based conditional trigger event is defined that if the time measured at the UE is in a time duration [t1, t2] the event condition is considered to be satisfied. The time duration defined in a trigger event for a candidate cell is considered to be the time during which the UE can execute CHO to the candidate cell. Thus, the duration should cover at least the time that the candidate cell serves the UE’s moving area. According to the analysis in TR 38.821-7.3.2.1 [3], the duration can span from 6 seconds to about 140 seconds depending on the cell diameter size, the satellite speed, and the UE speed. Taken into account the worst case scenario, integer values from 0 to 255 should be considered for the duration in seconds.

Proposal 1: For time-based CHO, the duration for the time-based conditional trigger event is defined as integer {0, 1, 2, …, 255} in seconds.

# 3	Conclusion

Based on the discussion, we have the following observations and proposals.

Proposal 1: For time-based CHO, the duration for the time-based conditional trigger event is defined as integer {0, 1, 2, …, 255} in seconds.

# Reference

[1] R2-2201896 116bise NTN open issues RRC_v06

[2] R2-2201895 Stage-3 running RRC CR for NTN Rel-17 post116bise

[3] 3GPP TR 38.821 V16.0.0 Solutions for NR to support non-terrestrial networks (NTN)