3GPP TSG-RAN WG2 #117 	R2-2203419

Online, February 21th – March 3rd 2022

Agenda Item:	8.20.2

Source:	ZTE Corporation, Sanechips

Title:	Remaining UP issues for extending to 71GHz

Document for:	Discussion, Decision

# Introduction

In last meeting, no-LBT mode has been discussed and the following agreement was made.

6: “no-LBT mode” is already implicitly supported by Rel-16 specifications. FFS if additional differentiation from licensed operation in specification is required for some cases.

About L2 buffer size, it has been discussed for two meetings due to too high maximum data rate. Some companies provide some solving schemes, such as introducing new capability. But it is still FFS.

In this contribution, we will discuss these remaining issues and provide our views.

# Discussion

“no-LBT” mode differentiation

According to the current Spec, only when LBT fails, LBT failure indication can be informed to the MAC layer, otherwise, there is no LBT failure indication, which means that ”no-LBT” mode is already implicitly supported. However, there is still a FFS regarding if additional differentiation form licensed operation in specification is required for some cases in the last meeting. When the UE operates in licensed spectrum, since there is no LBT, LBT failure indication cannot be received from the lower layer and the UE follows licensed procedure. When the UE operates in shared spectrum and no-LBT mode is configured, there is no LBT failure indication to be received either and the UE follows the procedure according to configuration. Hence, from the MAC layer perspective, for licensed spectrum and no-LBT mode of shared spectrum, it is no necessity to differentiate for licensed operation and no-LBT mode.

Proposal 1: From the MAC layer perspective, there is no need for additional differentiation for licensed operation and “no-LBT” mode of shared spectrum.

L2 buffer size

Regarding L2 buffer size, the following agreements were made in the past meeting.

#2: Keep the L2 buffer size definition as it reflects the upper bound of the L2 buffer size requirement. #3: FFS whether UE capability is needed to address concern on too high L2 buffer size requirement. Companies should bring analysis on this to next meeting.

The current L2 buffer size definition will be kept, then larger bandwidth in FR2-2 will lead to higher L2 buffer requirement. For this issue, there is a FFS regarding whether UE capability is needed to relax L2 buffer or not.

Regarding the maximum data rate in L2 buffer size definition, the formula in [3] is as below:

Wherein, is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. Scaling factor is as a UE capability per feature set to report to gNB due to the lower peak data may be beneficial in some cases for UE. Then for FR2-2, if the UE do not support higher L2 buffer size, it may report scaling factor to lower peak data rate to relax L2 buffer size. In other words, for the issue above, the existing UE capability scaling factor may be used to lessen L2 buffer requirement. In the current spec, the supported scaling factor values are 0.4, 0.75 and 0.8. In order to further lessen buffer requirement, smaller scaling values may be introduced, such as 0.1, 0.2. So there is no necessary to introduce new UE capability.

Proposal 2: Some smaller values may be introduced for the existing UE capability scaling factor to lessen high L2 buffer requirement.

# Conclusion

Based on the discussion in section 2 we observe and propose the following:

Proposal 1: From the MAC layer perspective, there is no need for additional differentiation for licensed operation and “no-LBT” mode of shared spectrum.

Proposal 2: Some smaller values may be introduced for the existing UE capability scaling factor to lessen high L2 buffer requirement.

# References

- Draft_R2-116e_Meeting_Report_v1

- Draft_R2-116bise_Meeting_Report_v1

- TS 38.306 g70.