- 3GPP TSG RAN WG2 Meeting #117-e	      	               R2-220xxxx
E-Conference, 21st Feb – 3rd Mar, 2022

Agenda item: 	8.22.3.2.1

Source:	Intel Corporation

Title: 	[AT117-e][018][MGE] Pre-configured MG patterns (Intel)

WID/SID:	NR_MG_enh-Core – Release 17

Document for:	Discussion and Decision

# Introduction

This is to start offline discussion below: Please provide your comment by 2/25/2022 13:00UTC.

- [AT117-e][018][MGE] Pre-configured MG patterns (Intel)

Scope: Based on R2-2203523, progress remaining proposals. Determine agreeable parts, points for discussion, open issues if needed. Converge as far as possible to reduce the need for on-line discussion

Intended outcome: Report

Deadline: In time for on-line CB W2 Tuesday

# Discussion

## Support pre-configured MG under CA based on BWP switching on a single CC

In RAN4 LS [16], it discusses the support of pre-configured MG under CA scenario:

Regarding use cases of pre-configured MG, RAN4 has reached the following conclusions:

- Support pre-configured MG under CA but based on BWP switching on a single CC

Q1: Do you agree to support of CA scenario for pre-configured gap?

| Company | Support/ No support | Comments |
| --- | --- | --- |
| MediaTek | Support |  |
| ZTE | Support |  |
| Huawei, HiSilicon | Support |  |
| Apple | Support |  |
| Samsung | Support |  |
| Ericsson | Support |  |
| Intel | Support |  |

Summary:

Here is RAN4 agreement:

Agreements in last RAN4 meeting:

The principle for the signaling-based Pre-MG (de)activation under CA can be based on:

- NW will forward the per-BWP pre-MG status indications to UE by RRC message, which is defined for each of induvial CCs (e.g. Table S1, S2 for the CC1 and CC2 respectively)

- UE will combined the pre-MG status indication from NW for each induvial CC for all configured measurement objects.

Combine the RAN4 agreement and LS, it seems like RAN4 intension is to support multiple CC scenarios where the procedure is as follow:

- Multiple CC CA case is supported but only single CC BWP switching at a time is supported

RAN4 sends LS indicates the support of pre-configured MG is based on BWP switching on a single CC. Legacy BWP switching in different CC is independent operation. The condition of “BWP switching on a single CC” put a limitation on existing BWP switching operation, i.e. only one BWP can be switched at any given time in a single CC. Do you support to add such limitation to existing BWP operation for pre-configured MG?

Q2: Which of the following option you prefer if CA scenario is supported for pre-configured MG:

- Option 1: limit BWP switching operation for pre-configured MG (only one BWP can be switched at a given time in a single CC as state in RAN4 LS)

- Option 2: Not add limitation on BWP switching operation as in legacy

| Company | Option | Comments |
| --- | --- | --- |
| MediaTek | See comment | After further checking with our R4 colleague, we understand they continue to discuss the CA operation after sending the LS. So, we prefer to wait. Rapp: RAN4 already indicates in the LS to limit to a single CC switching. RAN4 continue discuss the remaining part of the solution where how to determine the rule for UE to combine. This particular part has already determined by RAN4 in the LS. |
| ZTE | Wait for RAN4 Rapp: RAN4 already indicates in the LS to limit to a single CC switching. | We are fine to wait for more inputs from RAN4. |
| Huawei, HiSilicon | Option 2 | RAN4 will not define requirements for the simultaneous BWP switching of multiple CCs, but we think it does not affect the signaling design of RAN2. After all, whether the gap is activated depends on whether the MOs can be covered by active BWPs. |
| Apple | Option 2 | Whether RAN4 defines the requirements does not impact RAN2 signaling. |
| Samsung | Option 2 |  |
| Ericsson | Wait for RAN4 Rapp: RAN4 already indicates in the LS to limit to a single CC switching. |  |
| Intel | Option 2 |  |

Summary:

In RAN4 agreement, network configures per-BWP per-MG status for each CC (not all combination) via RRC, then the UE will determine the measurement gap status base on the per BWP per-MG status for each CC.

  - For example, gap status for pre-configured MG1 for each CC are:

|  | BWP 0 | BWP 1 | BWP 2 |
| --- | --- | --- | --- |
| CC 1 | ON | OFF | OFF |
| CC 2 | OFF | ON | OFF |

Q3: Do you support the network configuration includes per BWP per MG status for each CC to support CA scenario for Network-controlled activation/ deactivation support of pre-configured MG? (if not support, please provide comments or alternative)

| Company | Support/ not support | Comments |
| --- | --- | --- |
| MediaTek | Support |  |
| ZTE | Support |  |
| Huawei, HiSilicon | Support |  |
| Apple | Support |  |
| Samsung | Support |  |
| Ericsson | Support |  |
| Intel | Support |  |

Summary:

Multiple companies [3,10,13] have indicated that there is no specification impact for UE autonomous activation/ deactivation mechanism to support pre-configured MG for CA case. Do you agree RAN2 can support UE autonomous activation/deactivation mechanism without specification impact under CA case?

Q4: Do you agree RAN2 can support UE autonomous activation/deactivation mechanism without specification impact under CA case?

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| MediaTek | Agree |  |
| ZTE | Agree | Look, this is simpler than explicit indicators. 😉 Rapp: 😊 |
| Huawei, HiSilicon | See comments | It depends on whether the rule is captured in RAN4 or RAN2 spec. If captured in RAN2, we cannot say “without specification impact”. Rapp: That is correct. But hope it is in RAN4 spec 😊 |
| Apple | Agree |  |
| Samsung | Agree |  |
| Ericsson | Agree |  |
| Intel | Agree |  |

Summary:

Second part of RAN4 LS[16] is to indicate the overriding rule as follow:

Regarding activation/deactivation of Pre-configured MG, RAN4 has reached the following conclusions:

- If the network provides the activation/deactivation status via RRC signalling, the UE will not use autonomous rules to determine the activation/deactivation status of the pre-configured MG. It will follow the per-BWP status indicated by the network.

Do you agree network-controlled activation/deactivation (if provided by the network) will override UE autonomous activation/ deactivation?

Q5: Do you agree network-controlled activation/deactivation (if provided by the network) will override UE autonomous activation/ deactivation?

Rewording proposal suggested by MT: If network-controlled activation/deactivation (explicit indicator) is provided, UE follow the explicit status indicator and does not use implicit rule (UE autonomous activation/deactivation).

| Company | Agree/disagree | Comments |
| --- | --- | --- |
| MediaTek | Agree but please see comment | There are two different kind of controlling method (implicit rule or explicit indicator). We think that if the explicit indicator is present, the UE follow the explicit status indicator and does not use implicit rule. It is a little bit strange to say the rule is “overrided”. Rapp: agree with wording. See rewording above. |
| ZTE | Agree | Same understanding as MTK on how it works. |
| Huawei, HiSilicon | Agree | Agree with MTK that “override” is not the proper wording and the NW configuration will not imply a different result with the UE autonomous activation/ deactivation. |
| Apple | Agree | Share MTK’s understanding. |
| Samsung | Agree |  |
| Ericsson | Agree with Mediatek |  |
| Intel | Agree | MT rewording is fine with us |

Summary:

## Network-controlled activation/ deactivation support of the pre-configured measurement gap

During online, RAN2 has agreed:

- RAN2 introduces support of NW-Controlled activation/deactivation pre-configured gap

Multiple companies [7, 8, 12] have indicates BWP status configuration should be in BWP configuration. There is also contribution [7, 13] also indicates the BWP status information can be included in the measurement gap configuration. Which of the follow option you prefer to include the activation/ deactivation BWP status:

- Option 1: The activation/deactivation status of each BWP is included in the measurement gap configuration

  - In this option, measurement gap configuration will need to be updated when BWP configuration is modified.

- Option 2: The activation/deactivation status of the specific BWP is included in the configuration of BWP

  - In this option, BWP configuration will need to be updated when measurement gap configured is modified.

Q6: Which option do you prefer to indicates BWP status to support pre-configured MG?

- Option 1: The activation/deactivation status of each BWP is included in the configuration of pre-configured MG

- Option 2: The activation/deactivation status of the specific BWP is included in the configuration of BWP

| Company | Prefer option | Not acceptable option | Comments |
| --- | --- | --- | --- |
| MediaTek | Option 2 |  | Both options should be feasible. We consider that option 2 is simpler. |
| ZTE | Option 2 |  | Option 2 is straightforward, once BWP is deleted, there is no need to update gap configuration. |
| Huawei, HiSilicon | Option 2 |  |  |
| Apple | Option 2 |  |  |
| Samsung | Option 2 | Option 1 | We understand that both the options can work in R17. But option 1 needs more signaling for CA and can get more complicated when MR-DC is supported in future releases. |
| Ericsson | Option 2 |  |  |
| Intel | Option 2 |  |  |

Summary:

# Conclusion

TBD

# References

[1] R2-2202322, Discussion on per-configured measurement gap, VIVO

[2] R2-2202460, Discussion on support of case 4, Intel Corporation

[3] R2-2202461, Support of pre-configured MG under CA, Intel Corporation

[4] R2-2202513, RAN2 impact from pre-MG, Apple

[5] R2-2202647, Remaining issues on Pre-configured MG, ZTE Corporation

[6] R2-2202873, Discussion on open issue of pre-configured gap, MediaTek

[7] R2-2202890, Discussion on Pre-configured MG, Huawei, HiSilicon

[8] R2-2202944, Discussion on remaining issues of pre-configured MG, CATT

[9] R2-2202977	, Discussion on Pre-MG activation and deactivation, Samsung

[10] R2-2203011, Discussion on the support of Pre-MG for CA, Samsung R&D Institute India

[11] R2-2203037, Remaining issues on Pre-configured MG, LG Electronics Inc

[12] R2-2203060, Discussion on Pre-configured MG, Xiaomi

[13] R2-2203260, Discussion on open issues for pre-configured MG, Nokia, Nokia Shanghai Bell

[14] R2-2203448, Pre-configured measurement gaps, Ericsson

[15] R2-2203504, Pre-Configured gap case-4 discussion, Qualcomm

[16] R2-2202159/ R4-2202615, LS on R17 NR MG enhancements – Pre-configured MG, RAN4

[17] R2-2203523, [Pre117-e][018][MGE] AI Summary of 8.22.3.2.1 Pre-configured MG patterns, Intel