3GPP TSG-RAN WG2 Meeting #117-e		R2-220xxxx

Electronic, 21st Feb. – 3rd Mar. 2022

Title:	[Draft] Reply LS to RAN4 on RRM relaxation

Response to:	R2-2202163_R4-2202675

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	vivo [to be RAN2]

To:	RAN4

Cc:

- Contact Person:

- Name:	                Chenli

E-mail Address:	 Chenli5g@vivo.com

- 1. Overall Description:

RAN2 would like to thank RAN4 for the LS on RRM relaxation, and provide the following feedback on below RAN4 question:

Regarding higher priority inter-frequency measurement relaxation when only Rel-17 stationarity criterion is satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ or both Rel-17 criteria are satisfied, RAN4 kindly ask RAN2’s guidance on what is RAN2’s methodology designed for this scenario, if there is any.

RAN2 concluded there is no need to introduce an indication, similar as highPriorityMeasRelax in Rel-16, to control the RRM relaxation of higher priority frequency. When the criteria are satisfied, e.g. only Rel-17 stationarity criterion is satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ or both Rel-17 criteria are satisfied, RRM relaxation for higher priority frequency would be performed similar to qual/lower priority frequency. The detailed relaxation methods are up to RAN4.

Besides, RAN2 have agreed to define a Rel-17 indicator similar to combineRelaxedMeasCondition-r16. This indication is used to differentiate two cases 1) only stationary criterion is met, and 2) both criteria (stationary and not-at-cell-edge) are met, when both criteria are configured.

The detailed behaviour is: Reuse the specification approach from Rel-16 for combined relaxed measurement condition in Rel-17, i.e.

when both stationary and not-at-cell-edge criteria are configured,

- 1) if both criteria are fulfilled, UE performs RRM relaxation 1.

- 2) if stationary criterion is fulfilled but not-at-cell-edge is not fulfilled and combineRelaxedMeasCondition2 is not configured, UE performs RRM relaxation 2.

- 3) if stationary criterion is fulfilled but not-at-cell-edge is not fulfilled and combineRelaxedMeasCondition2 is configured, UE does not perform RRM relaxation.

where RRM relaxation 1 and 2 correspond to the methods agreed in RAN4.

- 2. Actions:

- To RAN WG4

- RAN2 kindly request RAN4 to take the above information into account during the following work, and study/specify the corresponding requirements/approaches for RRM relaxation, and provide feedback, if any.

- 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #118-e		16th – 27th May 2022				e-Meeting

TSG RAN WG2 Meeting #119		TBD								TBD